Explore more resources

Altera® Design Hub

Agilex™ 7 FPGAs and SoCs Device Data Sheet

M-Series

Online Version Send Feedback

769310

2025.09.05

Contents

Contents

Agilex™  7 FPGAs and SoCs Device Data Sheet: M-Series	3

Electrical Characteristics	4

Operating Conditions.	4

Switching Characteristics.	39

Configuration Specifications	117

I/O Timing.	125

Programmable IOE Delay	125

Glossary.	125

Document Revision History for the Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	129

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

2

769310 | 2025.09.05

Send Feedback

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

This data sheet describes the electrical characteristics, switching characteristics, configuration specifications, and timing.

Until the data sheet status for a device reaches Final, the specifications are subject to change at any time and at Altera's discretion.

Table 1.	Data Sheet Status for Agilex™ 7 FPGAs and SoCs M-Series

| Device | Tile | Package | Status |
| --- | --- | --- | --- |
| AGM 032/039 | F-Tile and R-Tile | R47A | Final |
| AGM 032/039 | F-Tile | R31B | Final |
| AGM 032/039 | F-Tile | R47B | Final |

The following descriptors designate the status level currently applicable to the relevant variant:

- Advance: These are target specifications based on simulation.

- Preliminary: These specifications are based on simulation, early validation, and/or early characterization data.

- Final: These are production specifications based on silicon validation and/or characterization.

Table 2.	Device Grades, Core Speed Grades, and Power Options Supported

For specification status, see the Data Sheet Status table

| Device Grade | Speed Grade and Power Option Supported |
| --- | --- |
| Extended | –E1V (fastest) |
|  | –E2V |
|  | –E3V |
| continued... |  |

© Altera Corporation. Altera, the Altera logo, the ‘a’ logo, and other Altera marks are trademarks of Altera Corporation. Altera reserves the right to make changes to any products and services at any time without notice. Altera assumes no responsibility or liability arising out of the application or use of any information, product, or service described herein except as expressly agreed to in writing by Altera. Altera customers are advised to obtain the latest version of device specifications before relying on any published information and before placing orders for products or services.

*Other names and brands may be claimed as the property of others.

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Device Grade | Speed Grade and Power Option Supported |
| --- | --- |
|  | –E3E |
| Industrial | –I1V |
|  | –I2V |
|  | –I3V |

The suffix after the speed grade denotes the power options offered.

- V—standard power (VID)

- E—lower power (VID)

Related Information

- Package and Thermal Resistance website

- Available Options section, Agilex 7 FPGAs and SoCs Device Overview

Electrical Characteristics

Operating Conditions

The devices are rated according to a set of defined parameters. To maintain the highest possible performance and reliability of the devices, you must consider the operating requirements described in this section.

Absolute Maximum Ratings

This section defines the maximum operating conditions. The values are based on experiments conducted with the devices and theoretical modeling of breakdown and damage mechanisms. The functional operation of the device is not implied for these conditions.

Caution:	Conditions outside the range listed in the following table may cause permanent damage to the device. Additionally, device operation at the absolute maximum ratings for extended periods of time may have adverse effects on the device.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

4

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Table 3.	Absolute Maximum Ratings

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition | Minimum | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| VCC | Core voltage power supply | SmartVID(1) : –1V, –2V, – 3V, –3E | –0.5 | 1.21 | V |
| VCCP | Periphery circuitry power supply | SmartVID(1): –1V, –2V, – 3V, –3E | –0.5 | 1.21 | V |
| VCCPT | Power supply for I/O PLL and I/O pre-driver | — | –0.5 | 2.08 | V |
| VCCRCORE | CRAM power supply | — | –0.5 | 1.64 | V |
| VCCH | Transceiver digital power supply | Devices with R-Tile and F- Tile | –0.5 | 1.07 | V |
|  |  | Devices with F-Tile only | –0.5 | 1.07 | V |
| VCCH_SDM | SDM block transceiver digital power sense | Devices with R-Tile and F- Tile | –0.5 | 1.21 | V |
|  |  | Device with F-Tile only | –0.5 | 1.07 | V |
| VCCIO_PIO_SDM | SDM block I/O bank power sense of bank 3A | — | –0.5 | 2.01 | V |
| VCCIO_SDM | SDM block configuration pins power supply | — | –0.5 | 2.08 | V |
| VCCL_SDM | SDM block core voltage power supply | — | –0.5 | 1.07 | V |
| VCCFUSEWR_SDM | SDM block fuse writing power supply | — | –0.5 | 2.4 | V |
| VCCPLLDIG_SDM | SDM block PLL digital power supply | — | –0.5 | 1.07 | V |
| VCCPLL_SDM | SDM block PLL analog power supply | — | –0.5 | 2.08 | V |
| continued... |  |  |  |  |  |

(1) The use of Power Management Bus (PMBus*) voltage regulator dedicated to SmartVID devices is mandatory. The PMBus voltage regulator and SmartVID devices are connected via PMBus.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

5

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Condition | Minimum | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| VCCBAT | Battery back-up power supply (For design security volatile key register) | — | –0.5 | 2.34 | V |
| VCCADC | ADC voltage sensor power supply | — | –0.5 | 2.08 | V |
| VCCIO_PIO | I/O bank power supply | VCCIO_PIO = 1.05 V | –0.5 | 1.43 | V |
|  |  | VCCIO_PIO = 1.1 V | –0.5 | 1.5 | V |
|  |  | VCCIO_PIO = 1.2 V | –0.5 | 1.64 | V |
|  |  | VCCIO_PIO = 1.3 V | –0.5 | 1.74 | V |
| VCCIO_UIB | Power supply for the Universal Interface Bus between the core and embedded HBM2E memory | — | –0.5 | 1.6 | V |
| VCCM_PUMP_HBM | HBM2E power supply | — | –0.5 | 3 | V |
| VCCIO_NOC | NOC configuration pins power supply | — | –0.5 | 2.08 | V |
| VCCPLL_NOC | NOC PLL analog power supply | — | –0.5 | 2.08 | V |
| VCCPLLDIG_NOC | NOC PLL digital power supply | — | –0.5 | 1.07 | V |
| VCCLPLL_NOC | NOC PLL core voltage power supply | — | –0.5 | 1.07 | V |
| VCCH_GXR | Transceiver analog high voltage power | R-Tile devices | –0.5 | 2.03 | V |
| VCCRT_GXR | Transceiver analog power supply | R-Tile devices | –0.5 | 1.33 | V |
| VCCED_GXR | Transceiver digital power supply | R-Tile devices | –0.5 | 1.21 | V |
| VCCE_PLL_GXR | PLLs power supply | R-Tile devices | –0.5 | 1.33 | V |
| VCCE_DTS_GXR | DTS power supply | R-Tile devices | –0.5 | 1.33 | V |
| continued... |  |  |  |  |  |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

6

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Condition | Minimum | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| VCCCLK_GXR | R-Tile reference clock power supply | R-Tile devices | –0.5 | 1.34 | V |
| VCCHFUSE_GXR | R-Tile efuse power supply | R-Tile devices | –0.5 | 1.34 | V |
| VCC_HSSI_GXR | R-Tile digital signal power supply | R-Tile devices | –0.5 | 1.21 | V |
| VCC_HSSI_GXF | F-Tile digital signal power supply | F-Tile devices | –0.5 | 1.07 | V |
| VCCFUSECORE_GXF | F-Tile fuse writing power supply | F-Tile devices | –0.5 | 1.37 | V |
| VCCFUSEWR_GXF | F-Tile efuse power supply | F-Tile devices | –0.5 | 1.37 | V |
| VCCCLK_GXF | F-Tile reference clock power supply | F-Tile devices | –0.5 | 2.04 | V |
| VCCERT1_FHT_GXF | FHT analog core supply 1 | F-Tile devices | –0.5 | 1.33 | V |
| VCCERT2_FHT_GXF | FHT analog core supply 2 | F-Tile devices | –0.5 | 1.33 | V |
| VCCEHT_FHT_GXF | FHT high voltage power supply for analog circuit | F-Tile devices | –0.5 | 1.99 | V |
| VCCERT_FGT_GXF | FGT analog core supply | F-Tile devices | –0.5 | 1.34 | V |
| VCCH_FGT_GXF | FGT analog I/O power supply | F-Tile devices | –0.5 | 2.04 | V |
| VCCERT_GXF_COMBINE | F-Tile reference clock power supply | F-Tile devices | –0.5 | 1.33 | V |
| VCCL_HPS | HPS core voltage and periphery circuitry power supply | — | –0.5 | 1.21 | V |
| VCCPLLDIG_HPS | HPS PLL digital power supply | — | –0.5 | 1.21 | V |
| VCCPLL_HPS | HPS PLL analog power supply | — | –0.5 | 2.08 | V |
| VCCIO_HPS | HPS I/O buffers power supply | — | –0.5 | 2.08 | V |
| continued... |  |  |  |  |  |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

7

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Condition | Minimum | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| VI | DC input voltage | VCCIO_PIO = 1.05 V(2) (3) | –0.3 | VCCIO_PIO(MAX) + 0.25 | V |
|  |  | VCCIO_PIO = 1.1 V(2) (3) | –0.3 | VCCIO_PIO(MAX) + 0.25 | V |
|  |  | VCCIO_PIO = 1.2 V(2) (3) | –0.3 | VCCIO_PIO(MAX) + 0.25 | V |
|  |  | VCCIO_PIO = 1.3 V(2) (3) | –0.3 | VCCIO_PIO(MAX) + 0.25 | V |
|  |  | VCCIO_SDM = 1.8 V | –0.3 | VCCIO_SDM(MAX) + 0.3 | V |
|  |  | VCCIO_HPS = 1.8 V | –0.3 | VCCIO_HPS(MAX) + 0.3 | V |
| IOUT (4) (5) | DC output current per pin | VCCIO_PIO = 1.05 V, 1.1 V, 1.2 V, 1.3 V (6) (7) | –7.5 | 7.5 | mA |
|  |  | VCCIO_SDM, VCCIO_HPS = 1.8 V (8) | –20 | 20 | mA |
| TJ | Absolute junction temperature | — | –55 | 125 | °C |
| TSTG | Storage temperature | — | –55 | 150 | °C |

(2) Applies to LVCMOS I/O standards only. For true differential input, refer to the VICM(min), VICM(max), and VID(max) specifications.

(3) For LVCMOS pin utilization of equal to or less than 25 pins within a bank, the VI (DC) for the LVCMOS input can go up to VCCIO_PIO(MAX) + 0.3 V.

(4) Total current per I/O bank must not exceed 100 mA.

(5) Applies to all I/O standards and settings supported by I/O banks, including single-ended and differential I/Os.

(6) The maximum current allowed through any GPIO-B bank pin during power-up/power-down conditions is 10 mA. Pin voltage during these conditions should not exceed 1.2 V or the VCCIO_PIO supply rail of the bank where the I/O pin resides in, whichever is the lower voltage. While this device is not turned on, the I/O pin should be tri-stated or not driven with any external voltages.

(7) The DC output current per pin may exceed 7.5 mA with a duration limit. For more details, refer to the related information.

(8) The maximum current allowed through any HPS/SDM pin when the device is not turned on or during power-up/power-down conditions is 10 mA. Pin voltage during these conditions should not exceed VCCIO_HPS or VCCIO_SDM supply rail of the bank where the I/O pin resides in.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

8

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Related Information

Agilex 7 General-Purpose I/O User Guide: M-Series

Maximum Allowed Overshoot and Undershoot Voltage

During transitions, the toggling input data or clock signals may overshoot to the voltage listed in the following tables and undershoot to the following limits for input currents less than 100 mA and periods shorter than 20 ns.

  - Undershoot limit of –1.1 V when using VCCIO_HPS or VCCIO_SDM of 1.8 V.

  - Undershoot limit of –0.3 V when using VCCIO_PIO of 1.3 V, 1.2 V , 1.1 V, and 1.05 V.

No overshooting beyond 1.65 V and undershooting below 0.273 V is allowed when using True Differential Signaling I/O standard at VCCIO_PIO = 1.3 V.

No overshooting beyond 1.177 V and undershooting below 0.573 V is allowed when using True Differential Signaling I/O standard at VCCIO_PIO = 1.2 V, 1.1 V, and 1.05 V.

The maximum allowed overshoot duration is specified as a percentage of high time (calculated as ([delta T]/T) × 100) over the lifetime of the device. A DC signal is equivalent to 100% duty cycle.

Table 4.	Maximum Allowed Overshoot During Transitions (for 1.05 V I/O in GPIO-B Bank)

This table lists the maximum allowed input overshoot voltage and the duration of the overshoot voltage as a percentage of device lifetime.

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition (V) | Overshoot Duration as % at TJ = 100°C | Unit |
| --- | --- | --- | --- | --- |
| Vi (AC) | AC input voltage | VCCIO_PIO + 0.25 | 100 | % |
|  |  | VCCIO_PIO + 0.30(9) | 30 | % |
|  |  | VCCIO_PIO + 0.35 | 4 | % |
|  |  | > VCCIO_PIO + 0.40 | No Overshoot Allowed | % |

(9) For LVCMOS pin utilization of equal to or less than 25 pins within a bank, the Vi (AC) for the LVCMOS input can go up to VCCIO_PIO +

0.3 V at an overshoot duration of 100%.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

9

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Table 5.	Maximum Allowed Overshoot During Transitions (for 1.1 V I/O in GPIO-B Bank)

| Symbol | Description | Condition (V) | Overshoot Duration as % at TJ = 100°C | Unit |
| --- | --- | --- | --- | --- |
| Vi (AC) | AC input voltage | VCCIO_PIO + 0.25 | 100 | % |
|  |  | VCCIO_PIO + 0.30(10) | 30 | % |
|  |  | VCCIO_PIO + 0.35 | 4 | % |
|  |  | > VCCIO_PIO + 0.40 | No Overshoot Allowed | % |

This table lists the maximum allowed input overshoot voltage and the duration of the overshoot voltage as a percentage of device lifetime. For specification status, see the Data Sheet Status table

Table 6.	Maximum Allowed Overshoot During Transitions (for 1.2 V I/O in GPIO-B Bank)

This table lists the maximum allowed input overshoot voltage and the duration of the overshoot voltage as a percentage of device lifetime.

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition (V) | Overshoot Duration as % at TJ = 100°C | Unit |
| --- | --- | --- | --- | --- |
| Vi (AC) | AC input voltage | VCCIO_PIO + 0.25 | 100 | % |
|  |  | VCCIO_PIO + 0.30(11) | 30 | % |
|  |  | VCCIO_PIO + 0.35 | 4 | % |
|  |  | > VCCIO_PIO + 0.40 | No Overshoot Allowed | % |

(10) For LVCMOS pin utilization of equal to or less than 25 pins within a bank, the Vi (AC) for the LVCMOS input can go up to VCCIO_PIO +

0.3 V at an overshoot duration of 100%.

(11) For LVCMOS pin utilization of equal to or less than 25 pins within a bank, the Vi (AC) for the LVCMOS input can go up to VCCIO_PIO +

0.3 V at an overshoot duration of 100%.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

10

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Table 7.	Maximum Allowed Overshoot During Transitions (for 1.3 V I/O in GPIO-B Bank)

| Symbol | Description | Condition (V) | Overshoot Duration as % at TJ = 100°C | Unit |
| --- | --- | --- | --- | --- |
| Vi (AC) | AC input voltage | VCCIO_PIO + 0.25 | 100 | % |
|  |  | VCCIO_PIO + 0.30(12) | 65 | % |
|  |  | VCCIO_PIO + 0.35 | 7 | % |
|  |  | > VCCIO_PIO + 0.40 | No Overshoot Allowed | % |

This table lists the maximum allowed input overshoot voltage and the duration of the overshoot voltage as a percentage of device lifetime. For specification status, see the Data Sheet Status table

Table 8.	Maximum Allowed Overshoot During Transitions (for 1.8 V I/O in HPS and SDM I/O Banks)

This table lists the maximum allowed input overshoot voltage and the duration of the overshoot voltage as a percentage of device lifetime.

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition (V) | Overshoot Duration as % at TJ = 100°C | Unit |
| --- | --- | --- | --- | --- |
| Vi (AC) | AC input voltage | VCCIO_SDM + 0.30, VCCIO_HPS + 0.30 | 100 | % |
|  |  | VCCIO_SDM + 0.35, VCCIO_HPS + 0.35 | 60 | % |
|  |  | VCCIO_SDM + 0.40, VCCIO_HPS + 0.40 | 30 | % |
|  |  | VCCIO_SDM + 0.45, VCCIO_HPS + 0.45 | 20 | % |
| continued... |  |  |  |  |

(12) For LVCMOS pin utilization of equal to or less than 25 pins within a bank, the Vi (AC) for the LVCMOS input can go up to VCCIO_PIO +

0.3 V at an overshoot duration of 100%.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

11

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Condition (V) | Overshoot Duration as % at TJ = 100°C | Unit |
| --- | --- | --- | --- | --- |
|  |  | VCCIO_SDM + 0.50, VCCIO_HPS + 0.50 | 10 | % |
|  |  | VCCIO_SDM + 0.55, VCCIO_HPS + 0.55 | 6 | % |
|  |  | > VCCIO_SDM + 0.55, > VCCIO_HPS + 0.55 | No Overshoot Allowed | % |

For example, when using 1.2 V I/O standard with 1.26 V VCCIO_PIO, a signal that overshoots to 1.61 V can only be at 1.61 V for ~4% over the lifetime of the device. For an overshoot of 1.51 V, the percentage of high time for the overshoot can be as high as 100% over the lifetime of the device.

Figure 1.	Overshoot Duration Example (for 1.2 V GPIO-B Bank at VCCIO_PIO = 1.26 V)

DT

T

1.61 V

1.51 V

1.2 V

Recommended Operating Conditions

This section lists the functional operation limits for the AC and DC parameters.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

12

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Recommended Operating Conditions

Table 9.	Recommended Operating Conditions

This table lists the steady-state voltage values expected. Power supply ramps must all be strictly monotonic, without plateaus.

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition | Minimum(13) | Typical | Maximum(13) | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VCC | Core voltage power supply | SmartVID (14): –1V, – 2V, –3V, –3E | (Typical) – 3% | 0.70 – 0.90(15) | (Typical) + 3% | V |
| VCCP | Periphery circuitry power supply | SmartVID(14): –1V, – 2V, –3V, –3E | (Typical) – 3% | 0.70 – 0.90(15) | (Typical) + 3% | V |
| VCCPT | Power supply for I/O PLL and I/O pre-driver | — | 1.71 | 1.8 | 1.89 | V |
| VCCRCORE | CRAM power supply | — | 1.14 | 1.2 | 1.26 | V |
| VCCH | Advanced interface bus (AIB) power supply | Devices with R-Tile and F-Tile | 0.776 | 0.8 | 0.824 | V |
|  |  | Devices with F-Tile only | 0.776 | 0.8 | 0.824 | V |
| VCCH_SDM | SDM block transceiver digital power sense | Devices with R-Tile and F-Tile | 0.87 | 0.9 | 0.93 | V |
|  |  | Devices with F-Tile only | 0.776 | 0.8 | 0.824 | V |
| continued... |  |  |  |  |  |  |

(13) This value describes the required voltage measured between the PCB power and ground ball during normal device operation. The voltage ripple includes both regulator DC ripple and the dynamic noise.

(14) The use of Power Management Bus (PMBus) voltage regulator dedicated to SmartVID devices is mandatory. The PMBus voltage regulator and SmartVID devices are connected via PMBus.

(15) The typical value is based on the SmartVID programmed value.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

13

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Condition | Minimum(13) | Typical | Maximum(13) | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VCCIO_UIB | Power supply for the Universal Interface Bus between the core and embedded HBM2E memory | — | 1.17 | 1.2 | 1.23 | V |
| VCCIO_PIO_SDM (16) | SDM block I/O bank power sense of Bank 3A | 1.05 V(17) | 1.0185 | 1.05 | 1.0815 | V |
|  |  | 1.1 V(17) | 1.067 | 1.1 | 1.133 | V |
|  |  | 1.2 V(17) | 1.164 | 1.2 | 1.236 | V |
|  |  | 1.3 V | 1.261 | 1.3 | 1.339 | V |
| VCCIO_SDM | SDM block configuration pins power supply | — | 1.71 | 1.8 | 1.89 | V |
| VCCL_SDM | SDM block core voltage power supply | — | 0.776 | 0.8 | 0.824 | V |
| VCCFUSEWR_SDM | SDM block fuse writing power supply | — | 1.75 | 1.8 | 1.85 | V |
| VCCPLLDIG_SDM | SDM block PLL digital power supply | — | 0.776 | 0.8 | 0.824 | V |
| continued... |  |  |  |  |  |  |

(13) This value describes the required voltage measured between the PCB power and ground ball during normal device operation. The voltage ripple includes both regulator DC ripple and the dynamic noise.

(16) Must be powered up with the same voltage level as VCCIO_PIO_3AT at 1.2 V when using Avalon® streaming x16/x32 configuration schemes. Tie to VCCRCORE when not using Avalon streaming x16/x32 configuration schemes, use the tighter tolerance requirement when the power rails have different tolerances.

(17) Each sub-bank can only support a single voltage tolerance. The VCCIO_PIO tolerance can be extended to ±5% if the entire GPIO-B sub- bank is operating in any of the following modes:

- LVDS SERDES receiver mode with the use of 1.05 V, 1.1 V, 1.2 V True Differential Signaling input standard

- PHYLITE mode

- GPIO mode

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

14

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Condition | Minimum(13) | Typical | Maximum(13) | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VCCPLL_SDM | SDM block PLL analog power supply | — | 1.71 | 1.8 | 1.89 | V |
| VCCM_PUMP_HBM | HBM2E power supply | — | 2.4 | 2.5 | 2.6 | V |
| VCCBAT (18) | Battery back-up power supply (For design security volatile key register) | — | 1 | — | 1.8 | V |
| IBAT | Battery back-up power supply (For design security volatile key register) | VCCBAT = 1.2 V | — | — | 200 | nA |
| VCCADC | ADC voltage sensor power supply | — | 1.71 | 1.8 | 1.89 | V |
| VCCIO_PIO | I/O bank power supply | 1.05 V(17) | 1.0185 | 1.05 | 1.0815 | V |
|  |  | 1.1 V(17) | 1.067 | 1.1 | 1.133 | V |
|  |  | 1.2 V(17) | 1.164 | 1.2 | 1.236 | V |
|  |  | 1.3 V | 1.261 | 1.3 | 1.339 | V |
| VCCIO_NOC | NOC configuration pins power supply | — | 1.71 | 1.8 | 1.89 | V |
| VCCPLL_NOC | NOC PLL analog power supply | — | 1.71 | 1.8 | 1.89 | V |
| VCCPLLDIG_NOC | NOC PLL digital power supply | — | 0.776 | 0.8 | 0.824 | V |
| VCCLPLL_NOC | NOC PLL core voltage power supply | — | 0.776 | 0.8 | 0.824 | V |
| continued... |  |  |  |  |  |  |

(13) This value describes the required voltage measured between the PCB power and ground ball during normal device operation. The voltage ripple includes both regulator DC ripple and the dynamic noise.

(18) Power up VCCBAT with a non-volatile battery power source when using the device security AES BBRAM key. When not using the AES BBRAM key, tie this pin to ground.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

15

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Condition | Minimum(13) | Typical | Maximum(13) | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VI (19) | DC input voltage | VCCIO_PIO = 1.05 V(20) (21) | –0.3 | — | VCCIO_PIO + 0.25 | V |
|  |  | VCCIO_PIO = 1.1 V(20) (21) | –0.3 | — | VCCIO_PIO + 0.25 | V |
|  |  | VCCIO_PIO = 1.2 V(20) (21) | –0.3 | — | VCCIO_PIO + 0.25 | V |
|  |  | VCCIO_PIO = 1.3 V(20) (21) | –0.3 | — | VCCIO_PIO + 0.25 | V |
|  |  | VCCIO_SDM = 1.8 V | –0.3 | — | VCCIO_SDM + 0.3 | V |
|  |  | VCCIO_HPS = 1.8 V | –0.3 | — | VCCIO_HPS + 0.3 | V |
| VO | Output voltage | VCCIO_PIO = 1.05 V, 1.1 V, 1.2 V, 1.3 V | 0 | — | VCCIO_PIO | V |
|  |  | VCCIO_SDM = 1.8 V | 0 | — | VCCIO_SDM | V |
|  |  | VCCIO_HPS = 1.8 V | 0 | — | VCCIO_HPS | V |
| continued... |  |  |  |  |  |  |

(13) This value describes the required voltage measured between the PCB power and ground ball during normal device operation. The voltage ripple includes both regulator DC ripple and the dynamic noise.

(19) This value applies to both input and tri-stated output configuration. Pin voltage should not be externally pulled higher than the maximum value.

(20) Applies to LVCMOS I/O standards only. For true differential input, refer to the VICM(min), VICM(max), and VID(max) specifications.

(21) For LVCMOS pin utilization of equal to or less than 25 pins within a bank, the VI (DC) for the LVCMOS input can go up to VCCIO_PIO +

0.3 V.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

16

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Condition | Minimum(13) | Typical | Maximum(13) | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| TJ | Operating junction temperature | Extended | 0(22) | — | 100(23) (24) | °C |
| IFUSEWR_SDM | SDM fuse Current | Static | — | — | 50 | mA |
|  |  | Dynamic average AC during sensing and programming | — | 0.6 | — | mA |
| tRAMP (25) (26) | Power supply ramp time | Standard POR | 200 μs | — | 100 ms | — |

(13) This value describes the required voltage measured between the PCB power and ground ball during normal device operation. The voltage ripple includes both regulator DC ripple and the dynamic noise.

(22) Agilex™ 7 M-Series devices are generally offered in Extended temperature range only. If an industrial temperature range is required, note that you can configure these devices at less than 0°C, but the HBM2E interface will be held reset and not calibrated until Tj reaches 0°C. Contact your sales representative for the availability of Agilex 7 M-Series Industrial temperature range devices.

(23) When using the device at TJ = 100°C, the device can operate under the recommended operating conditions over a minimum device lifetime of 11.4 years.

(24) The recommended maximum operating temperature for HBM2E devices is 95°C.

(25) tRAMP is the ramp time of each individual power supply, not the ramp time of all combined power supplies.

(26) To support AS fast mode, all power supplies to the device must be fully ramped-up within 10 ms to the recommended operating conditions.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

17

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

R-Tile Transceiver Power Supply Recommended Operating Conditions

Table 10.	R-Tile Transceiver Power Supply Recommended Operating Conditions

For specification status, see the Data Sheet Status table

| Symbol | Description | Typical DC Level (V) | Recommended DC Setpoint (% of Vnominal) | Recommended VR Ripple (% of Vnominal) | Recommended AC Transient (% of Vnominal) | Maximum (DC Setpoint + Ripple + AC Transient) (% of Vnominal) | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VCCH_GXR[L,R] (27) | Transceiver analog high voltage power | 1.8 | ±0.5% | ±2% |  | ±2.5% | V |
| VCCRT_GXR[L,R] | Transceiver analog power supply | 1 | ±0.5% | ±2% |  | ±2.5% | V |
| VCCED_GXR[L,R] | Transceiver digital power supply | 0.9 | ±0.8% | ±2.5% |  | ±3.3% | V |
| VCCE_PLL_GXR[L,R] | PLLs power supply | 1 | ±0.5% | ±1.5% |  | ±2% | V |
| VCCE_DTS_GXR[L,R] | DTS power supply | 1 | ±0.5% | ±1.5% |  | ±2% | V |
| VCCCLK_GXR[L,R] (27) | Reference clock power supply | 1 | ±0.5% | ±2.5% |  | ±3% | V |
| VCCHFUSE_GXR | R-tile efuse power supply | 1 | ±0.5% | ±2.5% |  | ±3% | V |
| VCC_HSSI_GXR | Digital signal power supply | 0.9 | ±0.8% | ±2.5% |  | ±3.3% | V |

Related Information

Agilex™ 7 Power Distribution Network Design Guidelines Provides the PCB design guidelines.

(27) Follow the more stringent tolerance range for the voltage rails connecting multiple power supplies.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

18

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

  - ile Transceiver Power Supply Recommended Operating Conditions

Table 11.	F-Tile Transceiver Power Supply Recommended Operating Conditions

To prevent F-Tile performance degradation, devices with F-Tile must not remain in a powered-up and unconfigured state for a cumulative time exceeding 12 months.

For specification status, see the Data Sheet Status table

| Symbol | Description | Typical DC Level (V) | Recommended DC Setpoint (% of Vnominal) | Recommended VR Ripple (% of Vnominal) | Recommended AC Transient (% of Vnominal) | Maximum (DC Setpoint + Ripple + AC Transient) (% of Vnominal) | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VCC_HSSI_GXF | F-Tile digital signal power supply | 0.8 | ±0.5% | ±0.5% | ±2%(28) | ±3% | V |
| VCCFUSECORE_GXF | F-Tile fuse writing power supply | 1 | ±0.5% | ±4.5% |  | ±5% | V |
| VCCFUSEWR_GXF | F-Tile efuse power supply | 1 | ±0.5% | ±4.5% |  | ±5% | V |
| VCCCLK_GXF | Reference clock power supply | 1.8 | ±0.5% | ±0.5% | ±2% | ±3% | V |
| VCCERT1_FHT_GXF (29) | FHT analog core supply 1 | 1 | ±0.5% | ±0.5% | ±1.5% | ±2.5% | V |
| VCCERT2_FHT_GXF (30) | FHT analog core supply 2 | 1 | ±0.5% | ±0.5% | ±1.5% | ±2.5% | V |
| VCCEHT_FHT_GXF (31) | FHT high voltage power supply for analog circuit | 1.5 | ±0.5% | ±0.5% | +1%/–1.5% | +2%/–2.5% | V |
| continued... |  |  |  |  |  |  |  |

(28) Board design can use 2.5% as indicated by AN910 if the simulation results include both VR ripple and AC transcient.

(29)   HF noise requires AC 10 mVpp above 1 MHz; switching regulator ripple switching frequency: <1 MHz, ripple: ≤5 mVpp.

(30)   HF noise requires AC 30 mVpp above 1 MHz; switching regulator ripple switching frequency: <1 MHz, ripple: ≤5 mVpp.

(31)   HF noise requires AC 30 mVpp above 1 MHz; switching regulator ripple switching frequency: <1 MHz, ripple: ≤7 mVpp.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

19

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Typical DC Level (V) | Recommended DC Setpoint (% of Vnominal) | Recommended VR Ripple (% of Vnominal) | Recommended AC Transient (% of Vnominal) | Maximum (DC Setpoint + Ripple + AC Transient) (% of Vnominal) | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VCCERT_FGT_GXF (32) | FGT analog core supply alone | 1 | ±0.5% | ±0.5% | ±2% | ±3% | V |
|  | FGT analog core supply when combined with VCCERT1_FHT_GX and VCCERT2_FHT_GXF 1 V supply | 1 | ±0.5% | ±0.5% | ±1.5% | ±2.5% | V |
| VCCH_FGT_GXF (32) | FGT analog I/O power supply | 1.8 | ±0.5% | ≤5 mV | ±1.5% | ±3% | V |

Related Information

AN 910: Agilex 7 Power Distribution Network Design Guidelines Provides the PCB design guidelines.

HPS Power Supply Operating Conditions

Table 12.	HPS Power Supply Operating Conditions

This table lists the steady-state voltage and current values expected for system-on-a-chip (SoC) devices with ARM-based hard processor system (HPS). Power supply ramps must all be strictly monotonic, without plateaus. Refer to the Recommended Operating Conditions table for the steady-state voltage values expected from the FPGA portion of the SoC devices.

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VCCL_HPS | HPS core voltage and periphery circuitry power supply | Performance boost, fixed voltage: –1V | (Typical) – 3% | 0.95 | (Typical) + 3% | V |
|  |  | SmartVID: –1V, –2V, – 3V, –3E (33) | (Typical) – 3% | 0.70 – 0.90 | (Typical) + 3% | V |
| continued... |  |  |  |  |  |  |

(32) HF noise requires AC 30 mVpp above 1 MHz; switching regulator ripple: ≤5 mVpp.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

20

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VCCPLLDIG_HPS | HPS PLL digital power supply (can be connected to VCCL_HPS) | Performance boost, fixed voltage: –1V | (Typical) – 3% | 0.95 | (Typical) + 3% | V |
|  |  | SmartVID: –1V, –2V, – 3V, –3E (33) | (Typical) – 3% | 0.70 – 0.90 | (Typical) + 3% | V |
| VCCPLL_HPS | HPS PLL analog power supply | 1.8 V | 1.71 | 1.8 | 1.89 | V |
| VCCIO_HPS | HPS I/O buffers power supply | 1.8 V | 1.71 | 1.8 | 1.89 | V |

Related Information

    - Recommended Operating Conditions on page 13

Provides the steady-state voltage values for the FPGA portion of the device.

    - HPS Clock Performance on page 84

DC Characteristics

Supply Current and Power Consumption

Altera offers two ways to estimate power for your design—the Power and Thermal Calculator (PTC) and the Quartus® Prime Power Analyzer feature.

Use the PTC before you start your design to estimate the supply current for your design. The PTC provides a magnitude estimate of the device power because these currents vary greatly with the usage of the resources.

The Quartus Prime Power Analyzer provides better quality estimates based on the specifics of the design after you complete place-and-route. The Power Analyzer can apply a combination of user-entered, simulation-derived, and estimated signal activities that, when combined with detailed circuit models, yield very accurate power estimates.

(33) The use of Power Management Bus (PMBus) voltage regulator dedicated to SmartVID devices is mandatory. The PMBus voltage regulator and SmartVID devices are connected via PMBus.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

21

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

GPIO-B DC Characteristics

GPIO-B I/O Pin Leakage Current

Table 13.	GPIO-B I/O Pin Leakage Current

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition | Minimum | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| II | Input pin | VI = 0 V to VCCIO_PIO (MAX) | –360 | 360 | µA |
| IOZ | Tri-stated I/O pin | VO = 0 V to VCCIO_PIO (MAX) | –360 | 360 | µA |

GPIO-B OCT Calibration Accuracy Specifications

If you enable on-chip termination (OCT) calibration, calibration is automatically performed at power up for I/Os connected to the calibration block.

Table 14.	GPIO-B OCT Calibration Accuracy Specifications

Calibration accuracy for the calibrated on-chip series termination (RS OCT) and on-chip parallel termination (RT OCT) are applicable at the moment of calibration. When process, voltage, and temperature (PVT) conditions change after calibration, the tolerance may change.

These specifications require RZQ reference accuracy of 240 Ω ±1%.

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition (V) | Calibration Accuracy | Unit |
| --- | --- | --- | --- | --- |
| 34-Ω and 40-Ω RS (34) | Internal series termination with calibration (34-Ω and 40-Ω setting) | SSTL-12, HSTL-12, HSUL-12, and POD12 I/O standards | 20 | % |
|  |  | POD11 and LVSTL11 I/O standards | 20 | % |
|  |  | LVSTL105 I/O standard | 20 | % |
| 40-Ω RS (34) | Internal series termination with calibration (40-Ω setting) | LVSTL700 I/O standard | 20 | % |
| continued... |  |  |  |  |

(34) This specification applies to both single-ended and pseudo-differential I/O buffers.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

22

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Condition (V) | Calibration Accuracy | Unit |
| --- | --- | --- | --- | --- |
| 50-Ω and 60-Ω RT (34) | Internal parallel termination with calibration (50-Ω and 60-Ω setting) | SSTL-12 and HSTL-12 I/O standards | 20 | % |
| 40-Ω, 50-Ω, and 60-Ω RT(34) | Internal parallel termination with calibration (40-Ω, 50-Ω, and 60- Ω setting) | POD11 and POD12 I/O standards | 20 | % |
|  |  | LVSTL11, LVSTL105, and LVSTL700 I/O standards | 20 | % |

GPIO-B OCT Without Calibration Resistance Tolerance Specifications

Table 15.	GPIO-B OCT Without Calibration Resistance Tolerance Specifications

This table lists the GPIO OCT without calibration resistance tolerance to PVT changes.

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition (V) | Calibration Accuracy | Unit |
| --- | --- | --- | --- | --- |
| 34-Ω and 40-Ω RS | Internal series termination without calibration (34-Ω and 40-Ω setting) | LVCMOS13 I/O standard | 30 | % |
| 34-Ω and 40-Ω RS (35) | Internal series termination without calibration (34-Ω and 40-Ω setting) | LVCMOS12, SSTL-12, HSTL-12, HSUL-12, and POD12 I/O standards | 25 | % |
|  |  | LVCMOS11, POD11, and LVSTL11 I/O standards | 25 | % |
|  |  | LVCMOS105 and LVSTL105 I/O standards | 25 | % |
| 50-Ω RT (35) | Internal parallel termination without calibration (50-Ω setting) | SSTL-12 and HSTL-12 I/O standards | 25 | % |
|  |  | POD11 and POD12 I/O standards | 25 | % |
| continued... |  |  |  |  |

(35) This specification applies to both single-ended and pseudo-differential I/O buffers.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

23

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Condition (V) | Calibration Accuracy | Unit |
| --- | --- | --- | --- | --- |
|  |  | LVSTL11 and LVSTL105 I/O standards | 25 | % |
| 100-Ω RD (36) | Internal differential termination (100-Ω setting) | True differential signaling I/O standard at VCCIO_PIO = 1.05 | 40 | % |
|  |  | True differential signaling I/O standard at VCCIO_PIO = 1.1 | 40 | % |
|  |  | True differential signaling I/O standard at VCCIO_PIO = 1.2 | 40 | % |
|  |  | True differential signaling I/O standard at VCCIO_PIO = 1.3 | 40 | % |
| 100-Ω RD | Internal differential termination (100-Ω setting) | SLVS400 I/O standard | 30 | % |

GPIO-B Pin Capacitance

Table 16.	GPIO-B Pin Capacitance

For specification status, see the Data Sheet Status table

| Symbol | Description | Maximum | Unit |
| --- | --- | --- | --- |
| CIO | Input/output capacitance of I/O pins | 2.6(37) | pF |

GPIO-B Internal Weak Pull-Up Resistor

All I/O pins in GPIO bank have an option to enable weak pull-up when using 1.05 V, 1.1 V, 1.2 V, and 1.3 V LVCMOS I/O standards.

(36) This specification applies to VICM(DC) ≤ 1.3V. For VICM(DC) > 1.3V, a specification range of -60% to +40% applies.

(37) This value refers to die-level pin capacitance without the device package.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

24

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Table 17.	GPIO-B Internal Weak Pull-Up Resistor

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition (V) | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| RPU | Value of the I/O pin pull-up resistor before | VCCIO_PIO = 1.3 ±3% | 3 | 10 | 30 | kΩ |
|  |  | VCCIO_PIO = 1.2 ±5% | 3 | 10 | 30 | kΩ |
|  | and during |  |  |  |  |  |
|  | configuration, as well |  |  |  |  |  |
|  |  | VCCIO_PIO = 1.1 ±5% | 3 | 10 | 30 | kΩ |
|  | as user mode if you |  |  |  |  |  |
|  | have enabled the programmable pull-up |  |  |  |  |  |
|  |  | VCCIO_PIO = 1.05 ±5% | 3 | 10 | 30 | kΩ |
|  | resistor option. |  |  |  |  |  |

HPS I/O DC Characteristics HPS I/O Pin Leakage Current

Table 18.	HPS I/O Pin Leakage Current

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition | Minimum | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| II | Input pin | VI = 0 V to VCCIO_HPS (MAX) | −15 | 15 | µA |
|  | Tri-stated I/O pin | VO = 0 V to VCCIO_HPS (MAX) | −15 | 15 | µA |

HPS I/O Pin Capacitance

Table 19.	HPS I/O Pin Capacitance

For specification status, see the Data Sheet Status table

| Symbol | Description | Maximum | Unit |
| --- | --- | --- | --- |
| CIO | Input/output capacitance of I/O pins | 5(38) | pF |

HPS I/O Internal Weak Pull-Up Resistor

The I/O pins in HPS bank are supported with weak pull-up and weak pull-down options.

(38) This value refers to die-level pin capacitance without the device package.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

25

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Table 20.	HPS Internal Weak Pull-Up Resistor

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition (V) | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| 20 kΩ RPU, 20 kΩ RPD | Value of the I/O pin pull-up and pull-down resistor during user mode if you have enabled the programmable pull-up or pull-down resistor option. | VCCIO_HPS = 1.8 ±5% | 15 | 20 | 25 | kΩ |
| 50 kΩ RPU, 50 kΩ RPD | Value of the I/O pin pull-up and pull-down resistor during user mode if you have enabled the programmable pull-up or pull-down resistor option. | VCCIO_HPS = 1.8 ±5% | 37.5 | 50 | 62.5 | kΩ |
| 80 kΩ RPU, 80 kΩ RPD | Value of the I/O pin pull-up and pull-down resistor during user mode if you have enabled the programmable pull-up or pull-down resistor option. | VCCIO_HPS = 1.8 ±5% | 57 | 80 | 105 | kΩ |

HPS I/O Hysteresis Specifications for Schmitt Trigger Input

Table 21.	HPS I/O Hysteresis Specifications for Schmitt Trigger Input

This device supports Schmitt trigger input on HPS I/O bank. A Schmitt trigger feature introduces hysteresis to the input signal for improved noise immunity, especially for signal with slow edge rate.

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VHYS | Hysteresis for Schmitt trigger input | VCCIO_HPS = 1.8 V | 180 | − | − | mV |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

26

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

SDM I/O DC Characteristics SDM I/O Pin Leakage Current

Table 22.	SDM I/O Pin Leakage Current

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition | Minimum | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| II | Input pin | VI = 0 V to VCCIO_SDM (MAX) | −15 | 15 | µA |
|  | Tri-stated I/O pin | VO = 0 V to VCCIO_SDM (MAX) | −15 | 15 | µA |

SDM I/O Pin Capacitance

Table 23.	SDM I/O Pin Capacitance

For specification status, see the Data Sheet Status table

| Symbol | Description | Maximum | Unit |
| --- | --- | --- | --- |
| CIO | Input/output capacitance of I/O pins | 5(39) | pF |

SDM I/O Internal Weak Pull-Up Resistor

The I/O pins in SDM bank are supported with weak pull-up and weak pull-down feature. The weak pull-up and weak pull-down feature is pre-configured according to the configuration mode.

Table 24.	SDM I/O Internal Weak Pull-Up Resistor

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition (V) | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| 20 kΩ RPU, 20 kΩ RPD | Value of the I/O pin pull-up and pull-down resistor during user mode if you have enabled the programmable pull-up or pull-down resistor option. | VCCIO_SDM = 1.8 ±5% | 15 | 20 | 25 | kΩ |

(39) This value refers to die-level pin capacitance without the device package.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

27

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

SDM I/O Hysteresis Specifications for Schmitt Trigger Input

Table 25.	SDM I/O Hysteresis Specifications for Schmitt Trigger Input

This device supports Schmitt trigger input on SDM I/O bank. A Schmitt trigger feature introduces hysteresis to the input signal for improved noise immunity, especially for signal with slow edge rate.

For specification status, see the Data Sheet Status table

| Symbol | Description | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VHYS | Hysteresis for Schmitt trigger input | VCCIO_SDM = 1.8 V | 180 | − | − | mV |

I/O Standard Specifications

GPIO-B I/O Standard Specifications

Tables in this section list the supported input voltage (VIH and VIL), output voltage (VOH and VOL), and current drive characteristics (IOH and IOL) for various I/O standards.

For minimum voltage values, use the minimum VCCIO_PIO values. For maximum voltage values, use the maximum VCCIO_PIO values.

You must perform timing closure analysis to determine the maximum achievable frequency for general purpose I/O standards.

Related Information

Recommended Operating Conditions on page 13

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

28

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

GPIO-B Single-Ended I/O Standards Specifications

Table 26.	GPIO-B Single-Ended I/O Standards Specifications

For specification status, see the Data Sheet Status table

| I/O Standard | VCCIO_PIO (V) |  |  | VIL (V) |  | VIH (V) |  | VOL (V) (40) | VOH (V)(40) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minimum | Typical | Maximum | Minimum | Maximum | Minimum | Maximum(41) | Maximum | Minimum |
| 1.3 V LVCMOS | 1.261 | 1.3 | 1.339 | –0.3 | 0.35 × VCCIO_PIO | 0.65 × VCCIO_PIO | VCCIO_PIO + 0.25 | 0.25 × VCCIO_PIO | 0.75 × VCCIO_PIO |
| 1.2 V LVCMOS | 1.14 | 1.2 | 1.26 | –0.3 | 0.35 × VCCIO_PIO | 0.65 × VCCIO_PIO | VCCIO_PIO + 0.25 | 0.25 × VCCIO_PIO | 0.75 × VCCIO_PIO |
| 1.1 V LVCMOS | 1.045 | 1.1 | 1.155 | –0.3 | 0.35 × VCCIO_PIO | 0.65 × VCCIO_PIO | VCCIO_PIO + 0.25 | 0.25 × VCCIO_PIO | 0.75 × VCCIO_PIO |
| 1.05 V LVCMOS | 0.9975 | 1.05 | 1.1025 | –0.3 | 0.35 × VCCIO_PIO | 0.65 × VCCIO_PIO | VCCIO_PIO + 0.25 | 0.25 × VCCIO_PIO | 0.75 × VCCIO_PIO |

GPIO-B Single-Ended SSTL, HSTL, HSUL, POD and LVSTL I/O Reference Voltage Specifications

Table 27.	GPIO-B Single-Ended SSTL, HSTL, HSUL, POD and LVSTL I/O Reference Voltage Specifications

For specification status, see the Data Sheet Status table

| I/O Standard | VCCIO_PIO (V) |  |  | Internal VREF (V) |  |  | VTT (V) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minimum | Typical | Maximum | Minimum | Typical | Maximum | Minimum | Typical | Maximum |
| SSTL-12 | 1.14 | 1.2 | 1.26 | 0.49 × VCCIO_PIO | 0.5 × VCCIO_PIO | 0.51 × VCCIO_PIO | 0.45 × VCCIO_PIO | 0.5 × VCCIO_PIO | 0.55 × VCCIO_PIO |
| HSTL-12 | 1.14 | 1.2 | 1.26 | 0.47 × VCCIO_PIO | 0.5 × VCCIO_PIO | 0.53 × VCCIO_PIO | 0.45 × VCCIO_PIO | 0.5 × VCCIO_PIO | 0.55 × VCCIO_PIO |
| continued... |  |  |  |  |  |  |  |  |  |

(40) Applicable to test condition of IOH and IOL at 2 mA.

(41) For LVCMOS pin utilization of equal to or less than 25 pins within a bank, the VIH(max) for the LVCMOS input can go up to VCCIO_PIO +

0.3 V.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

29

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| I/O Standard | VCCIO_PIO (V) |  |  | Internal VREF (V) |  |  | VTT (V) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minimum | Typical | Maximum | Minimum | Typical | Maximum | Minimum | Typical | Maximum |
| HSUL-12(42) | 1.14 | 1.2 | 1.26 | 0.49 × VCCIO_PIO | 0.5 × VCCIO_PIO | 0.51 × VCCIO_PIO | 0.45 × VCCIO_PIO | 0.5 × VCCIO_PIO | 0.55 × VCCIO_PIO |
| POD12 (GPIO)(43) (44) | 1.164 | 1.2 | 1.236 | 0.69 × VCCIO_PIO | 0.7 × VCCIO_PIO | 0.71 × VCCIO_PIO | — | VCCIO_PIO | — |
| POD12 (PHYLITE)(43) (44) | 1.164 | 1.2 | 1.236 | 0.74 × VCCIO_PIO | 0.75 × VCCIO_PIO | 0.76 × VCCIO_PIO | — | VCCIO_PIO | — |
| POD11 (GPIO)(43) (44) | 1.067 | 1.1 | 1.133 | 0.69 × VCCIO_PIO | 0.7 × VCCIO_PIO | 0.71 × VCCIO_PIO | — | VCCIO_PIO | — |
| POD11 (PHYLITE)(43) (44) | 1.067 | 1.1 | 1.133 | 0.74 × VCCIO_PIO | 0.75 × VCCIO_PIO | 0.76 × VCCIO_PIO | — | VCCIO_PIO | — |
| LVSTL11(43) | 1.067 | 1.1 | 1.133 | 0.24 × VCCIO_PIO | 0.25 × VCCIO_PIO | 0.26 × VCCIO_PIO | — | GND | — |
| LVSTL105(43) | 1.0185 | 1.05 | 1.0815 | 0.24 × VCCIO_PIO | 0.25 × VCCIO_PIO | 0.26 × VCCIO_PIO | — | GND | — |
| LVSTL700 | 1.0185 | 1.05 | 1.0815 | 0.174 × VCCIO_PIO | 0.184 × VCCIO_PIO | 0.194 × VCCIO_PIO | — | GND | — |

(42) Usage of on-board receiver termination is optional.

(43) Each sub-bank can only support a single voltage tolerance. The VCCIO_PIO tolerance can be extended to ±5% if the entire GPIO-B sub- bank is operating in any of the following modes:

- PHYLITE mode

- GPIO mode

(44) For I/O lane with mixture of GPIO and PHYLITE interfaces, the VREF specification for that I/O lane follows the PHYLITE VREF specifications.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

30

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

GPIO-B Single-Ended SSTL, HSTL, HSUL, and POD I/O Standards Signal Specifications

Table 28.	GPIO-B Single-Ended SSTL, HSTL, HSUL, and POD I/O Standards Signal Specifications

For specification status, see the Data Sheet Status table

| I/O Standard | VIL(DC) (V) | VIH(DC) (V) | VIL(AC) (V) | VIH(AC) (V) |
| --- | --- | --- | --- | --- |
|  | Maximum | Minimum | Maximum | Minimum |
| SSTL-12 | VREF – 0.075 | VREF + 0.075 | VREF – 0.100 | VREF + 0.100 |
| HSTL-12 | VREF – 0.080 | VREF + 0.080 | VREF – 0.150 | VREF + 0.150 |
| HSUL-12 | VREF – 0.100 | VREF + 0.100 | VREF – 0.135 | VREF + 0.135 |
| POD12 | VREF – 0.055 | VREF + 0.055 | VREF – 0.070 | VREF + 0.070 |
| POD11 | VREF – 0.055 | VREF + 0.055 | VREF – 0.070 | VREF + 0.070 |

Note:	For output voltage swing calculation example, refer to the related information. Differential voltage referenced I/O standard uses two single-ended outputs with second output programmed as inverted.

Note:	For eye height position estimation in EMIF interfaces, refer to the External Memory Interfaces Agilex™ 7 M-Series FPGA IP User Guide. The eye mask estimation methodology defined in the External Memory Interfaces Agilex™ 7 M-Series FPGA IP User Guide takes precedence over specifications in GPIO-B Single-Ended SSTL, HSTL, HSUL, and POD I/O Standards Signal Specifications table.

Related Information

1.05 V, 1.1 V, or 1.2 V I/O Interface Voltage Level Compatibility section, Agilex 7 General-Purpose I/O User Guide: M-Series Provides output voltage swing calculation examples.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

31

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

GPIO-B Single-Ended LVSTL I/O Standards Specifications

Table 29.	GPIO-B Single-Ended LVSTL I/O Standards Specifications

For specification status, see the Data Sheet Status table

| I/O Standard | VCCIO_PIO (V) |  |  | VIL(DC) (V) | VIH(DC) (V) | VIL(AC) (V) | VIH(AC) (V) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minimum | Typical | Maximum | Maximum | Minimum | Maximum | Minimum |
| LVSTL11(45) | 1.067 | 1.1 | 1.133 | VREF – 0.055 | VREF + 0.055 | VREF – 0.070 | VREF + 0.070 |
| LVSTL105(45) | 1.0185 | 1.05 | 1.0815 | VREF – 0.055 | VREF + 0.055 | VREF – 0.070 | VREF + 0.070 |
| LVSTL700 | 1.0185 | 1.05 | 1.0815 | VREF – 0.055 | VREF + 0.055 | VREF – 0.070 | VREF + 0.070 |

Note:	For eye height position estimation in EMIF interfaces, refer to the External Memory Interfaces Agilex™ 7 M-Series FPGA IP User Guide. The eye mask estimation methodology defined in the External Memory Interfaces Agilex™ 7 M-Series FPGA IP User Guide takes precedence over specifications in GPIO-B Single-Ended LVSTL I/O Standards Specifications table.

Related Information

External Memory Interfaces Agilex 7 M-Series FPGA IP User Guide Provides eye mask estimation for EMIF interfaces.

(45) CCIO_PIO tolerance can be extended to ±5% if the entire GPIO-B sub-bank is operating in any of the following modes:Each sub-bank can only support a single voltage tolerance. The V

- PHYLITE mode

- GPIO mode

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

32

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

GPIO-B Differential SSTL, HSTL, and HSUL I/O Standards Specifications

Table 30.	GPIO-B Differential SSTL, HSTL, and HSUL I/O Standards Specifications

For specification status, see the Data Sheet Status table

| I/O Standard | VCCIO_PIO (V) |  |  | VILdiff(DC) (V) | VIHdiff(DC) (V) | VILdiff(AC) (V) | VIHdiff(AC) (V) | VIX(AC) (V) |  |  | VOX(AC) (V) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minimum | Typical | Maximum | Maximum | Minimum | Maximum | Minimum | Minimum | Typical | Maximum | Minimum | Typical | Maximum |
| SSTL-12(4 6) | 1.14 | 1.2 | 1.26 | –0.15 | 0.15 | –0.2 | 0.2 | 0.5 × VCCIO_PIO – 0.12 | 0.5 × VCCIO_PIO | 0.5 × VCCIO_PIO + 0.12 | 0.5 × VCCIO_PIO – 0.12 | 0.5 × VCCIO_PIO | 0.5 × VCCIO_PIO + 0.12 |
| HSTL-12(4 6) | 1.14 | 1.2 | 1.26 | –0.16 | 0.16 | –0.3 | 0.3 | 0.5 × VCCIO_PIO – 0.12 | 0.5 × VCCIO_PIO | 0.5 × VCCIO_PIO + 0.12 | 0.5 × VCCIO_PIO – 0.12 | 0.5 × VCCIO_PIO | 0.5 × VCCIO_PIO + 0.12 |
| HSUL-12( 46) | 1.14 | 1.2 | 1.26 | –0.2 | 0.2 | –0.27 | 0.27 | 0.5 × VCCIO_PIO – 0.12 | 0.5 × VCCIO_PIO | 0.5 × VCCIO_PIO + 0.12 | 0.5 × VCCIO_PIO – 0.12 | 0.5 × VCCIO_PIO | 0.5 × VCCIO_PIO + 0.12 |

GPIO-B Differential POD I/O Standards Specifications

Table 31.	GPIO-B Differential POD I/O Standards Specifications

For specification status, see the Data Sheet Status table

| I/O Standard | VCCIO_PIO (V) |  |  | VILdiff(DC) (V) | VIHdiff(DC) (V) | VILdiff(AC) (V) | VIHdiff(AC) (V) | VIX(AC) (%)(47) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minimum | Typical | Maximum | Maximum | Minimum | Maximum | Minimum | Maximum |
| POD12(48) | 1.164 | 1.2 | 1.236 | –0.11 | 0.11 | –0.14 | 0.14 | 25 |
| POD11(48) | 1.067 | 1.1 | 1.133 | –0.11 | 0.11 | –0.14 | 0.14 | 25 |

(46) Each sub-bank can only support a single voltage tolerance. The VCCIO_PIO tolerance can be extended to ±5% if the entire GPIO-B sub- bank is operating in any of the following modes:

  - PHYLITE mode

  - GPIO mode

(47) Percentage of P-leg and N-leg crossing relative to the midpoint of P-leg and N-leg signal swings.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

33

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Note:	For eye height position estimation in EMIF interfaces, refer to the External Memory Interfaces Agilex™ 7 M-Series FPGA IP User Guide. The eye mask estimation methodology defined in the External Memory Interfaces Agilex™ 7 M-Series FPGA IP User Guide takes precedence over specifications in GPIO-B Differential POD I/O Standards Specifications table.

GPIO-B Differential LVSTL I/O Standards Specifications

Table 32.	GPIO-B Differential LVSTL I/O Standards Specifications

For specification status, see the Data Sheet Status table

| I/O Standard | VCCIO_PIO (V) |  |  | VILdiff(DC) (V) | VIHdiff(DC) (V) | VILdiff(AC) (V) | VIHdiff(AC) (V) | VIX(AC) (%)(49) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minimum | Typical | Maximum | Maximum | Minimum | Maximum | Minimum | Maximum |
| LVSTL11(50) | 1.067 | 1.1 | 1.133 | –0.11 | 0.11 | –0.14 | 0.14 | 25 |
| LVSTL105(50) | 1.0185 | 1.05 | 1.0815 | –0.11 | 0.11 | –0.14 | 0.14 | 25 |
| LVSTL700 | 1.0185 | 1.05 | 1.0815 | –0.11 | 0.11 | –0.14 | 0.14 | 25 |

Note:	For eye height position estimation in EMIF interfaces, refer to the External Memory Interfaces Agilex™ 7 M-Series FPGA IP User Guide. The eye mask estimation methodology defined in the External Memory Interfaces Agilex™ 7 M-Series FPGA IP User Guide takes precedence over specifications in GPIO-B Differential LVSTL I/O Standards Specifications table.

Related Information

External Memory Interfaces Agilex 7 M-Series FPGA IP User Guide Provides eye mask estimation for EMIF interfaces.

(48) Each sub-bank can only support a single voltage tolerance. The VCCIO_PIO tolerance can be extended to ±5% if the entire GPIO-B sub- bank is operating in any of the following modes:

- PHYLITE mode

- GPIO mode

(49) Percentage of P-leg and N-leg crossing relative to the midpoint of P-leg and N-leg signal swings.

(50) Each sub-bank can only support a single voltage tolerance. The VCCIO_PIO tolerance can be extended to ±5% if the entire GPIO-B sub- bank is operating in any of the following modes:

- PHYLITE mode

- GPIO mode

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

34

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

GPIO-B Differential I/O Standards Specifications

Table 33.	GPIO-B Differential I/O Standards Specifications

For specification status, see the Data Sheet Status table

| I/O Standard | VCCIO_PIO (V) |  |  | VID (mV) |  | VICM(DC) (V) |  |  | VOD (mV)(51) (52) |  |  | VOCM (V)(51) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minimum | Typical | Maximu m | Minimum | Maximu m | Minimum | Conditio n | Maximu m | Minimum | Typical | Maximu m | Minimum | Typical | Maximu m |
| True Differenti al Signaling -1.3 V (LVDS compatib le Transmitt er and | 1.261 | 1.3 | 1.339 | 100 | 500 | 0.5 | — | 1.40(56) | 247 | — | 454 | 0.9 | 1 | 1.1 |
| continued... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

(51) RL range: 90 ≤ RL ≤ 110 Ω.

(52) The specification is only applicable to default VOD and pre-emphasis setting.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

35

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| I/O Standard | VCCIO_PIO (V) |  |  | VID (mV) |  | VICM(DC) (V) |  |  | VOD (mV)(51) (52) |  |  | VOCM (V)(51) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minimum | Typical | Maximu m | Minimum | Maximu m | Minimum | Conditio n | Maximu m | Minimum | Typical | Maximu m | Minimum | Typical | Maximu m |
| Receiver) (53) (54) (55) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| True Differenti al Signaling -1.2V (Receiver only)(53) | 1.14 | 1.2 | 1.26 | 100 | 454 | 0.8 | — | 0.95 | — | — | — | — | — | — |
| True Differenti al Signaling -1.1V (Receiver only)(53) | 1.045 | 1.1 | 1.155 | 100 | 454 | 0.8 | — | 0.95 | — | — | — | — | — | — |
| continued... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

(51) RL range: 90 ≤ RL ≤ 110 Ω.

(52) The specification is only applicable to default VOD and pre-emphasis setting.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

36

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| I/O Standard | VCCIO_PIO (V) |  |  | VID (mV) |  | VICM(DC) (V) |  |  | VOD (mV)(51) (52) |  |  | VOCM (V)(51) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minimum | Typical | Maximu m | Minimum | Maximu m | Minimum | Conditio n | Maximu m | Minimum | Typical | Maximu m | Minimum | Typical | Maximu m |
| True Differenti al Signaling -1.05V (Receiver only)(53) | 0.9975 | 1.05 | 1.1025 | 100 | 454 | 0.8 | — | 0.95 | — | — | — | — | — | — |
| SLVS400 | 1.164 | 1.2 | 1.236 | 70 | — | 0.07 | 0.2 | 0.33 | — | — | — | — | — | — |
|  | 1.067 | 1.1 | 1.133 |  |  |  |  |  |  |  |  |  |  |  |

HPS I/O Standard Specifications

Tables in this section list the supported input voltage (VIH and VIL), output voltage (VOH and VOL), and current drive characteristics (IOH and IOL) for various I/O standards.

(51) RL range: 90 ≤ RL ≤ 110 Ω.

(52) The specification is only applicable to default VOD and pre-emphasis setting.

(53) The True Differential Signaling input buffer is supported on 1.05 V, 1.1 V, 1.2 V, and 1.3 V VCCIO_PIO bank. The maximum input voltage driven into the True Differential Signaling input buffer must not exceed VICM(max) + VID(max)/2.

(54) True Differential Signaling - 1.3 V standard is compatible with LVDS and capable to interface with LVDS subsets such as:

  - RSDS

  - Mini-LVDS

  - Any I/O standards using equivalent electrical specifications

(55) For further information on True Differential Signaling - 1.3 V feature support and guidelines on interfacing True Differential Signaling

-1.3V standard with LVDS and it's subset compliant standards, refer to the related information.

(56) The VICM(DC) voltage must not exceed 1.2 V when on-chip differential termination (RD OCT) is disabled with the use of external on- board termination.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

37

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

For minimum voltage values, use the minimum VCCIO_HPS values. For maximum voltage values, use the maximum VCCIO_HPS values.

You must perform timing closure analysis to determine the maximum achievable frequency for general purpose I/O standards.

HPS Single-Ended I/O Standards Specifications

Table 34.	HPS Single-Ended I/O Standards Specifications

For specification status, see the Data Sheet Status table

| I/O Standard | VCCIO_HPS |  |  | VIL (V) |  | VIH (V) |  | VOL (V) | VOH (V) | IOL (mA)(57) | IOH (mA)(57) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minimum | Typical | Maximum | Minimum | Maximum | Minimum | Maximum | Maximum | Minimum | Maximum | Minimum |
| 1.8 V LVCMOS | 1.71 | 1.8 | 1.89 | –0.3 | 0.35 × VCCIO_HPS | 0.65 × VCCIO_HPS | VCCIO_HPS + 0.3 | 0.4 | VCCIO_HPS – 0.4 | 8 | –8 |

SDM I/O Standard Specifications

Tables in this section list the supported input voltage (VIH and VIL), output voltage (VOH and VOL), and current drive characteristics (IOH and IOL) for various I/O standards.

For minimum voltage values, use the minimum VCCIO_SDM values. For maximum voltage values, use the maximum VCCIO_SDM values.

(57) To meet the IOH and IOL specifications, you must set the current strength settings accordingly. For example, to meet the 1.8 V LVCMOS specification (8 mA), you should set the current strength settings to 8 mA. Setting at lower current strength may not meet the IOH and IOL specifications in the data sheet.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

38

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

SDM Single-Ended I/O Standards Specifications

Table 35.	SDM Single-Ended I/O Standards Specifications

For specification status, see the Data Sheet Status table

| I/O Standard | VCCIO_SDM (V) |  |  | VIL (V) |  | VIH (V) |  | VOL (V) | VOH (V) | IOL (mA)(58) | IOH (mA)(58) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minimum | Typical | Maximum | Minimum | Maximum | Minimum | Maximum | Maximum | Minimum | Maximum | Minimum |
| 1.8 V LVCMOS | 1.71 | 1.8 | 1.89 | –0.3 | 0.35 × VCCIO_SDM | 0.65 × VCCIO_SDM | VCCIO_SDM + 0.3 | 0.4 | VCCIO_SDM – 0.4 | 8 | –8 |

Switching Characteristics

This section provides the performance characteristics of core and periphery blocks.

Core Performance Specifications

Clock Tree Specifications

Table 36.	Clock Tree Performance

For specification status, see the Data Sheet Status table

| Parameter | Performance |  | Unit |
| --- | --- | --- | --- |
|  | –1V, –2V | –3V, –3E |  |
| Programmable clock routing | 1,000 | 780 | MHz |

(58) To meet the IOH and IOL specifications, you must verify the current strength settings accordingly. For example, to meet the 1.8 V LVCMOS specification (8 mA), you should verify the current strength settings used is 8 mA . Using a lower current strength may not meet the IOH and IOL specifications in the data sheet.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

39

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

I/O PLL Specifications

Table 37.	I/O PLL Specifications

The specifications in this table are applicable to I/O bank I/O PLL , fabric-feeding I/O PLL, and UIB fabric-feeding PLL.

For specification status, see the Data Sheet Status table

| Symbol | Parameter | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fIN | Input clock frequency source from Core Clock Input or Reference Clock Input | –1V | 10 | — | 1,100(59) | MHz |
|  |  | –2V | 10 | — | 900(59) | MHz |
|  |  | –3V, –3E | 10 | — | 625(59) | MHz |
|  | Input clock frequency source from IO Clock Input | –1V | 10 | — | 800(59) | MHz |
|  |  | –2V | 10 | — | 717(59) | MHz |
|  |  | –3V, –3E | 10 | — | 625(59) | MHz |
| fINPFD | Input clock frequency to the PFD | — | 10 | — | 325 | MHz |
| fVCO | I/O PLL VCO operating range | –1V | 600 | — | 3,200 | MHz |
|  |  | –2V | 600 | — | 3,200 | MHz |
|  |  | –3V, –3E | 600 | — | 2,400 | MHz |
| fCLBW | I/O PLL closed-loop bandwidth | — | 0.5 | — | 20 | MHz |
| fOUT | Output frequency for internal clock (C counter) | –1V | — | — | 1,100 | MHz |
|  |  | –2V | — | — | 1,000 | MHz |
|  |  | –3V, –3E | — | — | 780 | MHz |
| fOUT_EXT | Output frequency for external clock output | –1V | — | — | 800 | MHz |
| continued... |  |  |  |  |  |  |

(59) This specification is limited by the I/O maximum frequency. The maximum achievable I/O frequency is different for each I/O standard and is dependent on design and system specific factors. Ensure proper timing closure in your design and perform HSPICE/IBIS simulations based on your specific design and system setup to determine the maximum achievable frequency in your system.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

40

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Parameter | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | –2V | — | — | 717 | MHz |
|  |  | –3V, –3E | — | — | 625 | MHz |
| tOUTDUTY | Duty cycle for dedicated external clock output (when set to 50%) | fOUT_EXT < 300 MHz | 45 | 50 | 55 | % |
|  |  | fOUT_EXT ≥ 300 MHz | 40/45 (60) | 50 | 55 (60)/60 | % |
| tFCOMP (61) | External feedback clock compensation time | — | — | — | 5 | ns |
| fDYCONFIGCLK | Dynamic configuration clock | — | — | — | 100 | MHz |
| tLOCK | Time required to lock from end-of-device configuration or deassertion of areset | — | — | — | 1 | ms |
| tDLOCK | Time required to lock dynamically (after switchover or reconfiguring any non- post-scale counters/ delays) | — | — | — | 1 | ms |
| tPLL_PSERR | Accuracy of PLL phase shift | — | — | — | ±50 | ps |
| tARESET | Minimum pulse width on the areset signal | — | 10 | — | — | ns |
| tINCCJ | Input clock cycle-to- cycle jitter | fREF < 100 MHz (62) | — | — | ±750 | ps (p-p) |
| continued... |  |  |  |  |  |  |

(60) To achieve 5% duty cycle for fOUT_EXT ≥ 300 MHz, you only can use tx_outclk port from the LVDS SERDES FPGA IP. Refer to the related information for the detail design guidelines.

(61) Not applicable for fabric-feeding PLL.

(62) fREF is fIN/N, specification applies when N = 1.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

41

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Parameter | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | fREF ≥ 100 MHz (62) | — | — | 0.15 | UI (p-p) |
| tREFPJ | Reference phase jitter (rms)(63) | Carrier frequency: 100 MHz with integrated bandwidth of 10 kHz to 50 MHz | — | — | 1.42 | ps |
| tREFPN | Reference phase noise(64) (63) | 10 Hz | — | — | –90 | dBc/Hz |
|  |  | 100 Hz | — | — | –100 | dBc/Hz |
|  |  | 1 kHz | — | — | –110 | dBc/Hz |
|  |  | 10 kHz | — | — | –120 | dBc/Hz |
|  |  | 100 kHz | — | — | –130 | dBc/Hz |
|  |  | 1 MHz | — | — | –138 | dBc/Hz |
|  |  | 10 MHz | — | — | –142 | dBc/Hz |
|  |  | 100 MHz | — | — | –144 | dBc/Hz |
| tOUTPJ_DC (61) (65) | Period jitter for dedicated clock output | fOUT < 100 MHz (62) | — | — | 17.5 | mUI (p-p) |
|  |  | fOUT ≥ 100 MHz (62) | — | — | 175 | ps (p-p) |
| tOUTCCJ_DC (61) (65) | Cycle-to-cycle jitter for dedicated clock output | fOUT < 100 MHz (62) | — | — | 17.5 | mUI (p-p) |
|  |  | fOUT ≥ 100 MHz (62) | — | — | 175 | ps (p-p) |
| continued... |  |  |  |  |  |  |

(63) Requirement for Advanced Interface Bus (AIB), High Bandwidth Memory (HBM) Interface, DDR/LPDDR protocol, and LVDS SERDES applications only.

(64) The phase noise numbers in the table above are the maximum acceptable phase noise values measured at a carrier frequency of 100 MHz. To calculate the phase noise requirement at any other frequency, use the formula: REFCLK phase noise at f (MHz) = REFCLK phase noise at 100 MHz + (20 × log10 (f/100)).

(65) This jitter specification does not include the effect of spread-spectrum clock. The magnitude of jitter deterioration is largely depend on the spread-spectrum clock profile used. Refer to the related information for the recommended spread-spectrum clock profile.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

42

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Parameter | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| tOUTPJ_IO (66) (65) | Period jitter for clock output on the regular I/O | fOUT < 100 MHz (62) | — | — | 60 | mUI (p-p) |
|  |  | fOUT ≥ 100 MHz (62) | — | — | 600 | ps (p-p) |
| tOUTCCJ_IO (66) (65) | Cycle-to-cycle jitter for clock output on the regular I/O | fOUT < 100 MHz (62) | — | — | 60 | mUI (p-p) |
|  |  | fOUT ≥ 100 MHz (62) | — | — | 600 | ps (p-p) |
| tCASC_OUTPJ_DC (61) | Period jitter for dedicated clock output in cascaded PLLs | fOUT < 100 MHz (62) | — | — | 17.5 | mUI (p-p) |
|  |  | fOUT ≥ 100 MHz (62) | — | — | 175 | ps (p-p) |
| tEINDUTY | Input clock or external feedback clock input duty cycle | fIN ≥ 600 MHz | 30 | — | 70 | % |
|  |  | 450 MHz ≤ fIN < 600 MHz | 35 | — | 65 | % |
|  |  | 250 MHz ≤ fIN < 450 MHz | 40 | — | 60 | % |
|  |  | 10 MHz ≤ fIN < 250 MHz | 45 | — | 55 | % |

Related Information

    - Memory Output Clock Jitter Specifications on page 57

Provides more information about the external memory interface clock output jitter specifications.

    - Agilex 7 Clocking and PLL User Guide: M-Series

Provides the recommended spread-spectrum clock profile and design guidelines to achieve 5% duty cycle using the LVDS SERDES FPGA IP.

(66) External memory interface clock output jitter specifications use a different measurement method, which are available in the Memory Output Clock Jitter Specifications table.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

43

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

DSP Block Specifications

Table 38.	DSP Block Performance Specifications

For specification status, see the Data Sheet Status table

| Mode | Performance |  |  | Unit |
| --- | --- | --- | --- | --- |
|  | –1V | –2V | –3V, –3E |  |
| Fixed-point 18 × 19 multiplication mode | 900 | 771 | 676 | MHz |
| Fixed-point 27 × 27 multiplication mode | 900 | 771 | 676 | MHz |
| Fixed-point 18 × 19 multiplier adder mode(67) | 900 | 771 | 676 | MHz |
| Fixed-point 18 × 19 multiplier adder summed with 36-bit input mode | 900 | 771 | 676 | MHz |
| Fixed-point four 9 × 9 multiplier adder mode | 900 | 771 | 676 | MHz |
| Fixed-point 18 × 19 systolic mode | 900 | 771 | 676 | MHz |
| Fixed-point 18 × 19 complex multiplication mode | 900 | 771 | 676 | MHz |
| FP32 floating-point multiplication mode | 750 | 579 | 507 | MHz |
| FP32 floating-point adder or subtract mode | 750 | 579 | 507 | MHz |
| continued... |  |  |  |  |

(67) When Chainout is enabled to send data to the next DSP but systolic registers are not used, the performance specifications for the following speed grades are as follows:

- –1V: 675 MHz

- –2V: 578 MHz

- –3V and –3E: 507 MHz

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

44

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Mode | Performance |  |  | Unit |
| --- | --- | --- | --- | --- |
|  | –1V | –2V | –3V, –3E |  |
| FP32 floating-point multiplier adder or subtract mode | 750 | 579 | 507 | MHz |
| FP32 floating-point multiplier accumulate mode | 750 | 579 | 507 | MHz |
| Addition or subtraction of two FP16 floating-point multiplication mode | 750 | 579 | 507 | MHz |
| Sum/sub of two FP16 multiplications with FP32 (addition/subtraction) | 750 | 579 | 507 | MHz |
| Sum/sub of two FP16 multiplications with accumulation (addition/ subtraction) | 750 | 579 | 507 | MHz |
| FP32 floating-point complex multiplication | 750 | 579 | 507 | MHz |
| FP32 floating-point vector dot product | 750 | 579 | 507 | MHz |
| FP16 floating-point complex multiplication | 750 | 579 | 507 | MHz |
| FP16 floating-point vector dot product | 750 | 579 | 507 | MHz |

Memory Block Specifications

To achieve the maximum memory block performance, use a memory block clock that comes through global clock routing from an on-chip PLL and set to 50% output duty cycle. Use the Quartus Prime software to report timing for the memory block clocking schemes.

When you use the error detection cyclical redundancy check (CRC) feature, there is no degradation in fMAX.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

45

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Table 39.	Memory Block Performance Specifications

For specification status, see the Data Sheet Status table

| Memory | Mode | Performance |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | –1V | –2V | –3V, –3E |  |
| MLAB | Single-port RAM/ROM Simple dual-port RAM | 1,000 | 782 | 667 | MHz |
|  | Simple dual-port RAM with read-during-write option set to New Data or Old Data | 630 | 510 | 460 | MHz |
| M20K Block(68) | Single-port RAM/ROM | 1,000 | 782 | 667 | MHz |
|  | Simple dual-port RAM |  |  |  |  |
|  | Simple dual-port RAM, | 1,000 | 782 | 667 | MHz |
|  | coherent read enabled |  |  |  |  |
|  | Single-port RAM with the | 800 | 640 | 560 | MHz |
|  | read-during-write option |  |  |  |  |
|  | set to Old Data |  |  |  |  |
|  | Simple dual-port RAM with |  |  |  |  |
|  | the read-during-write |  |  |  |  |
|  | option set to Old Data |  |  |  |  |
|  | Simple dual-port RAM with | 600 | 480 | 420 | MHz |
|  | ECC enabled, 512 × 32 |  |  |  |  |
|  | Simple dual-port RAM with | 1,000 | 782 | 667 | MHz |
|  | ECC, optional pipeline |  |  |  |  |
|  | registers enabled, 512 × |  |  |  |  |
|  | 32 |  |  |  |  |
|  | Dual-port ROM | 600 | 500 | 420 | MHz |
|  | True dual-port RAM |  |  |  |  |
|  | Simple quad-port RAM | 600 | 500 | 420 | MHz |
|  | Fabric NoC write/read | 700 | 700 | 500 | MHz |

(68) For the M20K block, Quartus automatically optimizes timing and power based on design requirements.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

46

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Related Information

Agilex 7 Embedded Memory User Guide

Provides details on timing/power optimization feature.

Local Temperature Sensor Specifications

Table 40.	Local Temperature Sensor Specifications

For specification status, see the Data Sheet Status table

| Description | Temperature Range (69) | Accuracy | Sampling Rate(70) | Conversion Time |
| --- | --- | --- | --- | --- |
| Local Temperature Sensor | –40 to 125°C(71) | ±5°C | 1 KSPS | < 1 ms |

Related Information

Recommended Operating Conditions on page 13

Remote Temperature Diode Specifications

Note the following for the remote temperature diode specifications:

  - The temperature diode characteristics in this table target for three-currents temperature sensing chip implementation. The characteristics can also apply to two-currents temperature sensing chip implementation.

  - Absolute accuracy is dependent on third-party external diode ADC and integration specifics.

(69) The supported operating temperature varies across the device variants. Refer to the Recommended Operating Conditions table for more information about the operating temperature supported.

(70) The read out is subject to the SDM mailbox activity status.

(71) Temperature range refers to junction temperature.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

47

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Table 41.	Remote Temperature Diode Specifications (Core Fabric TSD)

For specification status, see the Data Sheet Status table

| Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- |
| Ibias, diode source current | 30 | — | 170 | μA |
| Vbias, voltage across diode | 0.48 | — | 0.82 | V |
| Series resistance | — | — | <10 | Ω |
| Diode ideality factor | — | 1.006(72) | — | — |

Table 42.	Remote Temperature Diode Specifications (R-Tile TSD)

For specification status, see the Data Sheet Status table

| Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- |
| Ibias, diode source current | 30 | — | 170 | μA |
| Vbias, voltage across diode | 0.48 | — | 0.82 | V |
| Series resistance | — | — | <10 | Ω |
| Diode ideality factor | — | 1.001(73) | — | — |

Table 43.	Remote Temperature Diode Specifications (F-Tile TSD)

For specification status, see the Data Sheet Status table

| Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- |
| Ibias, diode source current | 30 | — | 170 | μA |
| Vbias, voltage across diode | 0.48 | — | 0.83 | V |
| Series resistance | — | — | <10 | Ω |
| Diode ideality factor | — | 1.006(74) | — | — |

(72)  When using lower injection current (two-currents) implementation, the ideality factor is 1.022.

(73)  When using lower injection current (two-currents) implementation, the ideality factor is 1.010.

(74)  When using lower injection current (two-currents) implementation, the ideality factor is 1.018.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

48

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Voltage Sensor Specifications

Table 44.	Voltage Sensor Specifications

For specification status, see the Data Sheet Status table

| Parameter |  | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Resolution |  | — | 7 | — | Bit |
| Sampling rate(75) |  | — | — | 1 | KSPS |
| Input capacitance |  | — | — | 40 | pF |
| Voltage sensor accuracy, Vin range: 0 V to 1.1 V(76) (77) |  | — | — | ±3.5 | % |
| Unipolar Input Mode | Input signal range for Vsigp | — | — | 1.35 | V |
|  | Common mode voltage on Vsign | — | — | 0.25 | V |
|  | Input signal range for Vsigp – Vsign | — | — | 1.1 | V |

Periphery Performance Specifications

This section describes the periphery performance, LVDS SERDES, and external memory interface.

Actual achievable frequency depends on design and system specific factors. Ensure proper timing closure in your design and perform HSPICE/IBIS simulations based on your specific design and system setup to determine the maximum achievable frequency in your system.

(75) The read out is subject to the SDM mailbox activity status.

(76) For low voltage channels in channels 0, 1, 2, 6, and 7, the ±3.5% accuracy equals to ±43.75mV. For high voltage channels in channels 3, 4, 5, and 9, the accuracy is ±4.5%. This equals to ±56.25mV.

(77) When Voltage Tamper Detection is enabled, the voltage sensor accuracy specifications are as follows:

- For low voltage channels in channels 0, 1, 2, 6, and 7, the accuracy is ±5.5%. This equals to ±68.75mV.

- For high voltage channels in channels 3, 4, 5, and 9, the accuracy is ±6.5%. This equals to ±81.25mV.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

49

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

LVDS SERDES Specifications

Table 45.	LVDS SERDES Specifications

LVDS serializer/deserializer (SERDES) block supports SERDES factor J = 4 and 8. DDR registers support SERDES factor J = 1 to 2.

You must calculate the leftover timing margin in the receiver by performing link timing closure analysis. You must consider the board skew margin, transmitter channel-to-channel skew, and receiver sampling margin to determine the leftover timing margin.

For specification status, see the Data Sheet Status table

| Parameter | Symbol | Condition | –1 Speed Grade |  |  | –2 Speed Grade |  |  | –3 Speed Grade |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Minimum | Typical | Maximum | Minimum | Typical | Maximum | Minimum | Typical | Maximum |  |
| Clock frequency | fHSCLK_in (input | Clock boost | 10 | — | 800 | 10 | — | 800 | 10 | — | 625 | MHz |
|  | clock frequency) | factor W = 1 to 40(78) |  |  |  |  |  |  |  |  |  |  |
|  | True |  |  |  |  |  |  |  |  |  |  |  |
|  | Differential |  |  |  |  |  |  |  |  |  |  |  |
|  | Signaling |  |  |  |  |  |  |  |  |  |  |  |
|  | I/O |  |  |  |  |  |  |  |  |  |  |  |
|  | Standards |  |  |  |  |  |  |  |  |  |  |  |
|  | fHSCLK_in | Clock boost factor W = 1 to 40(78) | 10 | — | 445.5 | 10 | — | 445.5 | 10 | — | 445.5 | MHz |
|  | (input |  |  |  |  |  |  |  |  |  |  |  |
|  | clock |  |  |  |  |  |  |  |  |  |  |  |
|  | frequency) |  |  |  |  |  |  |  |  |  |  |  |
|  | SLVS400 |  |  |  |  |  |  |  |  |  |  |  |
|  | I/O |  |  |  |  |  |  |  |  |  |  |  |
|  | Standards |  |  |  |  |  |  |  |  |  |  |  |
|  | fHSCLK_in | Clock boost factor W = 1 to 40(78) | 10 | — | 625 | 10 | — | 625 | 10 | — | 525 | MHz |
|  | (input |  |  |  |  |  |  |  |  |  |  |  |
|  | clock |  |  |  |  |  |  |  |  |  |  |  |
|  | frequency) |  |  |  |  |  |  |  |  |  |  |  |
|  | Single- |  |  |  |  |  |  |  |  |  |  |  |
|  | Ended I/O |  |  |  |  |  |  |  |  |  |  |  |
|  | Standards |  |  |  |  |  |  |  |  |  |  |  |
| continued... |  |  |  |  |  |  |  |  |  |  |  |  |

(78) Clock Boost Factor (W) is the ratio between the input data rate and the input clock rate.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

50

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Parameter | Symbol | Condition | –1 Speed Grade |  |  | –2 Speed Grade |  |  | –3 Speed Grade |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Minimum | Typical | Maximum | Minimum | Typical | Maximum | Minimum | Typical | Maximum |  |
|  | fHSCLK_OUT (output clock frequency) | — | — | — | 800 | — | — | 800 | — | — | 625 | MHz |
| Transmitte | True | SERDES factor J = 4 and 8(80) (81) | 600 | — | 1,600 | 600 | — | 1,600 | 600 | — | 1,250 | Mbps |
| r | Differential |  |  |  |  |  |  |  |  |  |  |  |
|  | Signaling |  |  |  |  |  |  |  |  |  |  |  |
|  | I/O |  |  |  |  |  |  |  |  |  |  |  |
|  | Standards - fHSDR (data rate)(79) |  |  |  |  |  |  |  |  |  |  |  |
|  |  | SERDES factor J = 2, uses DDR | (82) | — | 500(83) | (82) | — | 500(83) | (82) | — | 500(83) | Mbps |
|  |  | registers |  |  |  |  |  |  |  |  |  |  |
|  |  | SERDES | (82) | — | 250(83) | (82) | — | 250(83) | (82) | — | 250(83) | Mbps |
|  |  | factor J = |  |  |  |  |  |  |  |  |  |  |
|  |  | 1, uses |  |  |  |  |  |  |  |  |  |  |
|  |  | DDR |  |  |  |  |  |  |  |  |  |  |
|  |  | registers |  |  |  |  |  |  |  |  |  |  |
| continued... |  |  |  |  |  |  |  |  |  |  |  |  |

(79) Requires package skew compensation with PCB trace length.

(80) The Fmax specification is based on the fast clock used for serial data. The interface Fmax is also dependent on the parallel clock domain which is design dependent and requires timing analysis.

(81) The VCC and VCCP must be on a combined power layer and a maximum load of 5 pF for chip-to-chip interface.

(82) The minimum specification depends on the following factors. The differential I/O buffer within the IOE does not have a minimum data rate.

- The clock source, such as the PLL and clock pin

- The clock and data routing resource

(83) You must perform design timing analysis in Quartus Prime to achieve timing closure and run IBIS/HSPICE simulations to ensure that the I/O buffer's electrical performance meets the interface requirements.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

51

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Parameter | Symbol | Condition | –1 Speed Grade |  |  | –2 Speed Grade |  |  | –3 Speed Grade |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Minimum | Typical | Maximum | Minimum | Typical | Maximum | Minimum | Typical | Maximum |  |
|  | tx Jitter - True | Total jitter for data | ≤1,600 Mbps: 150 ≤1,434 Mbps: 160 ≤1,250 Mbps: 170 ≤1,000 Mbps: 190 ≤800 Mbps: 220 600 Mbps: 260 |  |  | ≤1,600 Mbps: 150 ≤1,434 Mbps: 160 ≤1,250 Mbps: 170 ≤1,000 Mbps: 190 ≤800 Mbps: 220 600 Mbps: 260 |  |  | ≤1,250 Mbps: 170 ≤1,000 Mbps: 190 ≤800 Mbps: 220 600 Mbps: 260 |  |  | ps |
|  | Differential | rate, 600 |  |  |  |  |  |  |  |  |  |  |
|  | Signaling | Mbps – 1.6 |  |  |  |  |  |  |  |  |  |  |
|  | I/O | Gbps |  |  |  |  |  |  |  |  |  |  |
|  | Standards |  |  |  |  |  |  |  |  |  |  |  |
|  | tDUTY (84) | TX output clock duty | 45 | 50 | 55 | 45 | 50 | 55 | 45 | 50 | 55 | % |
|  |  | cycle for |  |  |  |  |  |  |  |  |  |  |
|  |  | Differential |  |  |  |  |  |  |  |  |  |  |
|  |  | I/O |  |  |  |  |  |  |  |  |  |  |
|  |  | Standards |  |  |  |  |  |  |  |  |  |  |
|  | tRISE & tFALL (82) (85) | True Differential I/O | — | — | 160 | — | — | 180 | — | — | 200 | ps |
|  |  | Standards |  |  |  |  |  |  |  |  |  |  |
|  | TCCS (79) (84) | True Differential | — | — | 202 | — | — | 202 | — | — | 202 | ps |
|  |  | I/O |  |  |  |  |  |  |  |  |  |  |
|  |  | Standards |  |  |  |  |  |  |  |  |  |  |
| Receiver(86 ) | True Differential Signaling I/O Standards - fHSDRDPA (data rate) | SERDES factor J = 4 and 8(80) (81) | 600 | — | 1600(87) | 600 | — | 1600(87) | 600 | — | 1250(87) | Mbps |
| continued... |  |  |  |  |  |  |  |  |  |  |  |  |

(84) Not applicable for DIVCLK = 1.

(85) This applies to default pre-emphasis and VOD settings only.

(86) You must enable the receiver equalization feature of the input buffer when operating in DPA mode.

(87) 1.05 V, 1.1 V, and 1.2 V True Differential Signaling I/O standards on receiver supports data rate up to 1000 Mbps.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

52

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Parameter | Symbol | Condition | –1 Speed Grade |  |  | –2 Speed Grade |  |  | –3 Speed Grade |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Minimum | Typical | Maximum | Minimum | Typical | Maximum | Minimum | Typical | Maximum |  |
|  | SLVS400 | SERDES factor J = 4 and 8(80) (81) | 600 | — | 891 | 600 | — | 891 | 600 | — | 891 | Mbps |
|  | I/O |  |  |  |  |  |  |  |  |  |  |  |
|  | Standards |  |  |  |  |  |  |  |  |  |  |  |
|  | - fHSDRDPA |  |  |  |  |  |  |  |  |  |  |  |
|  | (data rate) |  |  |  |  |  |  |  |  |  |  |  |
|  | fHSDR (data rate) (without | SERDES factor J = 4 and 8(80) | 150 | — | (88) | 150 | — | (88) | 150 | — | (88) | Mbps |
|  | DPA)(79) | (81) |  |  |  |  |  |  |  |  |  |  |
|  |  | SERDES | (82) | — | 500(83) | (82) | — | 500(83) | (82) | — | 500(83) | Mbps |
|  |  | factor J = |  |  |  |  |  |  |  |  |  |  |
|  |  | 2, uses |  |  |  |  |  |  |  |  |  |  |
|  |  | DDR |  |  |  |  |  |  |  |  |  |  |
|  |  | registers |  |  |  |  |  |  |  |  |  |  |
|  |  | SERDES | (82) | — | 250(83) | (82) | — | 250(83) | (82) | — | 250(83) | Mbps |
|  |  | factor J = |  |  |  |  |  |  |  |  |  |  |
|  |  | 1, uses |  |  |  |  |  |  |  |  |  |  |
|  |  | DDR |  |  |  |  |  |  |  |  |  |  |
|  |  | registers |  |  |  |  |  |  |  |  |  |  |
| DPA (FIFO mode) | DPA run length | — | — | — | ≤10,000 | — | — | ≤10,000 | — | — | ≤10,000 | UI |
| DPA (soft CDR mode) | DPA run length | SGMII/GbE protocol | — | — | 5 | — | — | 5 | — | — | 5 | UI |
|  |  | All other protocols | — | — | 50 data transition per 208 UI | — | — | 50 data transition per 208 UI | — | — | 50 data transition per 208 UI | — |
| Soft CDR mode | Soft-CDR ppm tolerance | — | –300 | — | 300 | –300 | — | 300 | –300 | — | 300 | ppm |
| Non DPA mode | Sampling window | — | — | — | 330 | — | — | 330 | — | — | 330 | ps |

(88) You can estimate the achievable maximum data rate for non-DPA mode by performing link timing closure analysis. You must consider the board skew margin, transmitter delay margin, and receiver sampling margin to determine the maximum data rate supported.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

53

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

DPA Lock Time Specifications

Table 46.	DPA Lock Time Specifications

The DPA lock time is for one channel. One data transition is defined as a 0-to-1 or 1-to-0 transition.

For specification status, see the Data Sheet Status table

| Standard | Training Pattern | Number of Data Transitions in One Repetition of the Training Pattern | Number of Repetitions per 256 Data Transitions(89) | Maximum Data Transition |
| --- | --- | --- | --- | --- |
| SPI-4 | 00000000001111111111 | 2 | 128 | 768 |
| Parallel Rapid I/O | 00001111 | 2 | 128 | 768 |
|  | 10010000 | 4 | 64 | 768 |
| Miscellaneous | 10101010 | 8 | 32 | 768 |
|  | 01010101 | 8 | 32 | 768 |

(89) This is the number of repetitions for the stated training pattern to achieve the 256 data transitions.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

54

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

LVDS SERDES Soft-CDR Sinusoidal Jitter Tolerance Specifications

Figure 2.	LVDS SERDES Soft-CDR Sinusoidal Jitter Tolerance Specifications for a Data Rate Equal to 1.6 Gbps

25

Jitter Amplitude(UI)

8.5

0.22

0.1

F1	F2	F3	F4

Jitter Frequency (Hz)

Table 47.	LVDS SERDES Soft-CDR Sinusoidal Jitter Mask Values for a Data Rate Equal to 1.6 Gbps

For specification status, see the Data Sheet Status table

| Parameter | Jitter Frequency (Hz) | Sinusoidal Jitter (UI) |
| --- | --- | --- |
| F1 | 10,000 | 25 |
| F2 | 17,565 | 25 |
| F3 | 1,493,000 | 0.22 |
| F4 | 50,000,000 | 0.22 |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

55

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Figure 3.	LVDS SERDES Soft-CDR Sinusoidal Jitter Tolerance Specifications for a Data Rate Less than 1.6 Gbps

20db/dec

Sinusoidal Jitter Amplitude

0.1 UI P-P

baud/1667	20 MHz

Frequency

Memory Standards Supported

Table 48.	Memory Standards Supported

| Memory Standard | Controller Type | Maximum Frequency (MHz) |
| --- | --- | --- |
| DDR4 SDRAM | Hard memory controller | 1,600 |
| DDR5 SDRAM | Hard memory controller | 2,800 |
| LPDDR5 SDRAM | Hard memory controller | 2,750 |
| continued... |  |  |

This table lists the overall capability of the supported External Memory Interface. For specific details, refer to the External Memory Interface Spec Estimator. For specification status, see the Data Sheet Status table

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

56

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Memory Standard | Controller Type | Maximum Frequency (MHz) |
| --- | --- | --- |
| DDR4 SDRAM | HPS hard memory controller | 1,600 |
| DDR5 SDRAM | HPS hard memory controller | 2,800 |
| LPDDR5 SDRAM | HPS hard memory controller | 2,750 |

Related Information

External Memory Interface Spec Estimator

Provides the specific details of the memory standards supported.

Memory Output Clock Jitter Specifications

The clock jitter specification applies to the memory output clock pins clocked by an I/O PLL, or generated using double data I/O circuits clocked by a PLL output routed on a PHY clock network as specified. Altera recommends using PHY clock networks for better jitter performance.

The memory clock output jitter is within the JEDEC* specifications when the phase jitter (integration bandwidth 10 kHz to 50 MHz) of the input clock is not more than 20 ps peak-to-peak, or 1.42 ps RMS at 1e-12 BER and 1.22 ps at 1e-16 BER.

Performance Specifications of the HBM2E Interface

Table 49.	Performance Specifications of the HBM2E Interface

For specification status, see the Data Sheet Status table

| Device Speed Grade | Maximum Frequency (MHz) |
| --- | --- |
| –1 | 1,600 |
| –2 | 1,400 |
| –3 | 1,000 |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

57

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Performance Specifications of Network on Chip (NoC) Table 50.	Performance Specifications of NoC

For specification status, see the Data Sheet Status table

| Device Speed Grade | Frequency (MHz) |
| --- | --- |
| –1 | 1,400 |
| –2 | 1,400 |
| –3 | 1,000 |

Note:	This table content only applies to Horizontal NoC.

NOC Reference Clock Requirements

Table 51.	NOC PLL Input Requirements

The specifications in the below table are applicable to the NoC PLL. Supply the reference clock to this NoC PLL which generates the required clock for the Hard Memory NoC.

Connect 1.8 V single-ended clock to the NoC PLL input pins. The valid input reference clock frequencies are 25 MHz, 100 MHz or 125 MHz only.

The NOC PLL receives its clock signals from the NOC PLL pins. Once you set the PLL frequency during configuration mode, NOC PLL only runs with one specified frequency during user mode. Refer to the related information for information about assigning this pin.

For specification status, see the Data Sheet Status table

| Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- |
| Clock input range | 25/100/125 |  |  | MHz |
| Clock input accuracy | — | — | 50 | ppm |
| Clock input duty cycle | 45 | 50 | 55 | % |
| Clock input peak-to-peak period jitter tolerance | — | — | 2 | % |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

58

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

R-Tile Transceiver Performance Specifications

R-Tile Transceiver Performance

Table 52.	R-Tile Transmitter and Receiver Data Rate Performance

For specification status, see the Data Sheet Status table

| Symbol/Description | Condition | Transceiver Speed Grade |  | Unit |
| --- | --- | --- | --- | --- |
|  |  | –1 | –2 |  |
| Supported data rate | PCIe* | 2.5, 5, 8, 16, 32 | 2.5, 5, 8, 16, 32 | Gbps |
|  | CXL | 8, 16, 32 | — | Gbps |

Table 53.	R-Tile Slow PLL Performance

For specification status, see the Data Sheet Status table

| Symbol/Description | Condition | All Transceiver Speed Grades |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Typical | Maximum |  |
| VCO frequency | PCIe | — | 10 | — | GHz |
|  | CXL | — | — | — | GHz |
| PLL bandwidth (BWTX- PKG_PLL1)(90) | PCIe 2.5 GT/s | 1.5 | — | 22 | MHz |
|  | PCIe 5.0 GT/s | 8 | — | 16 | MHz |
| PLL bandwidth BWTX- PKG_PLL2)(90) | PCIe 2.5 GT/s | — | — | — | MHz |
|  | PCIe 5.0 GT/s | 5 | — | 16 | MHz |
| PLL peaking (PKGTX- PLL1)(90) | PCIe 2.5 GT/s | — | — | 3 | dB |
|  | PCIe 5.0 GT/s | — | — | 3 | dB |
| PLL peaking (PKGTX- PLL2)(90) | PCIe 2.5 GT/s | — | — | — | dB |
|  | PCIe 5.0 GT/s | 1 | — | — | dB |

(90) The Tx PLL bandwidth must lie between the minimum and maximum ranges given in this table. PLL peaking must lie below the value in this table. Note that the PLL bandwidth extends from zero up to the values specified in this table. The PLL bandwidth is defined at the point where its transfer function crosses the –3 dB point.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

59

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Table 54.	R-Tile Fast PLL Performance

For specification status, see the Data Sheet Status table

| Symbol/Description | Condition | All Transceiver Speed Grades |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Typical | Maximum |  |
| VCO frequency | PCIe | — | 16 | — | GHz |
|  | CXL | — | 16 | — | GHz |
| PLL bandwidth (BWTX- PKG_PLL1)(91) | PCIe 8.0 GT/s | 0.5 | — | 4 | MHz |
|  | PCIe 16.0 GT/s | 0.5 | — | 4 | MHz |
|  | PCIe 32.0 GT/s | 0.5 | — | 1.8 | MHz |
|  | CXL 8.0 GT/s | 0.5 | — | 4 | MHz |
|  | CXL 16.0 GT/s | 0.5 | — | 4 | MHz |
|  | CXL 32.0 GT/s | 0.5 | — | 1.8 | MHz |
| PLL bandwidth (BWTX- PKG_PLL2)(91) | PCIe 8.0 GT/s | 0.5 | — | 5 | MHz |
|  | PCIe 16.0 GT/s | 0.5 | — | 5 | MHz |
|  | PCIe 32.0 GT/s | — | — | — | — |
|  | CXL 8.0 GT/s | 0.5 | — | 5 | MHz |
|  | CXL 16.0 GT/s | 0.5 | — | 5 | MHz |
|  | CXL 32.0 GT/s | — | — | — | — |
| PLL peaking (PKGTX- PLL1)(91) | PCIe 8.0 GT/s | — | — | 2 | dB |
|  | PCIe 16.0 GT/s | — | — | 2 | dB |
|  | PCIe 32.0 GT/s | — | — | 2 | dB |
|  | CXL 8.0 GT/s | — | — | 2 | dB |
| continued... |  |  |  |  |  |

(91) The Tx PLL bandwidth must lie between the minimum and maximum ranges given in this table. PLL peaking must lie below the value in this table. Note that the PLL bandwidth extends from zero up to the values specified in this table. The PLL bandwidth is defined at the point where its transfer function crosses the –3 dB point.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

60

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol/Description | Condition | All Transceiver Speed Grades |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Typical | Maximum |  |
|  | CXL 16.0 GT/s | — | — | 2 | dB |
|  | CXL 32.0 GT/s | — | — | 2 | dB |
| PLL peaking (PKGTX- PLL2)(91) | PCIe 8.0 GT/s | — | — | 1 | dB |
|  | PCIe 16.0 GT/s | — | — | 1 | dB |
|  | PCIe 32.0 GT/s | — | — | — | — |
|  | CXL 8.0 GT/s | — | — | 1 | dB |
|  | CXL 16.0 GT/s | — | — | 1 | dB |
|  | CXL 32.0 GT/s | — | — | — | — |

R-Tile Transceiver Reference Clock Specifications

Table 55.	R-Tile Reference Clock Specifications

For specification status, see the Data Sheet Status table

| Symbol/Description | Condition | All Transceiver Speed Grades |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Typical | Maximum |  |
| Supported I/O standards | PCIe | HCSL |  |  | — |
|  | CXL | HCSL |  |  | — |
| Refclk frequency for devices that support 32.0 GT/s(92) | PCIe | 99.99 | 100 | 100.01 | MHz |
|  | CXL | 99.99 | 100 | 100.01 | MHz |
| Rising edge rate(93) | PCIe | 0.6 | — | 4 | V/ns |
| continued... |  |  |  |  |  |

(92) This number is with spread-spectrum clocking (SSC) turned off. For systems with spread spectrum clocking, follow the specifications in Section 8.6 Refclk Specifications of PCI Express* Base Specification Revision 5.0 Version 1.0.

(93) Measured from –150 mV to +150 mV on the differential waveform. The 300 mV measurement window is centered on the differential zero crossing.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

61

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol/Description | Condition | All Transceiver Speed Grades |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Typical | Maximum |  |
|  | CXL | 0.6 | — | 4 | V/ns |
| Falling edge rate(93) | PCIe | 0.6 | — | 4 | V/ns |
|  | CXL | 0.6 | — | 4 | V/ns |
| Duty cycle | PCIe | 40 | — | 60 | % |
|  | CXL | 40 | — | 60 | % |
| Absolute VMAX | PCIe | — | — | 1.15 | V |
|  | CXL | — | — | 1.15 | V |
| Absolute VMIN | PCIe | — | — | –0.3 | V |
|  | CXL | — | — | –0.3 | V |
| Differential Input High Voltage (VIH) (94) | PCIe | 150 | — | — | mV |
| Differential Input High Voltage (VIH) (94) | CXL | 150 | — | — | mV |
| Differential Input Low Voltage (VIL) (94) | PCIe | — | — | -150 | mV |
| Differential Input Low Voltage (VIL) (94) | CXL | — | — | -150 | mV |
| Peak-to-peak differential input voltage | PCIe | 300 | — | 1,450 | mV |
|  | CXL | 300 | — | 1,450 | mV |
| Vcross | PCIe | 250 | — | 550 | mV |
|  | CXL | 250 | — | 550 | mV |
| Cycle-to-cycle jitter (TCCJITTER)(95) | PCIe | — | — | 150 | ps |
|  | CXL | — | — | 150 | ps |
| continued... |  |  |  |  |  |

(94) Measurement taken from differential waveform.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

62

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol/Description | Condition | All Transceiver Speed Grades |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Typical | Maximum |  |
| Spread-spectrum modulating clock frequency | PCIe | 30 | — | 33 | kHz |
|  | CXL | 30 | — | 33 | kHz |
| SSC deviation for devices that support 32.0 GT/s and SRIS when operating in SRIS mode at all speeds | PCIe | –0.3 | — | 0 | % |
|  | CXL | –0.3 | — | 0 | % |
| TSSC-MAX-PERIOD-SLEW | Max SSC df/dt for PCIe | — | — | 1,250 | ppm/µs |
|  | Max SSC df/dt for CXL | — | — | 1,250 | ppm/µs |

Related Information

PCI Express Base Specification Revision 5.0

R-Tile Transmitter Specifications

Table 56.	R-Tile Transmitter Specifications

For specification status, see the Data Sheet Status table

| Symbol/Description | Condition | All Transceiver Speed Grades |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Typical | Maximum |  |
| Supported I/O standards | PCIe | High-Speed Differential I/O |  |  | — |
|  | CXL | High-Speed Differential I/O |  |  | — |
| Differential on-chip termination resistors | PCIe (96) | 80 | 100 | 120 | Ω |
|  | CXL(96) | 80 | 100 | 120 | Ω |
| continued... |  |  |  |  |  |

(95) For common reference clock architecture, you must meet the jitter limit specified in Section 8.6 Refclk Specifications of PCI Express* Base Specification Revision 5.0 Version 1.0.

(96) 100ohms (Typical) aligned to Base spec.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

63

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol/Description | Condition | All Transceiver Speed Grades |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Typical | Maximum |  |
| Differential peak-to-peak voltage for full swing | PCIe 2.5 GT/s | 800 | — | 1,200 | mV |
|  | PCIe 5.0 GT/s | 800 | — | 1,200 | mV |
|  | PCIe 8.0 GT/s | 800 | — | 1,300 | mV |
|  | PCIe 16.0 GT/s | 800 | — | 1,300 | mV |
|  | PCIe 32.0 GT/s | 800 | — | 1,300 | mV |
|  | CXL 8.0 GT/s | 800 | — | 1,300 | mV |
|  | CXL 16.0 GT/s | 800 | — | 1,300 | mV |
|  | CXL 32.0 GT/s | 800 | — | 1,300 | mV |
| Differential peak-to-peak voltage during EIEOS | PCIe 8.0 GT/s, 16.0 GT/s, and 32.0 GT/s | 250 | — | — | mV |
|  | CXL 8.0 GT/s, 16.0 GT/s, and 32.0 GT/s | 250 | — | — | mV |
| Lane-to-lane output skew | PCIe 2.5 GT/s | — | — | 2.5 | ns |
|  | PCIe 5.0 GT/s | — | — | 2 | ns |
|  | PCIe 8.0 GT/s | — | — | 1.5 | ns |
|  | PCIe 16.0 GT/s | — | — | 1.25 | ns |
|  | PCIe 32.0 GT/s | — | — | 1.25 | ns |
|  | CXL 8.0 GT/s | — | — | 1.5 | ns |
|  | CXL 16.0 GT/s | — | — | 1.25 | ns |
|  | CXL 32.0 GT/s | — | — | 1.25 | ns |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

64

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

R-Tile Receiver Specifications

Table 57.	R-Tile Receiver Specifications

For specification status, see the Data Sheet Status table

| Symbol/Description | Condition | All Transceiver Speed Grades |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Typical | Maximum |  |
| Supported I/O standards | PCIe | High-Speed Differential I/O |  |  | — |
|  | CXL | High-Speed Differential I/O |  |  | — |
| Peak-to-peak differential input voltage VID (diff p-p) | PCIe 2.5 GT/s(97) | 175 | — | 1,200 | mVPP |
|  | PCIe 5.0 GT/s(97) | 100 | — | 1,200 | mVPP |
|  | PCIe 8.0 GT/s(97) | 25 | — | 1,200 | mVPP |
|  | PCIe 16.0 GT/s(97) | 15 | — | 800 | mVPP |
|  | PCIe 32.0 GT/s(97) | 15 | — | 800 | mVPP |
|  | CXL 8.0 GT/s(97) | 25 | — | 1,200 | mVPP |
|  | CXL 16.0 GT/s(97) | 15 | — | 800 | mVPP |
|  | CXL 32.0 GT/s(97) | 15 | — | 800 | mVPP |
| Differential on-chip termination resistors | PCIe (98) | 80 | 85 | 120 | Ω |
|  | CXL(98) | 80 | 85 | 120 | Ω |
| RCOMP | PCIe (99) | 148.5 | 150 | 151.5 | Ω |
|  | CXL(99) | 148.5 | 150 | 151.5 | Ω |

(97) For PCIe at 2.5 GT/s and 5 GT/s, VID is measured at TP2, which is the accessible test point at the device under test. For PCIe and CXL

8.0 GT/s, 16.0 GT/s and 32.0 GT/s, VID is measured at TP2P. TP2P defines a reference point that comprehends the effects of the behavioral Rx package plus Rx equalization and represents the only location where a meaningful eye height and eye width limits can be defined.

(98) 85ohms (Typical) aligned to Base spec.

(99) Connecting RCOMP at 150 Ω calibrates PCIe and CXL channel on-chip termination to 85 Ω (aligned to Base spec).

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

65

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

F-Tile Transceiver Performance Specifications

F-Tile Transceiver Performance

Table 58.	F-Tile FHT Transmitter and Receiver Data Rate Performance

For specification status, see the Data Sheet Status table

| Symbol/Description | Condition | Transceiver Speed Grade |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | –1 | –2 | –3 |  |
| Supported data rate | NRZ | 24–29, 48–58 | 24–29 | 24–29 | Gbps |
|  | PAM4 | 48–58, 96–116 | 48–58 | 48–58 | Gbps |

Table 59.	F-Tile FGT Transmitter and Receiver Data Rate Performance

For specification status, see the Data Sheet Status table

| Symbol/Description | Condition | Transceiver Speed Grade |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | –1 | –2 | –3 |  |
| Supported data rate | NRZ | 1–32.45 | 1–32 | 1–17.4 | Gbps |
|  | PAM4 | 20–58.125 | 20–58.125 | 20–32 | Gbps |

F-Tile Transceiver Reference Clock Specifications Table 60.	F-Tile FHT Reference Clock Requirements

For specification status, see the Data Sheet Status table

| Parameter | Description | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | Reference clock frequency | — | 100 | 156.25 | 200 | MHz |
| Frequency accuracy | Frequency accuracy of the reference clock, including temperature variability, aging, and initial variation | — | — | — | ±100 | ppm |
| continued... |  |  |  |  |  |  |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

66

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Parameter | Description | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Single sideband phase noise | Measured SSB phase noise including spurs must be smaller than phase noise mask(100) | 10 kHz | — | — | –130 | dB |
|  |  | 100 kHz | — | — | –138 | dB |
|  |  | 500 kHz | — | — | –138 | dB |
|  |  | 3 MHz | — | — | –140 | dB |
|  |  | 10 MHz | — | — | –144 | dB |
|  |  | 20 MHz | — | — | –146 | dB |
|  |  | 1 GHz | — | — | –146 (101) | dB |
| Integrated RMS jitter | Integrated over 10 kHz – 20 MHz, include spurious | — | — | — | 522 | fs |

Table 61.	F-Tile FHT Reference Clocks Input Specifications

LVDS is recommended with on-board 100 nF AC-coupling capacitor.

For specification status, see the Data Sheet Status table

| Parameter | Description | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| TREF-DUTY | Duty cycle | — | 45 | 50 | 55 | % |
| TREF-RISE/FALL | Rising and falling edge rate | 20% – 80% | 40 | — | 300 | ps |
| TREF-SINGLEEND-SKEW | Skew between REFCLKP and REFCLKN | — | — | — | 5 | ps |
| ZREF-SINGLEEND-DC | Reference clock input impedance – terminated mode | — | 40 | 50 | 60 | Ω |
| continued... |  |  |  |  |  |  |

(100) Add an offset of 20 × log10(Fclk/156.25 MHz) dB to mask values, where Fclk is reference clock frequency.

(101) The phase noise mask requirement between 20 MHz and 1 GHz excludes any harmonics power of the fundamental clock.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

67

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Parameter | Description | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VREFIN-SE-PP | Input reference clock single-ended peak-to- peak voltage | — | 200 | — | 510 | mV |
| VREFIN-CM-AC | Input reference clock common-mode voltage when AC- coupled on board | — | Set on-chip (no user access) |  |  | — |
| VREFIN-IL-DC | Input reference clock input low voltage when DC-coupled on board | — | 0.1 | — | — | V |
| VREFIN-IH-DC | Input reference clock input high voltage when DC-coupled on board | — | — | — | 0.9 | V |

Table 62.	F-Tile FGT Reference Clock Input Specifications

For specification status, see the Data Sheet Status table

| Parameter | Description | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Supported I/O standards | — | Dedicated reference clock pin | CML, HCSL |  |  |  |
| FREF | Reference clock operating frequency | — | 100(102) | — | 380 | MHz |
| TREF-DUTY | Duty cycle | — | 45 | 50 | 55 | % |
| TREF-RISE/FALL | Rising and falling edge rate | 20% – 80% | — | — | 0.15 × Tref_period | ps |
| TREF-SINGLEEND-SKEW | Skew between REFCLKP and REFCLKN | — | — | — | 50 | ps |
| continued... |  |  |  |  |  |  |

(102) This value is 100 MHz for down SSC (Spread Spectrum Clocking) clocking. This value can also be 25 MHz for HDMI rate of less than 1 Gbps.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

68

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Parameter | Description | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| ZREF-DIFF-DC (103) | Reference clock differential input impedance – terminated mode | — | 80 | 100 | 120 | Ω |
| VREFIN-DIFF (104) | Input reference clock differential peak-to- peak voltage | — | 0.6 | 1.2 | 1.7 | V |
| VREFIN-IL-DC | Input reference clock input low voltage when DC-coupled on board | — | 0 | — | — | V |
| VREFIN-IH-DC | Input reference clock input high voltage when DC-coupled on board | — | — | — | 1 | V |
| VREFIN-CM-AC | Input reference clock common-mode voltage when AC- coupled on board | — | Set on-chip (no user access) |  |  | V |
| VREFIN-CM-DC | Input reference clock common-mode voltage when DC- coupled on board | — | 0.2 | — | 0.8 | V |
| PNREF-SSB | Reference clock measured single sideband phase noise mask including spurs must be smaller than phase noise mask (105) | 10 kHz | — | — | –130 | dBc/Hz |
| (156.25MHz) |  |  |  |  |  |  |
|  |  | 100 kHz | — | — | –138 | dBc/Hz |
|  |  | 500 kHz | — | — | –138 | dBc/Hz |
| continued... |  |  |  |  |  |  |

(103) These termination resistors are part of the reference clock input buffer on-chip, and are always present and no external termination or DC biasing is needed if AC-coupled on board. If DC-coupled on board, external biasing is not required unless a signaling standard other than differential 100 Ω termination is required.

(104) LVDS is recommended with on-board AC-coupling and subject to 0.6 V ≤ VREFIN-DIFF ≤ 1.7 V.

(105) Add an offset of 20 × log10(Fclk/156.25 MHz) dB to mask values, where Fclk is reference clock frequency.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

69

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Parameter | Description | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 MHz | — | — | –140 | dBc/Hz |
|  |  | 10 MHz | — | — | –144 | dBc/Hz |
|  |  | 20 MHz | — | — | –146 | dBc/Hz |
|  |  | 1 GHz | — | — | –146 | dBc/Hz |
| VREFIN-RJ-RMS | RMS jitter integrated from 10 kHz – 20 MHz including spurs | — | — | — | 522 | fs |
| VREFIN-PPM-ERROR | Reference clock frequency error | — | –350 + SSC | — | +350 + SSC | ppm |

Board

FGT Reference Clock Receiver

Bumps

REF+

50 Ω

ZREF-DIFF-DC

50 Ω

REF-

AC Coupled Network

Figure 4.	Simplified F-Tile FGT Reference Clock Input Buffer

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

70

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Table 63.	F-Tile FGT Reference Clock Output Driver Specifications

For specification status, see the Data Sheet Status table

| Parameter | Description | Condition | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| FREF_OUT | Reference clock operating frequency | — | 25 | — | 800 | MHz |
| TREF-DUTY_OUT | Duty cycle | — | 45 | 50 | 55 | % |
| TREF-RISE_OUT/FALL_OUT | Rising and falling edge rate | 20% – 80% | — | — | 0.15 × Tref_period | ps |
| TREF-SINGLEEND-SKEW | Skew between REFCLKP and REFCLKN | — | — | — | 50 | ps |
| ZREF-DIFF-DC_OUT | Reference clock differential output impedance – terminated mode | — | 80 | 100 | 120 | Ω |
| VREFIN-DIFF-AC_OUT | Output reference clock differential peak to peak voltage when AC-coupled on board | — | 0.9 | 1 | 1.1 | V |
| VREFIN-CM-OUT (106) | Output reference clock common-mode | — | 0.45 | 0.5 | 0.55 | V |

(106) If your far-end differential termination is comprised of two 50 Ω terminations to GND, the common-mode voltage is nominally 250 mV. If your far-end differential termination is comprised of a single 100 Ω differential termination between the P and N signals, the common-mode voltage is nominally 500 mV.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

71

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

F-Tile Transmitter Specifications

Table 64.	F-Tile FHT Transmitter Electrical Specifications

For specification status, see the Data Sheet Status table

| Parameter | Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Output eye specifications | VTX-DIFF-PKPK | Transmit amplitude (low frequency) | 800 | — | 1,200 | mVdiff-pkpk |
|  | VTX-DEEMP_STEP | Transmit tap resolution for c(0), c(1), and c(–1) | 0.5 | — | 5 | % |
|  |  | Transmit tap resolution for c(–2) | 0.5 | — | 2.5 | % |
|  | TTX-SLEW | Rise/fall time | 8 | — | — | ps |
| Transmitter output voltage | VTX-CM OUT | Transmitter output common-mode voltage | 820 | 850 | 880 | mV |
| Transmitter DC impedance | ZTX-DIFF-DC | Transmitter output differential DC impedance 100 W mode while configured | 80 | 100 | 120 | Ω |
|  | ZTX-CM-DC | Transmitter output common-mode DC impedance | 20 | 25 | 30 | Ω |
| Transmitter return loss | ZRL-DIFF-DC | Transmitter differential DC return loss | — | — | 14.5 | dB |
|  | ZRL-DIFF-NYQ | Transmitter differential return loss at Nyquist frequency (FBAUD/2) | — | — | 8 | dB |
|  | ZRL-CMN | Transmitter common- mode return loss below 10 GHz | — | — | 6 | dB |
| Electrical idle | VTX-IDLE | Idle output voltage | — | — | 50 | mVpkpk |
| continued... |  |  |  |  |  |  |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

72

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Parameter | Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | VCM-DELTA-SQUELCH | Maximum common- mode step entering/ exiting squelch mode | — | — | 100 | mV |
|  | TTX-IDLE-LATENCY | Latency entering/ exiting idle (cold boot) | — | — | 28 | ms |
|  |  | Power state cycle (re- establish CM) | — | — | 5 | µs |

Table 65.	F-Tile FGT Transmitter Electrical Specifications

For specification status, see the Data Sheet Status table

| Parameter | Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Output eye specifications | VTX-DIFF-PKPK | Back-porch transmit amplitude | 300 | — | 1,050 | mV |
|  | VTX-DEEMP_STEP | Transmit tap resolution | — | — | 2 | % |
|  | DTX-N+2-DEEMP | N+2 precursor tap de- emphasis | 0 | — | 2.5 | dB |
|  | DTX-N+1-DEEMP | N+1 precursor tap de- emphasis | 0 | — | 4.5 | dB |
|  | DTX-N-1-DEEMP | N-1 postcursor tap de- emphasis | 0 | — | 6.5 | dB |
|  | TTX-SLEW | Rise/fall time at 20%– 80% | 10 | — | 20 | ps |
|  | TTX-DJ | Transmit deterministic jitter at 25 Gbps | — | — | 0.15 | UIpkpk |
|  | TTX-RJ | Transmit total peak- peak random jitter (assumes 14σTXRJ- RMS)(107) | — | — | 0.15 | UIpkpk |
| continued... |  |  |  |  |  |  |

(107) Assume a 1st order high-pass jitter measurement filter with a cutoff of Fbaud/Fgpll = Ngpll.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

73

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Parameter | Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | TTX-TJ | Transmit total peak- peak jitter (TTX-TJ = TTX-DDJ + TTX-PJ + TTX- RJ)(107) | — | — | 0.28 | UIpkpk |
| Transmitter output voltage | VTX-CM OUT | Transmitter output common-mode voltage | 0.45 | 0.5 | 0.55 | V |
| Transmitter DC impedance | ZTX-DIFF-DC | Transmitter output differential DC impedance 90 Ω mode while configured(108) | 80 | 90 | 120 | Ω |
|  | ZTX-CM-DC | Transmitter output common-mode DC impedance | 20 | 25 | 30 | Ω |
| Transmitter return loss | ZRL-DIFF-DC | Transmitter differential DC return loss | — | — | 12 | dB |
|  | ZRL-DIFF-NYQ | Transmitter differential return loss at Nyquist frequency (FBAUD/2) | — | — | 6 | dB |
|  | ZRL-CMN | Transmitter common- mode return loss below 10 GHz | — | — | 6 | dB |
| Electrical idle | VTX-IDLE | Idle output voltage | — | — | 20 | mV |
|  | VCM-DELTA-SQUELCH | Maximum common- mode step entering/ exiting squelch mode | — | — | 100 | mV |
|  | TTX-IDLE-LATENCY | Latency entering/ exiting idle (cold boot), power state cycle (re-establish CM) | — | — | 8 | µs |
| continued... |  |  |  |  |  |  |

(108) TX pins are driven to 0 V before mode configuration.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

74

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Parameter | Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Receiver detect | VTX-RCV-DETECT | Voltage change allowed during receiver detection | — | — | 600 | mV |
| Lane-to-lane output skew | — | Lane count ≤ 8 | — | — | 2 UI + 200 ps | ps |
|  | — | Lane count = 16 | — | — | 2 UI + 300 ps | ps |

  - ile Receiver Specifications

Table 66.	F-Tile FHT Receiver Electrical Specifications

For specification status, see the Data Sheet Status table

| Parameter | Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Receiver input eye specifications | VRX-DIFF-PKPK | Receiver input differential peak-to- peak voltage(109) | Closed eye | — | 1,200 | mVdiff-pkpk |
|  | VRX-CM-DC | Receiver input DC common-mode voltage (110)(109) | 100 | — | 900 | mV |
| continued... |  |  |  |  |  |  |

(109) To support Hot Swap with FHT PMA’s, ensure the following: For AC Coupled connections:

    - RX inputs have external AC coupling caps of at least 100nF.

    - The single-ended voltages applied to the RX_p and RX_n pins should not exceed ±300mV (for a total of 600mV p-p).

    - The total differential voltage (combination of RX_p/RX_n) should not exceed 1,200mV.

    - The FHT RX Analog Parameter selection for External AC cap must be EXTERNAL_AC_CAP_ENABLE. For DC Coupled connections:

    - The maximum amount of time that the unpowered FHT RX PMA can be DC coupled to a link partner who is transmitting is 40 minutes. If you cannot meet this requirement, contact Altera Customer Support.

    - The single-ended voltages applied to the RX_p and RX_n pins should not exceed ±300mV (for a total of 600mV p-p).

    - The total differential voltage (combination of RX_p/RX_n) should not exceed 1,200mV.

    - The FHT RX Analog Parameter selection for External AC cap must be EXTERNAL_AC_CAP_DISABLE.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

75

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Parameter | Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | VRX-MAX | Receiver input maximum voltage(110) | — | — | 1,200 | mV |
|  | VRX-MIN | Receiver input minimum voltage | –200 | — | — | mV |
|  | TRX-DDJ | Receive input signal data dependent jitter (inter-symbol interference) | — | — | 1 | UIpkpk |
|  | TRX-RJ | Receive input random jitter | — | — | 0.15 | UIpkpk |
|  | TRX-PJ | Receive input periodic jitter (at high frequency) | — | — | 0.05 | UIpkpk |
|  | TRX-TJ | Receive input total jitter (DDJ + RJ + PJ) | — | — | 1 | UIpkpk |
| Equalizer specifications | FPPM-OFFSET | Tolerable data frequency offset | –200 | — | 200 | ppm |
| Receiver return loss | ZRL-DIFF-DC | Receiver differential DC return loss | — | — | 10 | dB |
|  | ZRL-DIFF-NYQ | Receiver differential return loss at Nyquist frequency (FBAUD/2) | — | — | 6 | dB |
|  | ZRL-CM | Receiver common- mode return loss below 10 GHz | — | — | 6 | dB |
| Receiver DC impedance | RDIFF-DC | DC differential receive impedance | 80 | 100 | 120 | Ω |
|  | RCM-DC | DC common-mode receive impedance | 20 | 25 | 30 | Ω |

(110) Referenced to RX GND. This specification is also supported before mode configuration.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

76

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Table 67.	F-Tile FGT Receiver Electrical Specifications

For specification status, see the Data Sheet Status table

| Parameter | Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Receiver input eye specifications | VRX-DIFF-PKPK | Receiver input differential peak-to- peak voltage(111) | Closed eye | — | 1,200 | mV |
|  | VRX-MAX | Receiver input maximum voltage (112) | — | — | 1 | V |
|  | VRX-MIN | Receiver input minimum voltage(112) | –0.3 | — | — | V |
|  | VRX-CM-DC | Receiver input DC common-mode voltage(113) | 0 | — | 700 | mV |
|  | TRX-RJ | Receive input random jitter | — | — | 0.15 | UIpkpk |
|  | TRX-PJ | Receive input periodic jitter (at high frequency) | — | — | 0.05 | UIpkpk |
| Insertion loss specifications | IINS-LOSS-56Gb/s | Insertion loss at 56 Gbps PAM42/BER <10–4 | — | — | 30 | dB |
|  | IINS-LOSS-53Gb/s | Insertion loss at 53 Gbps PAM42/BER <10–4 | 5(114) | — | — | dB |
| continued... |  |  |  |  |  |  |

(111) This is supported when the receiver is powered and configured, powered and unconfigured, or unpowered.

(112) VRX_MAX and VRX_MIN are before and after configuration.

(113) The specified common-mode range is supported when the receiver is powered and configured, powered and unconfigured, or unpowered. This specification is also supported before mode configuration. If squelch detect is used, receiver DC input common-mode voltage should be within 200 mV to 300 mV. Otherwise, use AC coupling capacitors on board.

(114) The minimum insertion loss specification assumes a PAM4 transmitter with 800 mVppd. For transmitters with output amplitude adjustment capabilities and can reduce output amplitude to below 800 mVppd, this minimum insertion loss can be further relaxed.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

77

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Parameter | Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | IINS-LOSS-30Gb/s | Insertion loss at 32 Gbps NRZ(115) /BER <10–12 | — | — | 30 | dB |
|  | IINS-LOSS-25Gb/s | Insertion loss at 25.78125 Gbps NRZ(115)/ BER <10–12 | — | — | 30 | dB |
| Receiver return loss | ZRL-DIFF-DC | Receiver differential DC return loss | — | — | 12 | dB |
|  | ZRL-DIFF-NYQ | Receiver differential return loss at Nyquist frequency (FBAUD/2) | — | — | 6 | dB |
|  | ZRL-CM | Receiver common- mode return loss below 10 GHz | — | — | 6 | dB |
| Receiver DC impedance | RDIFF-DC | DC differential receive impedance | 65 | 85 | 102 | Ω |
|  |  |  | 80 | 100 | 120 | Ω |
|  | RCM-DC | DC common-mode receive impedance | 20 | 25 | 30 | Ω |
| Receiver signal detection(116) | VIDLE-THRESH | Receiver signal detect input voltage threshold | 75 | 120 | 175 | mVdiff-pkpk |

(115) COM compliant package and channel.

(116) Receiver signal detection values in this table are applicable to PCIe and similar standards, such as SATA, where a clock pattern like PCIe EIEOS 500 MHz clock pattern is used.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

78

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

  - ile Electrical Compliance

Table 68.	F-Tile FHT Supported Electrical Compliance List

For specification status, see the Data Sheet Status table

| Specification/Clause | Protocol | Encoding | Lane Rate (Gbps) |
| --- | --- | --- | --- |
| IEEE 802.3-2022 IEEE 802.3ck-2022 | 400GAUI-4 | PAM4 | 106.25 |
|  | 200GAUI-2 | PAM4 | 106.25 |
|  | 200GAUI-4 | PAM4 | 53.125 |
| IEEE 802.3-2022 | 100GAUI-1 | PAM4 | 106.25 |
|  | 100GAUI-2 | PAM4 | 53.125 |
|  | 100GAUI-4 | NRZ | 25.78125 |
|  | 50GAUI-1 | PAM4 | 53.125 |
|  | 50GAUI-2 | NRZ | 25.78125 |
| IEEE 802.3-2022 | 25GAUI-1 | NRZ | 25.78125 |
| IEEE 802.3ck-2022 163/162 | 400GBASE-KR4/CR4 | PAM4 | 106.25 |
|  | 200GBASE-KR2/CR2 | PAM4 | 106.25 |
|  | 100GBASE-KR/CR | PAM4 | 106.25 |
| CEI 4.0/5.0 | OIF-CEI-112G VSR/MR/LR | PAM4 | 96 – 116 |
|  | OIF-CEI-56G VSR/MR | PAM4 | 48 – 58 |
|  | OIF-CEI-28G VSR/SR/MR | NRZ | 24 – 29 |

Table 69.	F-Tile FGT Supported Electrical Compliance List

For specification status, see the Data Sheet Status table

| Specification/Clause | Protocol | Encoding | Lane Rate (Gbps) |
| --- | --- | --- | --- |
| IEEE 802.3-2022 | 400GAUI-8 | PAM4 | 53.125 |
|  | 400GAUI-16 | NRZ | 26.5625 |
|  | 200GAUI-4 | PAM4 | 53.125 |
| continued... |  |  |  |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

79

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Specification/Clause | Protocol | Encoding | Lane Rate (Gbps) |
| --- | --- | --- | --- |
|  | 200GAUI-8 | NRZ | 26.5625 |
| IEEE 802.3-2022 | 100GAUI-2 | PAM4 | 53.125 |
|  | 100GAUI-4 | NRZ | 25.78125 |
|  | 50GAUI-1 | PAM4 | 53.125 |
|  | 50GAUI-2 | NRZ | 26.5625 |
| IEEE 802.3-2022 | 25GAUI-1 | NRZ | 25.78125 |
| IEEE 802.3-2022 | 200GBASE-KR4/CR4 | PAM4 | 53.125 |
|  | 100GBASE-KR2/CR2 | PAM4 | 53.125 |
|  | 50GBASE-KR/CR | PAM4 | 53.125 |
| IEEE 802.3-2022 | 100GBASE-KR4 | NRZ | 25.78125 |
|  | 100GBASE-CR4 | NRZ | 25.78125 |
| IEEE 802.3-2022 | 40GBASE-KR4 | NRZ | 10.3125 |
|  | 40GBASE-CR4 | NRZ | 10.3125 |
| IEEE 802.3-2022 | 10GBASE-KR/CR | NRZ | 10.3125 |
| IEEE 802.3-2022 | 10GBASE-KX4 | NRZ | 10.3125 |
| IEEE 802.3-2022 | 25GBASE-KR/CR | NRZ | 25.78125 |
| CEI 4.0/5.0 | OIF-CEI-56G VSR/MR/LR | PAM4 | 36 – 58 |
|  | OIF-CEI-28G VSR/SR/MR | NRZ | 19.9 – 28.1 |
|  | OIF-CEI-25G LR | NRZ | 19.9 – 28.1 |
|  | OIF-CEI-11G SR/MR/LR | NRZ | 9.95 – 11.2 |
|  | OIF-CEI-6G SR/LR | NRZ | 4.976 – 6.375 |
| G.709 G.sup56 G.sup43 G.sup58 | OTU0 | NRZ | 1.327451 |
|  | OTU1 | NRZ | 2.666057 |
|  | OTU1e | NRZ | 11.049107 |
| continued... |  |  |  |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

80

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Specification/Clause | Protocol | Encoding | Lane Rate (Gbps) |
| --- | --- | --- | --- |
|  | OTU2/2e | NRZ | 10.709225/11.095728 |
|  | OTU2f | NRZ | 11.317642 |
|  | OTU2r | NRZ | 12.639086 |
|  | OTU3 | NRZ | 4x10.754603 |
|  | OTU4(OTL4.4) | NRZ | 4x27.952493 |
|  | OTU4(OTL4.10) | NRZ | 10x11.180997 |
| G.709.1 | 100G FlexO-SR (FOIC1.4) | NRZ | 4x27.952369 |
|  | 100G FlexO-SR (FOIC1.2) | PAM4 | 2x55.904737 |
|  | 100G FlexO-SR (FOIC2.4) | PAM4 | 4x55.904737 |
|  | 100G FlexO-SR (FOIC4.8) | PAM4 | 8x55.904737 |
| G.709, G.709.4 | OTU25u | NRZ | 25.781651 |
|  | OTU25 | NRZ | 27.952.493 |
|  | OTU50u (OTU50u.2-RS) | NRZ | 2x26.562.914 |
|  | OTU50u (OTU50u.1-RS) | PAM4 | 53.125827 |
|  | OTU50 (OTU50.2-RS) | NRZ | 2x27.952493 |
|  | OTU50 | PAM4 | 55.904987 |
| PCIE BASE 4.0 | PCIE | NRZ | 2.5, 5, 8, 16 |
| SMPTE 259M | SDI(117) | NRZ | 0.27 |
| SMPTE 292M |  | NRZ | 1.485/1.483516484 |
| SMPTE 372M |  | NRZ | 2.97/2.967032967 |
| SMPTE ST 2081 |  | NRZ | 5.94/5.934065934 |
| SMPTE ST 2082 |  | NRZ | 11.88/11.868131868 |
| continued... |  |  |  |

(117) For SDI protocol support, please contact Altera Customer Support and reference case 15018078585.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

81

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Specification/Clause | Protocol | Encoding | Lane Rate (Gbps) |
| --- | --- | --- | --- |
| CPRI V7.0 | CPRI | NRZ | 1.2288 |
|  |  | NRZ | 2.4576 |
|  |  | NRZ | 3.072 |
|  |  | NRZ | 4.9152 |
|  |  | NRZ | 6.144 |
|  |  | NRZ | 8.11008 |
|  |  | NRZ | 9.8304 |
|  |  | NRZ | 10.1376 |
|  |  | NRZ | 12.16512 |
|  |  | NRZ | 24.33024 |
| JESD204B | JESD204B | NRZ | up to 19.2 |
| JESD204C | JESD204C | NRZ | up to 32.45 |
| DP 2.0 | DisplayPort 2.0 | NRZ | 1.62 |
|  |  | NRZ | 2.7 |
|  |  | NRZ | 5.4 |
|  |  | NRZ | 8.1 |
|  |  | NRZ | 10 |
|  |  | NRZ | 13.5 |
|  |  | NRZ | 20 |
| FC-PI-2 | Fiber Channel | NRZ | 1.0625 |
|  |  | NRZ | 2.125 |
| FC-PI-5 |  | NRZ | 4.25 |
|  |  | NRZ | 8.5 |
| 10GFC |  | NRZ | 10.52 |
| continued... |  |  |  |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

82

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Specification/Clause | Protocol | Encoding | Lane Rate (Gbps) |
| --- | --- | --- | --- |
| FC-PI-5 |  | NRZ | 14.025 |
| FC-PI-6 |  | NRZ | 28.05 |
| FC-PI-7 |  | PAM4 | 57.8 |
| Serial ATA revision 3.0 | Sata Gen 1, 2, 3 | NRZ | 1, 3, 6 |
| T10/BSR INCITS 519 | SAS 1, 2, 3, 4 | NRZ | 3, 6, 12, 22.5 |
| IEEE 802.3-2022 | 10G-EPON | NRZ | 10 |
| IEEE 802.3-2022 | 1G-EPON | NRZ | 1 |
| G.984 | GPON | NRZ | 1.25, 2.5, 10.3 |
| CEI-6G-SR | Interlaken | NRZ | 6.25 |
| CEI-11G-SR |  | NRZ | 10.3125 |
| CEI-11G-SR+ |  | NRZ | 12.5 |
| OIF-28G MR (OIF-CEI3.0) |  | NRZ | 25.78125 |
| OIF-CEI 56G PAM4-MR |  | PAM4 | 53.125 |
| — | SerialLite IV | NRZ | 32 |
| HDMI1.0-1.2a | HDMI(118) | NRZ | 1.65 × 3 = 4.95 |
| HDMI1.3-1.4b |  | NRZ | 3.4 × 3 = 10.2 |
| HDMI2.0-2.0b |  | NRZ | 6 × 3 = 18 |
| HDMI2.1 |  | NRZ | 12 × 4 = 48 |
| SFF8431 | SFP+ | NRZ | 9.8304 – 12.5 |
| ITU-T G.8261 and G.8262 | SyncE | NRZ/PAM4 | 10, 25, 50 |
| InfiniBand™ Architecture Specification | InfiniBand | NRZ | 5, 10, 14.062, 25.78125 |
| RapidIO™ Interconnect Specification | SRIO | NRZ | 1.25, 2.5, 3.125, 6.25, 10.3125 |

(118) For HDMI protocol support, please contact Altera Customer Support and reference case 15018077789

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

83

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

HPS Performance Specifications

This section provides hard processor system (HPS) specifications and timing.

HPS Clock Performance

Table 70.	Maximum HPS Clock Frequencies

For specification status, see the Data Sheet Status table

| Performance | VCCL_HPS (V) | MPU Frequency (MHz) | L3 Frequency (MHz) (l3_main_free_clk) | MPFE Frequency (MHz) |
| --- | --- | --- | --- | --- |
| –1 speed grade | Fixed: 0.95 | 1,500 | 400 | 400 |
|  |  |  |  | 667 |
|  | SmartVID | 1,350 | 400 | 400 |
|  |  |  |  | 667 |
| –2 speed grade | SmartVID | 1,200 | 400 | 334 |
|  |  |  |  | 600 |
| –3 speed grade | SmartVID | 1,000 | 400 | 300 |
|  |  |  |  | 534 |

Related Information

External Memory Interface Spec Estimator

Provides the specific details of the maximum allowed SDRAM operating frequency.

HPS Internal Oscillator Frequency

Table 71.	HPS Internal Oscillator Frequency

For specification status, see the Data Sheet Status table

| Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- |
| Internal oscillator frequency | 150 | 300 | 400 | MHz |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

84

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

HPS PLL Specifications

Table 72.	HPS PLL Input Requirements

| Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- |
| Clock input range | 25 | — | 125 | MHz |
| Clock input accuracy | — | — | 50 | ppm |
| Clock input duty cycle | 45 | 50 | 55 | % |

The main HPS PLL receives its clock signals from the HPS_OSC_CLK pin. Refer to the related information for information about assigning this pin. For specification status, see the Data Sheet Status table

Table 73.	HPS PLL Performance

For specification status, see the Data Sheet Status table

| Description | Minimum | Maximum | Unit |
| --- | --- | --- | --- |
| Main PLL VCO output | — | 3,000 | MHz |
| Peripheral PLL VCO output | — | 3,000 | MHz |
| h2f_user0_clk (119) | — | 500 | MHz |
| h2f_user1_clk(119) | — | 500 | MHz |

HPS Cold Reset

Table 74.	HPS Cold Reset

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Maximum | Unit |
| --- | --- | --- | --- | --- |
| tRST0 | Minimum time for HPS_COLD_nRESET asserted (120) | 3 | — | ms |

(119) The HPS PLL provides this clock to the FPGA fabric.

(120) HPS_COLD_nRESET may be ignored if HPS is not running or if the device is being configured.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

85

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

HPS SPI Timing Characteristics

Table 75.	SPI Master Timing Requirements

You can adjust the input delay timing by programming the rx_sample_dly register.

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tspi_ref_clk | The period of the SPI internal reference clock, sourced from l4_main_clk | 2.5 | — | — | ns |
| Tclk | SPIM_CLK clock period | 16.67 | — | — | ns |
| Tdutycycle | SPIM_CLK duty cycle | 45 | 50 | 55 | % |
| Tck_jitter | SPIM_CLK output jitter | — | — | 2 | % |
| Tdio | Master-out slave-in (MOSI) output skew | –3 | — | 2 | ns |
| Tdssfrst (121) | SPI_SS_N asserted to first SPIM_CLK edge | (1.5 × Tclk) – 2 | — | — | ns |
| Tdsslst (121) | Last SPIM_CLK edge to SPI_SS_N deasserted | Tclk 2 | — | — | ns |
| Tsu (122) | SPIM_MISO setup time with respect to SPIM_CLK capture edge | 5.0 – (rx_sample_dly × Tspi_ref_clk)(123) | — | — | ns |
| Th (122) | Input hold in respect to SPIM_CLK capture edge | 1.3 + (rx_sample_dly × Tspi_ref_clk) | — | — | ns |

(121) SPI_SS_N behavior differs depending on Motorola SPI, TI SSP, or Microwire operational mode.

(122) The capture edge differs depending on the operational mode. For Motorola SPI, the capture edge can be the rising or falling edge depending on the scpol register bit; for TI SSP, the capture edge is the falling edge; for Microwire, the capture edge is the rising edge.

(123) Valid values of rx_sample_dly range from 1 to 64 (units are in Tspi_ref_clk steps).

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

86

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Figure 5.	SPI Master Output Timing Diagram

scph* = 0

Tdssfrst

Tdio (max)

Tdsslst

Tdio (min)

OUT1

OUTn

OUT0

SPI_SS

SPI_CLK (scpol = 0) SPI_CLK (scpol = 1)

SPI_MOSI

SPI_MISO

Tdssfrst

Tdio (max)

Tdio (min)

Tdsslst

OUT0

OUT1

OUTn

scph* = 1

SPI_SS

SPI_CLK (scpol = 0) SPI_CLK (scpol = 1)

SPI_MOSI

SPI_MISO

*Serial clock phase configuration bit, in the SPI controller’s CTRLR0 register

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

87

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Figure 6.	SPI Master Input Timing Diagram

Tsu

Th

INn

scph* = 0

SPI_SS

SPI_CLK (scpol = 0) SPI_CLK (scpol = 1)

SPI_MOSI

IN0

IN1

SPI_MISO

scph* = 1

Tsu

Th

INn

SPI_SS

SPI_CLK (scpol = 0) SPI_CLK (scpol = 1)

SPI_MOSI

IN0

IN1

SPI_MISO

*Serial clock phase configuration bit, in the SPI controller’s CTRLR0 register

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

88

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Table 76.	SPI Slave Timing Requirements

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tspi_ref_clk | The period of the SPI internal reference clock, sourced from l4_main_clk | 2.5 | — | — | ns |
| Tclk | SPIM_CLK clock period | 30 | — | — | ns |
| Tdutycycle | SPIM_CLK duty cycle | 45 | 50 | 55 | % |
| Td | Master-in slave-out (MISO) output skew | (2 × Tspi_ref_clk) + 3 | — | (3 × Tspi_ref_clk) + 11 | ns |
| Tsu | Master-out slave-in (MOSI) setup time | 4 | — | — | ns |
| Th | Master-out slave-in (MOSI) hold time | 9 | — | — | ns |
| Tsuss | SPI_SS_N asserted to first SPIM_CLK edge | Tspi_ref_clk + 4.2 | — | — | ns |
| Thss | Last SPIM_CLK edge to SPI_SS_N deasserted | Tspi_ref_clk + 4.2 | — | — | ns |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

89

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Figure 7.	SPI Slave Output Timing Diagram

scph* = 0

Td (max)

Td (min)

OUT0

OUT1

SPI_SS

SPI_CLK (scpol = 0) SPI_CLK (scpol = 1)

OUTn

SPI_MISO

SPI_MOSI

Td (max)

Td (min)

OUT0

OUT1

scph* = 1

SPI_SS

SPI_CLK (scpol = 0) SPI_CLK (scpol = 1)

OUTn

SPI_MISO

SPI_MOSI

*Serial clock phase configuration bit, in the SPI controller’s CTRLR0 register

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

90

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Figure 8.	SPI Slave Input Timing Diagram

scph* = 0

Tsuss

Thss

T

s

IN0

IN1

Th

INn

SPI_SS

SPI_CLK (scpol = 0) SPI_CLK (scpol = 1)

SPI_MISO

SPI_MOSI

Tsuss

Thss

Ts

Th

IN0

IN1

INn

scph* = 1

SPI_SS

SPI_CLK (scpol = 0) SPI_CLK (scpol = 1)

SPI_MISO

SPI_MOSI

*Serial clock phase configuration bit, in the SPI controller’s CTRLR0 register

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

91

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

HPS SD/MMC Timing Characteristics

Table 77.	HPS Secure Digital (SD)/MultiMediaCard (MMC) Timing Requirements

These timings apply to SD, MMC, and embedded MMC (eMMC) cards operating at 1.8 V.

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tsdmmc_cclk | SDMMC_CCLK clock period (Identification mode) | 2,500 | — | — | ns |
|  | SDMMC_CCLK clock period (SDR12) | 40 | — | — | ns |
|  | SDMMC_CCLK clock period (SDR25) | 20 | — | — | ns |
| Tdutycycle | SDMMC_CCLK duty cycle | 45 | 50 | 55 | % |
| Tsdmmc_cclk_jitter | SDMMC_CCLK output jitter | — | — | 2 | % |
| Tsdmmc_clk | Internal reference clock before division by 4 | 5 | — | — | ns |
| Td | SDMMC_CMD/ SDMMC_DATA[7:0] output delay(124) | –0.5 + Tsdmmc_clk × drvsel/2 | — | 2.5 + (Tsdmmc_clk × drvsel/2) | ns |
| Tsu | SDMMC_CMD/ SDMMC_DATA[7:0] input setup (125) | 6 – (Tsdmmc_clk × smplsel/2) | — | — | ns |
| Th | SDMMC_CMD/ SDMMC_DATA[7:0] input hold(125) | 0.5 + (Tsdmmc_clk × smplsel/2) | — | — | ns |

(124) When the drvsel bitfield in the sdmmc register is set to 3 (in the system manager) and the reference clock (sdmmc_clk) is 200 MHz for example, the output delay time is 7.5 to 10.5 ns.

(125) When the smplsel bitfield in the sdmmc register is set to 2 (in the system manager) and the reference clock (sdmmc_clk) is 200 MHz for example, the setup time is 1 ns and the hold time is 5.5 ns.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

92

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

None of the HPS I/Os supports 3 V mode, while SD/MMC cards must operate at 3 V at power on. eMMC devices can operate at

  - V at power on.

Note:	SD cards power up at 3 V. To support SD, your design must include a level shifter between the SD card and the HPS SD/MMC interface.

Figure 9.	SD/MMC Timing Diagram

Td

Command/Data Out

TSU

Th

SDMMC_CCLK

SDMMC_CMD and SDMMC_DATA (Out)

SDMMC_CMD and SDMMC_DATA (In)

Command/Data In

HPS USB UPLI Timing Characteristics

Table 78.	HPS USB 2.0 Transceiver Macrocell Interface Plus (UTMI+) Low Pin Interface (ULPI) Timing Requirements

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tusb_clk | USB_CLK clock period | — | 16.667 | — | ns |
| Td | Clock to USB_STP/ USB_DATA[7:0] output delay | 2 | — | 7 | ns |
| Tsu | Setup time for USB_DIR/ USB_NXT/USB_DATA[7:0] | 4 | — | — | ns |
| Th | Hold time for USB_DIR/ USB_NXT/USB_DATA[7:0] | 1 | — | — | ns |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

93

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Figure 10.	USB ULPI Timing Diagram

Td

To PHY

From PHY

TSU

Th

USB_CLK USB_STP

USB_DATA[7:0]

USB_DIR and USB_NXT

Note:	The USB interface supports single data rate (SDR) timing only.

Note:	If you need to adjust the timings of certain signals, you can use the HPS registers Pin_Mux.io0_delay through Pin_Mux.io47_delay to allow software to set the delay chains in each of the dedicated I/Os. For example, to add output and input delay to the USB_DATA3 signal (HPS_IOA_8), program Pin_Mux.io7_delay.output_val_en = 1, and Pin_Mux.io7_delay.output_val = 15 to add approximately 1.6 ns output delay from the HPS, and program Pin_Mux.io7_delay.input_val_en = 3, and Pin_Mux.io7_delay.input_val = 30 to add approximately 2.9 ns input delay into the HPS. See HPS Programmable I/O Timing Characteristics for more information.

HPS Ethernet Media Access Controller (EMAC) Timing Characteristics

Table 79.	Reduced Gigabit Media Independent Interface (RGMII) TX Timing Requirements

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tclk (1000Base-T) | TX_CLK clock period | — | 8 | — | ns |
| Tclk (100Base-T) | TX_CLK clock period | — | 40 | — | ns |
| Tclk (10Base-T) | TX_CLK clock period | — | 400 | — | ns |
| continued... |  |  |  |  |  |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

94

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tdutycycle (1000Base-T) | TX_CLK duty cycle | 45 | 50 | 55 | % |
| Tdutycycle (10/100Base-T) | TX_CLK duty cycle | 40 | 50 | 60 | % |
| Td (126) (127) | TXD/TX_CTL to TX_CLK output skew | –0.5 | — | 0.5 | ns |

Figure 11.	RGMII TX Timing Diagram

D0

D1

Td

TX_CLK TX_D[3:0]

TX_CTL

Table 80.	RGMII RX Timing Requirements

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tclk (1000Base-T) | RX_CLK clock period | — | 8 | — | ns |
| Tclk (100Base-T) | RX_CLK clock period | — | 40 | — | ns |
| Tclk (10Base-T) | RX_CLK clock period | — | 400 | — | ns |
| Tdutycycle (1000Base-T) | RX_CLK duty cycle | 45 | 50 | 55 | % |
| continued... |  |  |  |  |  |

(126) Rise and fall times depend on the I/O standard, drive strength, and loading. Altera recommends simulating your configuration.

(127) If you connect a PHY that does not implement clock-to-data skew, you can delay TX_CLK by 1.5—2.0 ns with the HPS I/O programmable delay, to meet the PHY's 1-ns data-to-clock skew requirement.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

95

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tdutycycle (10/100Base-T) | RX_CLK duty cycle | 40 | 50 | 60 | % |
| Tsu | RX_D/RX_CTL to RX_CLK setup time | 1 | — | — | ns |
| Th (128) | RX_CLK to RX_D/RX_CTL hold time | 1 | — | — | ns |

Figure 12.	RGMII RX Timing Diagram

TSU

Th

D0

D1

RX_CLK

RX_D[3:0] RX_CTL

Table 81.	Reduced Media Independent Interface (RMII) Clock Timing Requirements

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tclk | REF_CLK clock period, sourced by HPS TX_CLK | — | 20 | — | ns |
|  | REF_CLK clock period, sourced by external clock source | — | 20 | — | ns |
| Tdutycycle_int | Clock duty cycle, REF_CLK sourced by TX_CLK | 35 | 50 | 65 | % |
| Tdutycycle_ext | Clock duty cycle, REF_CLK sourced by external clock source | 35 | 50 | 65 | % |

(128) If you connect a PHY that does not implement clock-to-data skew, you can meet the HPS EMAC’s 1 ns setup time by delaying RX_CLK by 1.5 – 2 ns, using the HPS I/O programmable delay.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

96

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Table 82.	RMII TX Timing Requirements

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Td | TX_CLK to TXD/TX_CTL output data delay | 2 | — | 10 | ns |

Figure 13.	RMII TX Timing Diagram

D0

D1

Td

Reference Clock (RX_CLK) (1)

TX_D[1:0]

TX_CTL

Note:

    - For RMII mode, RX_CLK is always used as the reference clock. Refer to the related information for example system-level topologies.

Table 83.	RMII RX Timing Requirements

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tsu | RX_D/RX_CTL setup time | 2 | — | — | ns |
| Th | RX_D/RX_CTL hold time | 1 | — | — | ns |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

97

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Figure 14.	RMII RX Timing Diagram

TSU

Th

D0

D1

Reference Clock (RX_CLK) (1)

RX_D[1:0]

RX_CTL

Note:

1. For RMII mode, RX_CLK is always used as the reference clock. Refer to the related information for example system-level topologies.

Table 84.	Management Data Input/Output (MDIO) Timing Requirements

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tclk | MDC clock period | 400 | — | — | ns |
| Td | MDC to MDIO output data delay | 10 | — | 300 | ns |
| Tsu | Setup time for MDIO data | 10 | — | — | ns |
| Th | Hold time for MDIO data | 0 | — | — | ns |

Figure 15.	MDIO Timing Diagram

Td

Dout0

Dout1

TSU

Th

Din0

MDC MDIO_OUT

MDIO_IN

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

98

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Related Information

HPS-to-PHY Interface Diagrams section, Agilex 7 Hard Processor System Technical Reference Manual Provides the example system-level topologies.

HPS I2C Timing Characteristics

Table 85.	HPS I2C Timing Requirements

For specification status, see the Data Sheet Status table

| Symbol | Description | Standard Mode |  | Fast Mode |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Maximum | Minimum | Maximum |  |
| Tclk | Serial clock (SCL) clock period | 10 | — | 2.5 | — | μs |
| Tclk_jitter | I2C clock output jitter | — | 2 | — | 2 | % |
| THIGH (129) | SCL high period | 4 (130) | — | 0.6 (131) | — | μs |
| TLOW (132) | SCL low period | 4.7 (133) | — | 1.3(134) | — | μs |
| TSU;DAT | Setup time for serial data line (SDA) data to SCL | 0.25 | — | 0.1 | — | μs |
| continued... |  |  |  |  |  |  |

(129) You can adjust Thigh using the ic_ss_scl_hcnt or ic_fs_scl_hcnt register.

(130) The recommended minimum setting for ic_ss_scl_hcnt is 428. Refer to the related information for the SCL_High_time equation.

(131) The recommended minimum setting for ic_fs_scl_hcnt is 75. Refer to the related information for the SCL_High_time equation.

(132) You can adjust Tlow using the ic_ss_scl_lcnt or ic_fs_scl_lcnt register.

(133)  The recommended minimum setting for ic_ss_scl_lcnt is 464. Refer to the related information for the SCL_Low_time equation.

(134)  The recommended minimum setting for ic_fs_scl_lcnt is 163. Refer to the related information for the SCL_Low_time equation.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

99

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Standard Mode |  | Fast Mode |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Maximum | Minimum | Maximum |  |
| THD;DAT (135) | Hold time for SCL to SDA data | 0 | 3.15 | 0 | 0.6 | μs |
| TVD;DAT and TVD;ACK (136) | SCL to SDA output data delay | — | 3.45(137) | — | 0.9 (138) | μs |
| TSU;STA | Setup time for a repeated start condition | 4.7 | — | 0.6 | — | μs |
| THD;STA | Hold time for a repeated start condition | 4 | — | 0.6 | — | μs |
| TSU;STO | Setup time for a stop condition | 4 | — | 0.6 | — | μs |
| TBUF | SDA high pulse duration between STOP and START | 4.7 | — | 1.3 | — | μs |
| Tscl:r (139) | SCL rise time | — | 1,000 | 20 | 300 | ns |
| Tscl:f (139) | SCL fall time | — | 300 | 6.54 | 300 | ns |
| Tsda:r (139) | SDA rise time | — | 1,000 | 20 | 300 | ns |
| Tsda:f (139) | SDA fall time | — | 300 | 6.54 | 300 | ns |

(135) THD;DAT is affected by the rise and fall time.

(136) TVD;DAT and TVD;ACK are affected by the rise and fall time, as well as the SDA hold time (set by adjusting the ic_sda_hold register).

(137) Use maximum SDA_HOLD = 240 to be within the specification.

(138) Use maximum SDA_HOLD = 60 to be within the specification.

(139) Rise and fall time parameters vary depending on external factors such as the characteristics of the I/O driver, pull-up resistor value, and total capacitance on the transmission line.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

100

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Figure 16.	I2C Timing Diagram

SDA

SCL

70%

tf

tr

tSU;DAT

t

tHD;DAT

t

tHIGH

t

VD;DAT

f

r

tHD;STA

Tclk

tLOW

30%

70%

30%

tSU;STA

tHD;STA

tVD;ACK

tSU;STO

SDA

SCL

tBUF

70%

30%

70%

30%

Related Information

Clock Synchronization section, Agilex 7 Hard Processor System Technical Reference Manual Provides the SCL high and low time equations.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

101

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

HPS NAND Timing Characteristics

Table 86.	HPS NAND ONFI 1.0 Timing Requirements

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Maximum | Unit |
| --- | --- | --- | --- | --- |
| TWP (140) | Write enable pulse width | 10 | — | ns |
| TWH (140) | Write enable hold time | 7 | — | ns |
| TRP (140) | Read enable pulse width | 10 | — | ns |
| TREH (140) | Read enable hold time | 7 | — | ns |
| TCLS (140) | Command latch enable to write enable setup time | 10 | — | ns |
| TCLH (140) | Command latch enable to write enable hold time | 5 | — | ns |
| TCS (140) | Chip enable to write enable setup time | 15 | — | ns |
| TCH (140) | Chip enable to write enable hold time | 5 | — | ns |
| TALS (140) | Address latch enable to write enable setup time | 10 | — | ns |
| TALH (140) | Address latch enable to write enable hold time | 5 | — | ns |
| TDS (140) | Data to write enable setup time | 7 | — | ns |
| TDH (140) | Data to write enable hold time | 5 | — | ns |
| TWB (140) | Write enable high to R/B low | — | 200 | ns |
| TCEA | Chip enable to data access time | — | 100 | ns |
| continued... |  |  |  |  |

(140) This timing is software programmable. Refer to the related information for more information about software-programmable timing in the NAND flash controller.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

102

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Minimum | Maximum | Unit |
| --- | --- | --- | --- | --- |
| TREA | Read enable to data access time | — | 40 | ns |
| TRHZ | Read enable to data high impedance | — | 200 | ns |
| TRR | Ready to read enable low | 20 | — | ns |

Figure 17.	NAND Command Latch Timing Diagram

tCLS

tCS

tCLH

tCH

tWP

tALS

tALH

tDS	tDH

Command

tWB

CLE

CE

WE ALE

IO0-7

R/B

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

103

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Figure 18.	NAND Address Latch Timing Diagram

tCLS

tCS

tWP

tWH

tALS

tDS

tALH

tDH

Address

CLE

CE

WE

ALE

IO0-7

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

104

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

tCLH

tCH

tWP

tDS	tDH

DOUT n

Figure 19.	NAND Data Output Cycle Timing Diagram

tWP

tWP

tWH

tALS

tDS	tDH

tDS	tDH

DOUT 0

DOUT 1

CLE

CE

WE

ALE

IOx

tCEA

tRP

tRP

tRP

tRR

tREH

tREA

tRHZ

DIN 0

tREA

tRHZ

DIN 1

tREA

tRHZ

DIN n

Figure 20.	NAND Data Input Cycle Timing Diagram

CE

RE

R/B IOx

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

105

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

tRP

tREH

tRR

tREA

tREA

DIN 0

DIN 1

tRHZ

DIN n

tCEA

Figure 21.	NAND Data Input Timing Diagram for Extended Data Output (EDO) Cycle

CE

RE

R/B IOx

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

106

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Figure 22.	NAND Read Status Timing Diagram

tCLS

tCS

tCLH

tCH

tCEA

tWP

tRHZ

tDS

tDH

70h

Status

tREA

CLE

CE

WE

RE IO0-7

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

107

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

tCLS

tCS

tCLH

tCH

tCEA

tWP

t

ALH

tWP

t

t

ALS

tWH

ALH

tDS

tDH

78h

tREA

tRHZ

R1

R2

R3

Status

Figure 23.	NAND Read Status Enhanced Timing Diagram

CLE

CE

WE ALE

RE

IO0-7

Related Information

NAND Flash Controller section, Agilex 7 Hard Processor System Technical Reference Manual Provides more information about software-programmable timing in the NAND flash controller

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

108

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

HPS Trace Timing Characteristics

Table 87.	Trace Timing Requirements

To increase the trace bandwidth, Altera recommends routing the trace interface to the FPGA in the HPS Platform Designer component. The FPGA trace interface offers a 64-bit single data rate path that can be converted to double data rate to minimize FPGA I/O usage.

Depending on the trace module that you connect to the HPS trace interface, you may need to include board termination to achieve the maximum sampling speed possible. Refer to your trace module data sheet for termination recommendations.

Most trace modules implement programmable clock and data skew, to improve trace data timing margins. Alternatively, you can change the clock-to-data timing relationship with the HPS programmable I/O delay.

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tclk | Trace clock period | 6.667 | — | — | ns |
| Tclk_jitter | Trace clock output jitter | — | — | 2 | % |
| Tdutycycle | Trace clock maximum duty cycle | 45 | 50 | 55 | % |
| Td | Tclk to D0–D15 output data delay | –0.5 | — | 1.3 | ns |

Figure 24.	Trace Timing Diagram

Clock (DDR)

Trace Data (DDR)

Td

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

109

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

HPS GPIO Interface

The general-purpose I/O (GPIO) interface has debounce circuitry included to remove signal glitches. The debounce clock frequency ranges from 125 Hz to 32 kHz. The minimum pulse width is 1 debounce clock cycle and the minimum detectable GPIO pulse width is 62.5 µs (at 32 kHz).

If the external signal is driven into the GPIO for less than one clock cycle, the external signal is filtered. If the external signal is between one and two clock cycles, the external signal may or may not be filtered depending on the phase of the signal. If the external signal is more than two clock cycles, the external signal is not filtered.

HPS JTAG Timing Characteristics

Table 88.	HPS JTAG Timing Requirements

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| tJCP | TCK clock period | 41.66 | — | — | ns |
| tJCH | TCK clock high time | 20 | — | — | ns |
| tJCL | TCK clock low time | 20 | — | — | ns |
| tJPSU (TDI) | TDI JTAG port setup time | 5 | — | — | ns |
| tJPSU (TMS) | TMS JTAG port setup time | 5 | — | — | ns |
| tJPH | JTAG port hold time | 0.5 | — | — | ns |
| tJPCO | JTAG port clock to output | 0 | — | 8 | ns |
| tJPZX | JTAG port high impedance to valid output | — | — | 10 | ns |
| tJPXZ | JTAG port valid output to high impedance | — | — | 10 | ns |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

110

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Figure 25.	HPS JTAG Timing Diagram

tJCP

tJCH	tJCL

JPSU	JPH

t

t

tJPZX

tJPCO

tJPXZ

TMS TDI

TCK TDO

HPS Programmable I/O Timing Characteristics

Table 89.	HPS Programmable I/O Delay (Output Path)

For specification status, see the Data Sheet Status table

| Name | output_val_en | output_val | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ZERO_CHAIN_DEL AY | 0 | 0 | Intrinsic I/O delay. Bypasses the delay chain | — | 0 | — | ps |
| CHAIN_DELAY | 1 | 0 | Intrinsic I/O delay + Minimum + 0 × Chain Delay | — | 0 | — | ps |
| ONE_CHAIN_DELA Y | 1 | 1 | Intrinsic I/O delay + Minimum + 1 × Chain Delay | — | 422 | — | ps |
| TWO_CHAIN_DELA Y | 1 | 2 | Intrinsic I/O delay + Minimum + 2 × Chain Delay | — | 518 | — | ps |
| THREE_CHAIN_DEL AY | 1 | 3 | Intrinsic I/O delay + Minimum + 3 × Chain Delay | — | 607 | — | ps |
| FOUR_CHAIN_DEL AY | 1 | 4 | Intrinsic I/O delay + Minimum + 4 × Chain Delay | — | 705 | — | ps |
| continued... |  |  |  |  |  |  |  |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

111

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Name | output_val_en | output_val | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FIVE_CHAIN_DELA Y | 1 | 5 | Intrinsic I/O delay + Minimum + 5 × Chain Delay | — | 786 | — | ps |
| SIX_CHAIN_DELAY | 1 | 6 | Intrinsic I/O delay + Minimum + 6 × Chain Delay | — | 874 | — | ps |
| SEVEN_CHAIN_DEL AY | 1 | 7 | Intrinsic I/O delay + Minimum + 7 × Chain Delay | — | 955 | — | ps |
| EIGHT_CHAIN_DEL AY | 1 | 8 | Intrinsic I/O delay + Minimum + 8 × Chain Delay | — | 1,042 | — | ps |
| NINE_CHAIN_DELA Y | 1 | 9 | Intrinsic I/O delay + Minimum + 9 × Chain Delay | — | 1,126 | — | ps |
| TEN_CHAIN_DELAY | 1 | 10 | Intrinsic I/O delay + Minimum + 10 × Chain Delay | — | 1,214 | — | ps |
| ELEVEN_CHAIN_DE LAY | 1 | 11 | Intrinsic I/O delay + Minimum + 11 × Chain Delay | — | 1,296 | — | ps |
| TWELVE_CHAIN_D ELAY | 1 | 12 | Intrinsic I/O delay + Minimum + 12 × Chain Delay | — | 1,382 | — | ps |
| THIRTEEN_CHAIN_ DELAY | 1 | 13 | Intrinsic I/O delay + Minimum + 13 × Chain Delay | — | 1,462 | — | ps |
| FOURTEEN_CHAIN _DELAY | 1 | 14 | Intrinsic I/O delay + Minimum + 14 × Chain Delay | — | 1,552 | — | ps |
| FIFTEEN_CHAIN_D ELAY | 1 | 15 | Intrinsic I/O delay + Minimum + 15 × Chain Delay | — | 1,626 | — | ps |
| — | 1 | [16:30] | INVALID | — | — | — | — |
| — | 2 | — | INVALID | — | — | — | — |
| continued... |  |  |  |  |  |  |  |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

112

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Name | output_val_en | output_val | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| — | 3 | [0:15] | INVALID | — | — | — | — |
| SIXTEEN_CHAIN_D ELAY | 3 | 16 | Intrinsic I/O delay + Minimum + 16 × Chain Delay | — | 1,798 | — | ps |
| SEVENTEEN_CHAIN _DELAY | 3 | 17 | Intrinsic I/O delay + Minimum + 17 × Chain Delay | — | 1,885 | — | ps |
| EIGHTEEN_CHAIN_ DELAY | 3 | 18 | Intrinsic I/O delay + Minimum + 18 × Chain Delay | — | 1,967 | — | ps |
| NINETEEN_CHAIN_ DELAY | 3 | 19 | Intrinsic I/O delay + Minimum + 19 × Chain Delay | — | 2,054 | — | ps |
| TWENTY_CHAIN_D ELAY | 3 | 20 | Intrinsic I/O delay + Minimum + 20 × Chain Delay | — | 2,137 | — | ps |
| TWENTYONE_CHAI N_DELAY | 3 | 21 | Intrinsic I/O delay + Minimum + 21 × Chain Delay | — | 2,222 | — | ps |
| TWENTYTWO_CHAI N_DELAY | 3 | 22 | Intrinsic I/O delay + Minimum + 22 × Chain Delay | — | 2,305 | — | ps |
| TWENTYTHREE_CH AIN_DELAY | 3 | 23 | Intrinsic I/O delay + Minimum + 23 × Chain Delay | — | 2,395 | — | ps |
| TWENTYFOUR_CHA IN_DELAY | 3 | 24 | Intrinsic I/O delay + Minimum + 24 × Chain Delay | — | 2,475 | — | ps |
| TWENTYFIVE_CHAI N_DELAY | 3 | 25 | Intrinsic I/O delay + Minimum + 25 × Chain Delay | — | 2,564 | — | ps |
| TWENTYSIX_CHAIN _DELAY | 3 | 26 | Intrinsic I/O delay + Minimum + 26 × Chain Delay | — | 2,644 | — | ps |
| continued... |  |  |  |  |  |  |  |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

113

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Name | output_val_en | output_val | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TWENTYSEVEN_CH AIN_DELAY | 3 | 27 | Intrinsic I/O delay + Minimum + 27 × Chain Delay | — | 2,732 | — | ps |
| TWENTYEIGHT_CH AIN_DELAY | 3 | 28 | Intrinsic I/O delay + Minimum + 28 × Chain Delay | — | 2,808 | — | ps |
| TWENTYNINE_CHA IN_DELAY | 3 | 29 | Intrinsic I/O delay + Minimum + 29 × Chain Delay | — | 2,901 | — | ps |
| THIRTY_CHAIN_DE LAY | 3 | 30 | Intrinsic I/O delay + Minimum + 30 × Chain Delay | — | 2,979 | — | ps |

Table 90.	HPS Programmable I/O Delay (Input Path)

For specification status, see the Data Sheet Status table

| Name | input_val_en | input_val | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ZERO_CHAIN_DEL AY | 0 | 0 | Intrinsic I/O delay. Bypasses the delay chain | — | 0 | — | ps |
| CHAIN_DELAY | 1 | 0 | Intrinsic I/O delay + Minimum + 0 × Chain Delay | — | 0 | — | ps |
| ONE_CHAIN_DELA Y | 1 | 1 | Intrinsic I/O delay + Minimum + 1 × Chain Delay | — | 422 | — | ps |
| TWO_CHAIN_DELA Y | 1 | 2 | Intrinsic I/O delay + Minimum + 2 × Chain Delay | — | 518 | — | ps |
| THREE_CHAIN_DEL AY | 1 | 3 | Intrinsic I/O delay + Minimum + 3 × Chain Delay | — | 607 | — | ps |
| FOUR_CHAIN_DEL AY | 1 | 4 | Intrinsic I/O delay + Minimum + 4 × Chain Delay | — | 705 | — | ps |
| continued... |  |  |  |  |  |  |  |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

114

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Name | input_val_en | input_val | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FIVE_CHAIN_DELA Y | 1 | 5 | Intrinsic I/O delay + Minimum + 5 × Chain Delay | — | 786 | — | ps |
| SIX_CHAIN_DELAY | 1 | 6 | Intrinsic I/O delay + Minimum + 6 × Chain Delay | — | 874 | — | ps |
| SEVEN_CHAIN_DEL AY | 1 | 7 | Intrinsic I/O delay + Minimum + 7 × Chain Delay | — | 955 | — | ps |
| EIGHT_CHAIN_DEL AY | 1 | 8 | Intrinsic I/O delay + Minimum + 8 × Chain Delay | — | 1,042 | — | ps |
| NINE_CHAIN_DELA Y | 1 | 9 | Intrinsic I/O delay + Minimum + 9 × Chain Delay | — | 1,126 | — | ps |
| TEN_CHAIN_DELAY | 1 | 10 | Intrinsic I/O delay + Minimum + 10 × Chain Delay | — | 1,214 | — | ps |
| ELEVEN_CHAIN_DE LAY | 1 | 11 | Intrinsic I/O delay + Minimum + 11 × Chain Delay | — | 1,296 | — | ps |
| TWELVE_CHAIN_D ELAY | 1 | 12 | Intrinsic I/O delay + Minimum + 12 × Chain Delay | — | 1,382 | — | ps |
| THIRTEEN_CHAIN_ DELAY | 1 | 13 | Intrinsic I/O delay + Minimum + 13 × Chain Delay | — | 1,462 | — | ps |
| FOURTEEN_CHAIN _DELAY | 1 | 14 | Intrinsic I/O delay + Minimum + 14 × Chain Delay | — | 1,552 | — | ps |
| FIFTEEN_CHAIN_D ELAY | 1 | 15 | Intrinsic I/O delay + Minimum + 15 × Chain Delay | — | 1,626 | — | ps |
| — | 1 | [16:30] | INVALID | — | — | — | — |
| — | 2 | — | INVALID | — | — | — | — |
| continued... |  |  |  |  |  |  |  |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

115

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Name | input_val_en | input_val | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| — | 3 | [0:15] | INVALID | — | — | — | — |
| SIXTEEN_CHAIN_D ELAY | 3 | 16 | Intrinsic I/O delay + Minimum + 16 × Chain Delay | — | 1,798 | — | ps |
| SEVENTEEN_CHAIN _DELAY | 3 | 17 | Intrinsic I/O delay + Minimum + 17 × Chain Delay | — | 1,885 | — | ps |
| EIGHTEEN_CHAIN_ DELAY | 3 | 18 | Intrinsic I/O delay + Minimum + 18 × Chain Delay | — | 1,967 | — | ps |
| NINETEEN_CHAIN_ DELAY | 3 | 19 | Intrinsic I/O delay + Minimum + 19 × Chain Delay | — | 2,054 | — | ps |
| TWENTY_CHAIN_D ELAY | 3 | 20 | Intrinsic I/O delay + Minimum + 20 × Chain Delay | — | 2,137 | — | ps |
| TWENTYONE_CHAI N_DELAY | 3 | 21 | Intrinsic I/O delay + Minimum + 21 × Chain Delay | — | 2,222 | — | ps |
| TWENTYTWO_CHAI N_DELAY | 3 | 22 | Intrinsic I/O delay + Minimum + 22 × Chain Delay | — | 2,305 | — | ps |
| TWENTYTHREE_CH AIN_DELAY | 3 | 23 | Intrinsic I/O delay + Minimum + 23 × Chain Delay | — | 2,395 | — | ps |
| TWENTYFOUR_CHA IN_DELAY | 3 | 24 | Intrinsic I/O delay + Minimum + 24 × Chain Delay | — | 2,475 | — | ps |
| TWENTYFIVE_CHAI N_DELAY | 3 | 25 | Intrinsic I/O delay + Minimum + 25 × Chain Delay | — | 2,564 | — | ps |
| TWENTYSIX_CHAIN _DELAY | 3 | 26 | Intrinsic I/O delay + Minimum + 26 × Chain Delay | — | 2,644 | — | ps |
| continued... |  |  |  |  |  |  |  |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

116

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Name | input_val_en | input_val | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TWENTYSEVEN_CH AIN_DELAY | 3 | 27 | Intrinsic I/O delay + Minimum + 27 × Chain Delay | — | 2,732 | — | ps |
| TWENTYEIGHT_CH AIN_DELAY | 3 | 28 | Intrinsic I/O delay + Minimum + 28 × Chain Delay | — | 2,808 | — | ps |
| TWENTYNINE_CHA IN_DELAY | 3 | 29 | Intrinsic I/O delay + Minimum + 29 × Chain Delay | — | 2,901 | — | ps |
| THIRTY_CHAIN_DE LAY | 3 | 30 | Intrinsic I/O delay + Minimum + 30 × Chain Delay | — | 2,979 | — | ps |

You can program the number of delay steps by adjusting the I/O Delay register (io0_delay through io47_delay for I/Os 0 through 47).

Configuration Specifications

General Configuration Timing Specifications

Table 91.	General Configuration Timing Specifications

For specification status, see the Data Sheet Status table

| Symbol | Description | Requirement |  | Unit |
| --- | --- | --- | --- | --- |
|  |  | Minimum | Maximum |  |
| tCF12ST1 | nCONFIG high to nSTATUS high | — | 20 | ms |
| tCF02ST0 | nCONFIG low to nSTATUS low | — | 400 | ms |
| tST0 | nSTATUS low pulse during configuration error | 0.5 | 10 | ms |
| continued... |  |  |  |  |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

117

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Requirement |  | Unit |
| --- | --- | --- | --- | --- |
|  |  | Minimum | Maximum |  |
| tCD2UM (141) | CONF_DONE high to user mode | — | 5 | ms |
| tST12CF0 | Minimum time to drive nCONFIG from high to low after nSTATUS transitions from low to high | 0 | — | ms |
| tST02CF1 | Minimum time to drive nCONFIG from low to high after nSTATUS transitions from high to low | 0 | — | ms |

Figure 26.	General Configuration Timing Diagram

tST02CF1

Reconfiguration Triggered    Reconfiguration

tCF12ST1

nCONFIG

tST0

tST12CF0

Configuration Error

tCF02ST0

tCD2UM

Recovered Reconfiguration

nSTATUS CONF_DONE INIT_DONE

Configuration_State

User Mode

Device Clean

Idle

Configuration	Err Configuration Fail

Device Clean

Idle

Configuration

Initialization	User Mode

Note:	CONF_DONE and INIT_DONE are de-asserted during device clean state after full device reconfiguration is triggered.

POR Specifications

Power-on reset (POR) delay is defined as the delay between the time when all the power supplies monitored by the POR circuitry reach the minimum recommended operating voltage to the time when the nSTATUS is released high and your device is ready to begin configuration.

(141) This specification is the initialization time that indicates the time from CONF_DONE signal goes high to INIT_DONE signal goes high.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

118

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Table 92.	POR Delay Specification

For specification status, see the Data Sheet Status table

| POR Delay | Minimum | Maximum | Unit |
| --- | --- | --- | --- |
| AS (Normal mode), AVST ×8, AVST ×16, AVST ×32 | 11.5 | 20.2 | ms |
| AS (Fast mode) | 1.5 | 7.6 | ms |

External Configuration Clock Source Requirements

Table 93.	External Configuration Clock Source (OSC_CLK_1) Clock Input Requirements

For specification status, see the Data Sheet Status table

| Description | External Clock Source | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Clock input frequency(142) | Powered by VCCIO_SDM | 25/100/125 |  |  | MHz |
| Clock input peak-to-peak period jitter tolerance |  | — | — | 2 | % |
| Clock input duty cycle |  | 45 | 50 | 55 | % |

JTAG Configuration Timing

Table 94.	JTAG Timing Parameters and Values

For specification status, see the Data Sheet Status table

| Symbol | Description | Requirement |  | Unit |
| --- | --- | --- | --- | --- |
|  |  | Minimum | Maximum |  |
| tJCP | TCK clock period | 30 | — | ns |
| tJCH | TCK clock high time | 14 | — | ns |
| continued... |  |  |  |  |

(142) The acceptable clock frequencies are 25 MHz, 100 MHz, and 125 MHz only. You must match the external configuration clock frequency on the OSC_CLK_1 pin to the configuration clock source assignment in the Quartus Prime software. Other frequencies in the range are not supported.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

119

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Requirement |  | Unit |
| --- | --- | --- | --- | --- |
|  |  | Minimum | Maximum |  |
| tJCL | TCK clock low time | 14 | — | ns |
| tJPSU (TDI) (143) | TDI JTAG port setup time | 2 | — | ns |
| tJPSU (TMS) (143) | TMS JTAG port setup time | 3 | — | ns |
| tJPH (143) | JTAG port hold time | 5 | — | ns |
| tJPCO | JTAG port clock to output | — | 7(144) | ns |
| tJPZX | JTAG port high impedance to valid output | — | 14 | ns |
| tJPXZ | JTAG port valid output to high impedance | — | 14 | ns |

Figure 27.	JTAG Timing Diagram

tJCP

tJCH	tJCL

JPSU	JPH

t

t

tJPZX

tJPCO

tJPXZ

TMS TDI

TCK TDO

(143) For boundary-scan testing, the TMS and TDI JTAG ports minimum setup time and hold time are 7 ns.

(144) Capacitance loading at 10 pF.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

120

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

AS Configuration Timing

Table 95.	AS Timing Parameters

Altera recommends performing trace length matching for nCSO and AS_DATA pins to AS_CLK to minimize the skew. Refer to the related information to calculate the maximum allowable skew tolerance for nCSO and AS_DATA pins to AS_CLK.

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tclk (145) | AS_CLK clock period | — | 6.02 | — | ns |
| Tdutycycle | AS_CLK duty cycle | 45 | 50 | 55 | % |
| Tdcsfrs | AS_nCSO[3:0] asserted to first AS_CLK edge | 8.5(146) | — | — | ns |
| Tdcslst | Last AS_CLK edge to AS_nCSO[3:0] deasserted | 6.8(146) | — | — | ns |
| continued... |  |  |  |  |  |

(145) AS_CLK fMAX has dependency on the maximum board loading. For AS single device configuration or AS using multiple serial flash devices configuration, use the equations in Tdo and Text_delay notes to ensure your board has sufficient timing margin to meet flash setup/hold time specifications and AS timing specifications in this data sheet. For AS using multiple serial flash devices, refer to the related information for the recommended AS_CLK frequency and maximum board loading.

(146) AS operating at maximum clock frequency = 166 MHz. The delay is larger when operating at AS clock frequency lower than 166 MHz.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

121

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Minimum | Typical | Maximum | Unit |
| --- | --- | --- | --- | --- | --- |
| Tdo (147) | AS_DATA[3:0] output delay | –0.6 | — | 0.6 | ns |
| Text_delay (148) (149) | Total external propagation delay on AS signals | 0 | — | 13.5 | ns |
| Tdcsb2b | Minimum delay of slave select deassertion between two back-to-back transfers | 62 | — | — | ns |

Figure 28.	AS Configuration Serial Output Timing Diagram

Tdcsfrs

Tdo (min)

Tdcslst

Tdo (max)

OUT0

OUT1

OUTn

nCSO AS_CLK AS_DATA

(147) Load capacitance for DCLK = 12 pF and AS_DATA = 27 pF. Altera recommends obtaining the Tdo for a given link (including receiver, transmission lines, connectors, termination resistors, and other components) through IBIS or HSPIC simulation. To analyze flash setup time,

    - Tsu  = Tclk/2 - Tdo(max)  + Tbd_clk  – Tbd_data(max)

    - Tho  = Tclk/2 + Tdo(min)  – Tbd_clk  + Tbd_data(min)

(148) Text_delay = Tbd_clk + Tco + Tbd_data + Tadd

    - Tbd_clk: Propagation delay for AS_CLK between FPGA and flash device.

    - Tco: Output hold time and clock low to output valid of flash device. This delay must be used to ensure Text_delay is within the minimum and maximum specification values.

    - Tbd_data: Propagation delay for AS_DATA bus between FPGA and flash device.

    - Tadd: Propagation delay for active/passive components on AS_DATA interfaces.

(149) Text_delay specification is based on AS_CLK = 166 MHz. The value can be larger at lower AS_CLK frequency. For more details, refer to the related information.

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

122

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

Figure 29.	AS Configuration Serial Input Timing Diagram

Tdcsb2b

Text_delay

IN0

IN1

nCSO AS_CLK

INn

AS_DATA

Related Information

Agilex 7 Configuration User Guide

Provides more information about AS_CLK.

Avalon Streaming Configuration Timing

Table 96.	Avalon Streaming Timing Parameters for x8, ×16, and ×32 Configurations

For specification status, see the Data Sheet Status table

| Symbol | Description | Minimum | Unit |
| --- | --- | --- | --- |
| tACLKH | AVST_CLK high time | 3.6 | ns |
| tACLKL | AVST_CLK low time | 3.6 | ns |
| tACLKP | AVST_CLK period | 8 | ns |
| tADSU (150) | AVST_DATA setup time before rising edge of AVST_CLK | 2.1 | ns |
| continued... |  |  |  |

(150) Data sampled by the FPGA (sink) at the next rising clock edge.

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

123

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Symbol | Description | Minimum | Unit |
| --- | --- | --- | --- |
| tADH (150) | AVST_DATA hold time after rising edge of AVST_CLK | 0.1 | ns |
| tAVSU | AVST_VALID setup time before rising edge of AVST_CLK | 2.1 | ns |
| tAVDH | AVST_VALID hold time after rising edge of AVST_CLK | 0 | ns |

tACLKP

tACLKH

K

tACLKL

Y Y

D D

t

AVDH

must deassert within 6 cycles

]

]]

data1 data2 data3

ta0

da

ADSU

t

tADH

AVSU

t

Figure 30.	Avalon Streaming Configuration Timing Diagram

AVSTx8_CL or AVST_CLK

AVST_READ

or AVSTx8_READ

AVSTx8_VALI or AVST_VALI

AVSTx8_DATA[7:0] AVST_DATA[15:0

AVST_data[31:0

Configuration Bit Stream Sizes

Table 97.	Configuration Bit Stream Sizes

Configuration bit stream sizes shown in this table are based on worst-case scenarios. The sizes are typically substantially smaller because of the use of the bit stream compression. The bit stream compression efficiency has dependency on your design complexity.

For specification status, see the Data Sheet Status table

| Variant | Compressed Configuration Bit Stream Size (Mbits) |
| --- | --- |
| AGM 032, AGM 039 | 1,443 |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

124

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

I/O Timing

I/O timing data is typically used prior to designing the FPGA to get an estimate of the timing budget as part of the timing analysis. You may generate the I/O timing report manually using the Timing Analyzer.

The Quartus Prime Timing Analyzer provides a more accurate and precise I/O timing data based on the specifics of the design after you complete place-and-route.

Related Information

AN 775: Generating Initial I/O Timing Data and I/O Element Delays for Altera FPGAs

Provides the techniques to generate I/O timing information using the Quartus Prime software.

Programmable IOE Delay

Table 98.	Programmable IOE Delay

For specification status, see the Data Sheet Status table

| Parameter | Maximum Offset | Minimum Offset | Fast Model | Slow Model |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Extended | –E1V, –I1V | –E2V, –I2V | –E3V, –E3E, –I3V |  |
| Input Delay Chain (INPUT_DELAY_CH AIN) | 63 | 0 | 2.279 | 2.4495 | 2.735 | 3.105 | ns |
| Output Delay Chain (OUTPUT_DELAY_C HAIN) | 15 | 0 | 0.546 | 0.5865 | 0.657 | 0.747 | ns |
| Output Enable Delay Chain (OUTPUT_ENABLE_ DELAY_CHAIN) | 15 | 0 | 0.546 | 0.5865 | 0.657 | 0.747 | ns |

Glossary

Table 99.	Glossary

| Term | Definition |
| --- | --- |
| Differential I/O Standards | Receiver Input Waveforms |
| continued... |  |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

125

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Term | Definition |
| --- | --- |
|  | Single-Ended Waveform Positive Channel (p) = VIH VID Negative Channel (n) = VIL VCM Ground Differential Waveform VID p - n = 0 V VID Transmitter Output Waveforms Single-Ended Waveform Positive Channel (p) = VOH VOD Negative Channel (n) = VOL VCM Ground Differential Waveform VOD p - n = 0 V VOD |
| fHSCLK | I/O PLL input clock frequency. |
| fHSDR | LVDS SERDES block—maximum/minimum LVDS data transfer rate (fHSDR = 1/TUI), non-DPA. |
| fHSDRDPA | LVDS SERDES block—maximum/minimum LVDS data transfer rate (fHSDRDPA = 1/TUI), DPA. |
| J (SERDES factor) | LVDS SERDES block—deserialization factor (width of parallel data bus). |
| JTAG Timing Specifications | JTAG Timing Specifications: |
| continued... |  |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

126

| 0.5 x TCCS | RSKM | Sampling Window (SW) | RSKM | 0.5 x TCCS |
| --- | --- | --- | --- | --- |

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Term | Definition |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | TMS |  |  |  |  |  |  |
|  | TDI |  |  |  |  |  |  |
|  |  | tJCP tJCH | t JCL |  | tJPSU | tJPH |  |
|  | TCK |  |  |  |  |  |  |
|  |  | tJPZX |  | tJPCO |  |  | tJPXZ |
|  | TDO |  |  |  |  |  |  |
| RL | Receiver differential input discrete resistor (external to the device). |  |  |  |  |  |  |
| Sampling window (SW) | Timing Diagram—the period of time during which the data must be valid in order to capture it correctly. The setup and hold times determine the ideal strobe position in the sampling window, as shown: Bit Time |  |  |  |  |  |  |
| Single-ended voltage referenced I/O standard | The JEDEC standard for the SSTL and HSTL I/O defines both the AC and DC input signal values. The AC values indicate the voltage levels at which the receiver must meet its timing specifications. The DC values indicate the voltage levels at which the final logic state of the receiver is unambiguously defined. After the receiver input has crossed the AC value, the receiver changes to the new logic state. The new logic state is then maintained as long as the input stays beyond the DC threshold. This approach is intended to provide predictable receiver timing in the presence of input waveform ringing. Single-Ended Voltage Referenced I/O Standard |  |  |  |  |  |  |
| continued... |  |  |  |  |  |  |  |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

127

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Term | Definition |
| --- | --- |
|  | V CCIO VOH V IH(AC) V IH(DC) V REF V IL(DC) V IL(AC) VOL V SS |
| tC | High-speed receiver/transmitter input and output clock period. |
| TCCS (channel-to-channel-skew) | The timing difference between the fastest and slowest output edges, including the tCO variation and clock skew, across channels driven by the same PLL. The clock is included in the TCCS measurement (refer to the Timing Diagram figure under SW in this table). |
| tDUTY | LVDS SERDES block—duty cycle on high-speed transmitter output clock. |
| tFALL | Signal high-to-low transition time (80–20%). |
| tINCCJ | Cycle-to-cycle jitter tolerance on the PLL clock input. |
| tOUTPJ_IO | Period jitter on the GPIO driven by a PLL. |
| tOUTPJ_DC | Period jitter on the dedicated clock output driven by a PLL. |
| tRISE | Signal low-to-high transition time (20–80%). |
| Timing Unit Interval (TUI) | The timing budget allowed for skew, propagation delays, and the data sampling window. (TUI = 1/(Receiver Input Clock Frequency Multiplication Factor) = tC/w). |
| VCM(DC) | DC Common mode input voltage. |
| VICM | Input Common mode voltage—the common mode of the differential signal at the receiver. |
| VICM(DC) | VCM(DC) DC Common mode input voltage. |
| VID | Input differential voltage swing—the difference in voltage between the positive and complementary conductors of a differential transmission at the receiver. |
| VDIF(AC) | AC differential input voltage—minimum AC input differential voltage required for switching. |
| continued... |  |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

128

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Term | Definition |
| --- | --- |
| VDIF(DC) | DC differential input voltage—minimum DC input differential voltage required for switching. |
| VIH | Voltage input high—the minimum positive voltage applied to the input which is accepted by the device as a logic high. |
| VIH(AC) | High-level AC input voltage. |
| VIH(DC) | High-level DC input voltage. |
| VIL | Voltage input low—the maximum positive voltage applied to the input which is accepted by the device as a logic low. |
| VIL(AC) | Low-level AC input voltage. |
| VIL(DC) | Low-level DC input voltage. |
| VOCM | Output Common mode voltage—the common mode of the differential signal at the transmitter. |
| VOD | Output differential voltage swing—the difference in voltage between the positive and complementary conductors of a differential transmission line at the transmitter. |
| VSWING | Differential input voltage. |
| VOX | Output differential cross point voltage. |
| VIX(AC) | VIX Input differential cross point voltage. |
| W | LVDS SERDES block—Clock Boost Factor. |

Document Revision History for the Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

| Document Version | Changes |
| --- | --- |
| 2025.09.05 | - Updated footnote about powering VCCIO_PIO_SDM the same voltage as VCCIO_PIO_3AT when using Avalon streaming configurations in the Recommended Operating Conditions table. - Updated the status of AGM 032/039 device to Final in Data Sheet Status for Agilex 7 FPGAs and SoCs M-Series table. - Added -IE3 speed in the industrial in the device Grades, Core Speed Grades, and Power Options Supported table. - Updated the footnote about the ripple switching frequency on FHT Power Rail Tolerance in F-Tile Transceiver Power Supply Recommended Operating Conditions table. - Removed the footnote about PAM4 bounding box in F-Tile FGT Supported Electrical Compliance List table. - Updated the protocols in the F-Tile FHT Supported Electrical Compliance List and F-Tile FGT Supported Electrical Compliance List tables. - Added Differential Input High Voltage and Differential Input Low Voltage parameters in the R-Tile Reference Clock Specifications table. |
| continued... |  |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

129

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Document Version | Changes |
| --- | --- |
|  | - Revised the HPS and SDM DC Characteristics and HPS and SDM I/O Standard Specifications, organizing them into individual sections: - HPS I/O DC Characteristics - SDM I/O DC Characteristics - HPS I/O Standard Specifications - SDM I/O Standard Specifications - Updated the maximum specifications for Clock Frequency fHSCLK_in(input clock frequency) SLVS400 I/O Standards in the LVDS SERDES Specifications table. - Added POD12 and POD11 parameters for GPIO and PHYLITE modes in the GPIO-B Single-Ended SSTL, HSTL, HSUL, POD, and LVSTL I/O Reference Voltage Specifications table. - Added LVSTL700 VREF specifications in the GPIO-B Single-Ended SSTL, HSTL, HSUL, POD, and LVSTL I/O Reference Voltage Specifications table. - Added footnote about POD EMIF Interface to refer to eye mask estimator guidelines in the GPIO-B Single-Ended SSTL, HSTL, HSUL, and POD I/O Standards Signal Specifications and GPIO-B Differential POD I/O Standards Specifications tables. - Added notes to the True Differential Signaling-1.3 V (LVDS compatible Transmitter and Receiver) specifications in the GPIO-B Differential I/O Standards Specifications tables. - Removed mentions of SLVS400 standard in the GPIO-B OCT Calibration Accuracy Specifications table. - Added SLVS400 I/O standard to the GPIO-B OCT Without Calibration Resistance Tolerance Specifications table. - Updated the description about overshooting values when using True Differential Signaling I/O standard at VCCIO_PIO at 1.3 V in the Maximum Allowed Overshoot and Undershoot Voltage section. |
| 2025.04.07 | - Updated the footnote about VCCIO_PIO_SDM when using Avalon® streaming x16/x32 configuration schemes in the Recommended Operating Conditions table. - Updated I/O PLL Specifications table. - Updated the description on FHT Power Rail Tolerance in the F-Tile Transceiver Power Supply Recommended Operating Conditions table. |
| 2025.04.07 | - Updated note about M20K block in Memory Block Performance Specifications table. - Updated specifications in the HPS and SDM I/O Pin Leakage Current table. - Removed Max and Typ specifications in the HPS and SDM I/O Hysteresis Specifications for Schmitt Trigger Input table. - Added notes in the GPIO-B Single-Ended LVSTL I/O Standards Specifications and GPIO-B Differential LVSTL I/O Standards Specifications tables. - Removed the QDR-IV memory standard in Memory Standards Supported table. - Updated note related to the tolerance requirement in Recommended Operating Conditions table. - Updated the Transmitter tX Jitter and TCCS specifications in LVDS SERDES Specifications. - Updated the recommended AC transient values for VCC_HSSI_GXF and VCCERT_FGT_GXF FGT Analog Core supply alone in F-Tile Transceiver Power Supply Recommended Operating Conditions. - Updated description for the single sideband phase noise parameter in F-Tile FHT Reference Clock Requirements. - Added note about voltage sensor accuracy specifications in Voltage Sensor Specifications table. - Added footnote for Reference phase noise parameter in the I/O PLL Specifications. |
| continued... |  |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

130

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Document Version | Changes |
| --- | --- |
| 2025.01.23 | - Added Industrial in Device Grades, Core Speed Grades, and Power Options Supported table. - Updated Internal VREF specifications for LVSTL700, LVSTL105 and LVSTL11 I/O standards in HSIO Single-Ended SSTL, HSTL, HSUL, POD, and LVSTL I/O Reference Voltage Specifications table . - Removed the QDR-IV memory standard in Memory Standards Supported table. |
| 2024.11.25 | - Updated the maximum clock frequencies for -2 speed grade in the LVDS SERDES Specifications table. - Updated SLVS and LVDS SERDES Receiver specifications at 1.05V/1.1V/1.2V TDS frequency in the LVDS SERDES Specifications table. - Updated the -E3V and -3E3E speed grade specifications in the Programmable IOE delay table. - Updated the fHSDR data rate (without DPA) in SERDES factor J = 4 and 8 specifications in the LVDS SERDES Specifications table. - Updated the titles to F-Tile FGT Supported Electrical Compliance List and F-Tile FHT Supported Electrical Compliance List. - Added footnote to refer to case 14023487435 for BER compliance in the F-Tile FGT Supported Electrical Compliance List table. - Updated the Hold time for USB_DIR/USB_NXT/USB_DATA[7:0] (Th) specifications in the HPS USB UPLI Timing Characteristics table. - Added a description about preventing F-Tile performance degradation in the F-Tile Power Supply Recommended Operating Conditions table. - Added note about supporting Hot Swap with FHT PMA's for AC and DC coupled connections in F-Tile Receiver Specifications table. |
| 2024.07.15 | - Updated the status of AGM 032/039 device in Data Sheet Status for Agilex™ 7 FPGAs and SoCs M-Series table. - Added AGM 039 R31B and R47B packages in Data Sheet Status for Agilex™ 7 FPGAs and SoCs M-Series table. - Updated description for TTX-RJ parameter in F-Tile FGT Transmitter Electrical Specifications table. - Updated "Typical" values of Differential on-chip termination resistors in the R-Tile Transmitter Specifications and R-Tile Receiver Specifications tables. - Added footnote in R-Tile Transmitter Specifications and R-Tile Receiver Specifications tables. - Updated calibration accuracy for SLVS-400 standard in GPIO-B OCT Calibration Accuracy Specifications table. - Updated note to remove LVDS SERDES support for the following tables: - GPIO-B Single-Ended SSTL, HSTL, HSUL, POD and LVSTL I/O Reference Voltage Specifications - GPIO-B Single-Ended LVSTL I/O Standards Specifications - GPIO-B Differential SSTL, HSTL, and HSUL I/O Standards Specifications - GPIO-B Differential LVSTL I/O Standards Specifications - Added note in Performance Specifications of Network on Chip (NoC) section. - Updated the "Typical" and "Maximum" values of Single sideband phase parameter in the F-Tile FHT Reference Clock Requirements table. - Updated the "Typical" and "Maximum" values of PNREF-SSB (156.25MHz) parameter and the footnote in the F-Tile FGT Reference Clock Requirements table. - Updated specifications and added note related to VCCIO_PIO in GPIO-B OCT Without Calibration Resistance Tolerance Specifications table. - Updated footnote related to the 1.05 V, 1.1 V, and 1.2 V True Differential Signaling I/O standards in LVDS SERDES Specifications table. - Added NOC PLL Input Requirements table. - Updated tINCCJ specifications for FREF <100 MHz in I/O PLL Specifications table. - Updated the footnote in GPIO-B OCT Without Calibration Resistance Tolerance Specifications table. |
| continued... |  |

Send Feedback	Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series

131

Agilex™ 7 FPGAs and SoCs Device Data Sheet: M-Series

769310 | 2025.09.05

| Document Version | Changes |
| --- | --- |
| 2023.12.15 | - Added AGM 039 R31B package in the Data Sheet Status for Intel Agilex 7 FPGAs and SoCs M-Series table. - Updated the CONF_DONE and INIT_DONE signals in the General Configuration Timing Diagram and added a note for the signals. - Included specifications for Input Clock Frequency Source from IO Clock Input in I/O PLL Specifications table. - Updated Parameter: Input Clock Frequency to Input Clock Frequency Source from Core Clock Input or Reference Clock Input. - Updated the Recommended Operating Conditions table. - Added IFUSEWR symbol. - Updated the extended grade minimum TJ specifications. - Added clarity for the industrial grade minimum TJ specifications. - Updated the maximum value of V CCFUSEWR_SDM in the Absolute Maximum Ratings table. - Updated VCCIO_PIO footnote in the following tables: - Recommended Operating Conditions - GPIO-B Single-Ended SSTL, HSTL, HSUL, POD and LVSTL I/O Reference Voltage Specifications - GPIO-B Single-Ended LVSTL I/O Standards Specifications - GPIO-B Differential SSTL, HSTL, and HSUL I/O Standards Specifications - GPIO-B Differential POD I/O Standards Specifications - GPIO-B Differential LVSTL I/O Standards Specifications - Added LVSTL11, LVSTL105, and LVSTL700 I/O Standards in GPIO-B Single-Ended SSTL, HSTL, HSUL, and POD I/O Reference Voltage table. - Removed 1.1 V support for LVSTL700 standard in the Table: GPIO-B Single-Ended SSTL, HSTL, HSUL, POD and LVSTL I/O Reference Voltage Specifications and GPIO-B Single-Ended LVSTL I/O Standards Specifications. - Updated the VOD maximum specifications for True Differential Signaling -1.3V (Transmitter and Receiver) I/O Standard in GPIO-B Differential I/O Standards Specifications table. - Added SLVS-400 support and footnote in LVDS SERDES Specifications table. - Updated the ideality factors when using lower injection current (two-currents) in Remote Temperature Diode section. - Updated note in Voltage Sensor Specifications section. - Updated supported –2 transceiver speed grade data rate for NRZ in the F-Tile FGT Transmitter and Receiver Data Rate Performance. - Added footnote in F-Tile FGT Electrical Compliance List table. - Removed XSR support in CEI 4.0/5.0 specifications in the F-Tile FHT Electrical Compliance List table. - Updated the VRX-DIFF-PKPK specifications F-Tile FGT Receiver Electrical Specifications table. - Updated the Programmable IOE Delay section. - Removed -I1V, -I2V, and -I3V speed grades. - Updated IOE delay in all speed grades. |
| 2023.07.13 | Removed the note about restricted support for M-Series FPGAs. |
| 2023.05.05 | Initial release. |

Agilex 7 FPGAs and SoCs Device Data Sheet: M-Series	   Send Feedback

132