Agilex® 7 FPGAs and SoCs M-Series Product Table

Version 2026.04.01

# Product Line	AGM 032	AGM 039

| Resources | Specification Options¹ | A, B, E, F, G, H | A, B, E, F, G, H |
| --- | --- | --- | --- |
|  | Logic elements (LEs) | 3,245,000 | 3,851,520 |
|  | Adaptive logic modules (ALMs) | 1,100,000 | 1,305,600 |
|  | ALM registers | 4,400,000 | 5,222,400 |
|  | M20K memory blocks | 15,932 | 18,960 |
|  | M20K memory size (Mb) | 311 | 370 |
|  | MLAB memory count | 55,000 | 65,280 |
|  | MLAB memory size (Mb) | 33 | 40 |
|  | HBM2E High Bandwidth DRAM Memory (Gbytes) Size | 16 / 32 | 16 / 32 |
|  | Fabric PLL | 8 | 8 |
|  | I/O PLL | 16 | 16 |
|  | Variable-precision digital signal processing (DSP) blocks | 9,375 | 12,300 |
|  | 18 x 19 multipliers | 18,750 | 24,600 |
|  | Single-precison or half-precision tera floating point operations per second (TFLOPS) | 14 / 28 | 18.4 / 37 |
| Maximum Available Device Resources | Maximum EMIF x72 | 4 |  |
|  | Memory devices supported | DDR5, LPDDR5, and DDR4 |  |
|  | Maximum AIB Interfaces | 4 |  |
|  | Secure device manager (SDM) | AES-256/SHA-256 bitstream encryption/authentication, physically unclonable function (PUF), ECDSA 256/384 boot code authentication, side channel attack protection |  |
|  | Hard processor system (HPS) | Quad-core 64 bit Arm* Cortex*-A53 up to 1.5 GHz with 32KB I/D cache, Neon* coprocessor, 1 MB L2 Cache, direct memory access (DMA), system memory management unit, cache coherency unit, hard memory controllers, USB 2.0 x2, 1G EMAC x3, UART x2, SPI x4, I2C x5, general purpose timers x7, watchdog timer x4 |  |

Notes:

- Specification code definition:

| Code | HPS | Crypto | HBM2e Density |
| --- | --- | --- | --- |
| A | No | No | None |
| B | Yes | No | None |
| C | No | Yes | None |
| D | Yes | Yes | None |
| E | Yes | No | 16 GB |
| F | Yes | No | 32 GB |
| G | No | No | 16 GB |
| H | No | No | 32 GB |

Agilex® 7 FPGAs and SoCs M-Series Product Table

Version 2026.04.01

# Product Line	AGM 032	AGM 039

| Tile Resources | F-Tile | PCI Express* (PCIe*) hard IP block (4.0 x16 ) or bifurcatable 2x PCIe 4.0 x8 (EP) or 4x 4.0 x4 (RP) Transciever channel count : - 4 channels at 116 Gbps (PAM4) / 58 Gbps (NRZ) - 16 channels at 32 Gbps (NRZ) /12 channels at 58 Gbps (PAM4) - RS & KP FEC Advanced networking support: - Bifurcatable 400 G hard IP block (10/25/50/100/200/400 GbE FEC/PCS/MAC) - Bifurcatable 200 G hard IP block (10/25/50/100/200 GbE FEC/PCS) 300G Interlaken IEEE 1588 support PMA direct |
| --- | --- | --- |
|  | R-Tile | Compute Express Interface (CXL) - Link width x16 lanes, x8 lanes PCIe hard IP block (5.0 x16 ) or Bifurcatable 2x PCIe 5.0 x8 (EP) or 4x 5.0 x4 (RP) Virtualization (SR-IOV) supporting 8 PFs/2k VFs Scalable IOV VirtIO Support Precise Time Management PIPE Direct |

| Package Options (Non-HBM Packages) Tile Configuration | GPIO(LVDS) / F-Tile 32G NRZ (58G PAM4) / High-Speed Transceiver 58G NRZ (116G PAM4) / R- Tile 32G PCIe (CXL) Lanes |  |  |  |
| --- | --- | --- | --- | --- |
| R47A (56 mm x 66 mm, 0.92 mm Hex, 4700A) F-Tile x3, R-Tile x1 | 768(384) / 48(36) / 8(8) / 16(16) |  | 768(384) / 48(36) | / 8(8) / 16(16) |
| R47D (56 mm x 66 mm, 0.92 mm Hex, 4700A) F-Tile x1, R-Tile x1 | 768(384) / 16(12) / 4(4) / 16(16) |  | 768(384) / 16(12) | / 4(4) / 16(16) |
| R47C (56 mm x 66 mm, 0.92 mm Hex, 4700C) F-Tile x4 | 768(384) / 64(48) / 16(16) |  | 768(384) / 64(48) / 16(16) |  |
| R31B (56 mm x 45 mm, 0.92 mm Hex, 3184B) F-Tile x4 | 720(360) / 64(48) / 8(8) |  | 720(360) / 64(48) / 8(8) |  |
| R31G (56 mm x 45 mm, 0.92 mm Hex, 3184G) F-Tile x2, R-Tile x2 | 768(384) / 32(24) / 8(8) / 32(32) |  | 768(384) / 32(24) | / 8(8) / 32(32) |

| Package Options (HBM Packages) Tile Configuration | GPIO(LVDS) / F-Tile 32G NRZ (58G PAM4) / High-Speed Transceiver 58G NRZ (116G PAM4) / R- Tile 32G PCIe (CXL) Lanes |  |  |  |
| --- | --- | --- | --- | --- |
| R47A (56 mm x 66 mm, 0.92 mm Hex, 4700A) F-Tile x3, R-Tile x1, HBM | 768(384) / 48(36) / 8(8) / 16(16) |  | 768(384) / 48(36) | / 8(8) / 16(16) |
| R47B (56 mm x 66 mm, 0.92 mm Hex, 4700B) F-Tile x4, HBM | 768(384) / 64(48) / 8(8) |  | 768(384) / 64(48) / 8(8) |  |

Note:

- All data is correct at the time of printing, and may be subject to change without prior notice. For the latest information, please visit www.altera.com.

Altera technologies may require enabled hardware, software or service activation. No product or component can be absolutely secure. Your costs and results may vary.

© Altera Corporation. Altera, the Altera logo, and other Altera marks are trademarks of Altera Corporation or its subsidiaries.

*Other names and brands may be claimed as the property of others.